1. APPLE-DOS KERMIT

Authors:    Antonino N. J. Mione, Stevens Institute of Technology

Peter Trei, Columbia University

Documentation:

Antonino N.J. Mione, Stevens Institute of Technology

Peter Trei, Columbia University

Version:    2.1(45)

Date:       July 1984

Kermit-65 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     No

^X/^Y interruption:                No

Filename collision avoidance:      Yes

Can time out:                      No

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes (VT52)

Communication settings:            Yes; local echo, parity

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    No

Raw upload:                        No

Act as server:                     No

Talk to server:                    Yes

Advanced commands for servers:     No

Local file management:             No

Handle file attributes:            No

Command/init files:                No

Printer control:                   No

KERMIT-65 is a program that implements the KERMIT file transfer protocol

for  the  Apple ][ micro computer system. It is written in 6502 assembly

language and should run on any Apple ][ or Apple ][ Plus system  running

DOS  3.3.    This section will describe the things you should know about

the DOS 3.3 file system in order to make effective use  of  KERMIT,  and

then it will describe the special features of the KERMIT-65 program.

1.1. The DOS 3.3 File System

Items  of  importance  which  will  be discussed in this section include

Filenames and File Characteristics.

Apple DOS Filenames

Filenames  under  Apple  DOS  may  contain  almost  any  ASCII character

(including space). It is not recommended that special characters,  (i.e.

control characters or spaces) be used in a filename to be transferred by

Kermit-65 since they may cause problems when parsing the filename.

Filenames  may  be  up to 40 characters in length. No wildcarding of any

kind can be done in KERMIT-65.

Apple DOS File Characteristics

All files in Apple DOS have a file type associated with  them  which  is

contained  in  the  directory  entry for the file but is not part of the

filename itself. There are four types of files in DOS 3.3. They are:

- APPLESOFT BASIC

- INTEGER BASIC

- BINARY

- TEXT

All file types have their data stored in eight-bit  bytes  although  not

all  of  them  need  the eighth bit. The two file types containing basic

programs required the eighth bit due to the nature  of  the  data  being

stored.  BINARY  files  are  images of memory copied into a file. Often,

these are machine code programs. These files  require  all  eight  bits.

TEXT  files  normally contain only printable or carriage control charac-

ters. They are stored in the form of seven-bit ASCII characters but  the

eighth  bit should always be set since Apples manipulate all text inter-

nally as 'NEGATIVE ASCII'.

When transmitting text files, the byte size should be set to  seven-bit.

When  transmitting anything else, the user must insure that both Kermits

are handling eight bit data so  that  no  information  is  lost.  If  an

eight-bit data path is not available (i.e. the remote Kermit needs to do

parity  checking  with  the eigth bit), then eight-bit quoting should be

used. Of course, BINARY files as well as Apple BASIC files will not have

much meaning on a different system.  If the user desires to edit a BASIC

file on a mainframe, for instance, he must convert it to a TEXT file be-

fore sending it over. After receiving the file back on  the  Apple,  the

user  may  convert  it  back to BASIC once again. The reason BASIC files

would be meaningless to a different machine is  that  the  Apple  stores

BASIC  keywords  as single character tokens to save space and processing

time. To convert a BASIC program to and from a TEXT  file,  consult  the

Apple DOS 3.3 Manual.

File information can be obtained by issuing the CATALOG command. For ex-

ample:

]CATALOG

DISK VOLUME 010

*A 002 HELLO

B 078 KERMIT

A 002 READER

T 005 TESTFILE

]

When KERMIT-65 is receiving a file, the file it creates on diskette will

be  of the type indicated by the FILE-TYPE parameter.  The file will al-

ways be left in an unlocked state after it is closed by KERMIT-65.  When

sending  a file, KERMIT-65 will use the FILE-TYPE parameter to determine

how to detect an End-of-file condition. Thus, it is  important  to  have

this set properly in all cases.

Recommendations for archiving files

When  using a large system for archiving purposes, there is no reason to

convert Apple Basic programs into text files before sending  them  since

there is no need to edit them on the mainframe.

The  FILE-TYPE  parameter  must always be set correctly when sending and

receiving files.  Also,  when  sending  files  which  require  eight-bit

FILE-BYTE-SIZEs,   this  parameter  must  also  be  set  properly  since

KERMIT-65 does not change it automatically based on FILE-TYPE.

The procedure for archiving TEXT files is:

- Run Kermit on remote system

- SET FILE-BYTE-SIZE SEVEN-BIT ! On KERMIT-65

- SET FILE-TYPE-MODE TEXT ! On KERMIT-65

- Send files

The procedure for archiving APPLESOFT, INTEGER, and BINARY files is:

- Run Kermit on remote system

- Set File-byte-size to Eight-bit ! On Remote Kermit

- SET FILE-BYTE-SIZE EIGHT-BIT ! On KERMIT-65

- SET FILE-TYPE-MODE APPLESOFT !  (or  INTEGER,  or  BINARY)  On

KERMIT-65

- Send files

1.2. Program Operation

HARDWARE CONSIDERATIONS

Prior  to  using  KERMIT-65  for transferring files, the modem interface

must be set to handle data in a certain manner. Firstly, the data format

should be 8 data bits and 1 stop bit. Secondly, the card should  be  set

to no parity. The baud rate (if adjustable) must be set to whatever rate

the  modem  can  handle.  For  the  Apple  Com  card  and the D.C. Hayes

Micromodem, these parameters are set correctly by default, so very  lit-

tle has to be done. For the Apple Super Serial Card, however, these have

to  be  set before using KERMIT-65. In all cases, use the same procedure

to connect to the mainframe host as is indicated in the section below on

Installing KERMIT.

Some mainframes cannot handle data in the format of 8 data  bits  and  1

stop bit because they may need parity checking (i.e. most IBM machines).

In this case, 7 data bits and 1 stop bit plus some parity setting (other

than  none) will usually work. When talking with such mainframes, binary

and  basic  files  on   the   Apple   cannot   be   transferred   unless

Eighth-bit-quoting is used.

CONVERSING WITH KERMIT-65

KERMIT-65's prompt is "KERMIT-65>".

To run KERMIT-65 and issue commands to it, type the following:

]BRUN KERMIT

STEVENS/CU - APPLE ][ KERMIT-65 - VER 2.1

Kermit-65>SEND TESTFILE

file is sent

Kermit-65>STATUS

performance statistics are printed

Kermit-65>Other commands

.

.

.

Kermit-65>EXIT

]

KERMIT-65 uses a TOPS-20 style command parser.

During  interactive  operation, you may use the ?-prompting help feature

("?") and recognition (ESC) features while typing commands.  A question

mark typed at any point in a command displays the options  available  at

that  point;  typing  an  ESC character causes the current keyword to be

completed (or default value to be supplied).  If you have not typed suf-

ficient characters to uniquely specify the keyword or  filename  (or  if

there  is no default value) then a beep will be sounded and you may con-

tinue typing. Keywords may be abbreviated to any prefix that is unique.

There are several different Apple]['s which can  run  KERMIT-65.  Kermit

will  have  no  problems  running on an Apple ][ or Apple ][+ system. It

will run on the Apple //e as well, however, the 80-column  board  cannot

be used at this time.

Of the different communication devices available for the Apple ][, three

are supported:

- Apple Com Card

- D.C. Hayes Micromodem

- Apple Super Serial Card

It  is  possible  that  other cards may have operational characteristics

very similar or identical to one of the devices above. If  this  is  the

case,  it  may work using one of the currently available device drivers.

The user may want to try each of the above options to see if any of them

work.

KERMIT-65 must be told which device is being used so  that  it  may  run

with  the  correct device driver. It also must be told in which slot the

card resides. This may  be  done  with  the  'SET'  command  (documented

below).

1.3. Remote and Local Operation

KERMIT-65  is normally run in local mode. It may be run as a remote Ker-

mit as well although there is no advantage to  doing  things  that  way.

KERMIT-65  supports  User-mode commands for talking to a Server. It does

not currently support server mode.

1.4. KERMIT-65 Commands

THE SEND COMMAND

Syntax: SEND filespec

The SEND command causes a file to be sent from the Apple to  the  remote

system. The Filespec is the name of the file on the Apple diskette to be

sent.  The parser will not accept control characters and certain special

characters in a filename (i.e. a comma), so the user may have to  rename

the  file before it is sent. The user may also have problems in filename

compatibility with remote Kermits. If the remote Kermit  does  not  have

the facilities to beat the filename into a format that its system likes,

the  user  may  have  to rename the file before sending it.  The default

disk drive used for file transfers is the drive used to boot the  system

or  the last drive accessed with a DOS command. This can be changed with

the 'SET DEFAULT-DISK' command (explained below). Either the slot or the

drive or both may be altered.

As a file is being sent, the screen displays either 'SENDING  PACKET...'

or  'WAITING  PACKET...'  followed  by  the absolute packet number since

start of transmission. If a packet must be transmitted several times and

it reaches the maximum retry count,  the  transfer  will  fail  and  the

'KERMIT-65>'  prompt  will  return.  If the remote Kermit sends an error

packet, the text of the packet will be displayed on the screen  and  the

prompt will return.

Currently,  a packet can be retransmitted manually by typing anything on

the keyboard. If a  'Q'  is  typed,  the  entire  transmission  will  be

aborted.

THE RECEIVE COMMAND

Syntax: RECEIVE [filespec]

The RECEIVE command tells KERMIT-65 to receive a file or file group from

the  other  system.  If only one file is being received, you may include

the optional filespec as the name to  store  the  incoming  file  under;

otherwise, the name is taken from the incoming file header.  If the name

in the header is not a legal filename in DOS 3.3, KERMIT-65 will attempt

to  change  it  into something legal. There are very few things that are

illegal in DOS 3.3. If FILE-WARNING is on and an  incoming  file  has  a

name  identical  to  a  file already existing on the diskette, KERMIT-65

will issue a warning to the user and attempt to modify the  filename  to

make it unique.

GET

Syntax: GET remote-filespec

The GET command requests a remote KERMIT server to send the file or file

group  specified  by  remote-filespec.   This command can be used with a

KERMIT server on the other end.

The remote filespec is any string that can be a legal file specification

for the remote system; it is not parsed or validated locally.

If the remote KERMIT is not capable of server functions, then  you  will

probably  get  an error message back from it like "Illegal packet type".

In this case, you must connect to the other Kermit, give a SEND command,

escape back, and give a RECEIVE command.

THE CONNECT COMMAND

Syntax: CONNECT

Establish  a  terminal  connection  to  the  remote  system. Get back to

KERMIT-65 by typing the escape character followed by the  letter  C. The

escape  character  is  Control-@  by  default.  When you type the escape

character, several single-character commands are possible:

B   Send a BREAK Signal.

C   Close the connection and return to KERMIT-65.

S   Show status of the connection.

0   Send a null.

Connect-escape

Send the Connect-escape character itself.

?   List all the possible single-character arguments.

You can use the SET ESCAPE command to define a different escape  charac-

ter.

When  'CONNECTED',  KERMIT-65  will be passing characters entered on the

keyboard to the remote system, and passing characters  from  the  remote

system  to the Apple screen. If VT52-EMULATION is turned on, Kermit will

trap escape codes and simulate the appropriate functions of a VT52  ter-

minal.

On  an  Apple ][+ with an incomplete keyboard, special characters can be

obtained by prefixing regular characters with a right-arrow.  Also,  Up-

percase  is  shown  in inverse and lowercase characters are displayed as

normal uppercase characters.

Here are the rules for using the special 2/2+ input, to get  all  print-

able ASCII characters, and how they appear on the screen:

Special  meanings are applied in various contexts to certain characters.

The left and right arrow keys do special things, and sometimes  the  es-

cape key does as well.

For  letters, the keyboard is always in either default UPPERCASE mode or

default lowercase mode.  When in UPPERCASE, all letters typed  are  sent

out  as  uppercase.  In lowercase, all letters are sent as lowercase. To

reverse the case for  the  next  character  only,  hit  the  right-arrow

("prefix")  key. To switch the default case, hit the prefix-key twice in

a row.

For funny characters, the prefix key is also used  to  get  the  unusual

punctuation  characters  which  are not on the Apple keyboard. Here they

are:  (To represent the prefix character I am using the letter p).

To get                  Type    Appearence

Left Square Bracket     p(      [

Right Square Bracket    p)      ]

Left Curly Bracket      p<      {

Right Curly Bracket     p>      }

Underline               p-      _

Backslash               p/      \

Tilde (wiggle)          p^      ~

Vertical Line           p.      |

The left-arrow key sends a rubout.

With  left-arrow and right arrow doing special things, its a little hard

to enter their characters (^H and ^U respectivly). There is therefore an

escape from prefix mode sequence.  If  you  type  prefix-ESC,  the  next

character is sent without any interpretation.

Currently,  it  is  impossible to turn this special input and display on

and off, so its a bit of a pain if you are on a Apple 2e.

THE HELP COMMAND

Syntax: HELP

Typing HELP alone prints a brief summary of the KERMIT-65 commands.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit from KERMIT-65.  You can restart the program, provided you  haven't

run anything else, by typing 'CALL 2049'.

Syntax: QUIT

This is merely a synonym for EXIT.

THE SET COMMAND

Syntax: SET parameter [option] [value]

Establish  or  modify  various  parameters for file transfer or terminal

connection.  You can examine their values with the SHOW  command.    The

following parameters may be SET:

DEBUGGING       TERSE or VERBOSE packet information.

DEFAULT-DISK    Which Diskette drive is used for file transfer?

DEVICE-DRIVER   Which communication device is being used?

EIGHT-BIT-QUOTING

Should eight-bit-quoting be used?

ESCAPE          Character for terminal connection.

FILE-BYTE-SIZE  SEVEN or EIGHT significant bits in a byte.

FILE-TYPE       Of Apple DOS file being sent/received.

FILE-WARNING    Warn users if incoming file exists?

IBM             For communicating with an IBM mainframe

KEYBOARD        ][+ or //e keyboard.

LOCAL-ECHO      Full or half duplex switch.

PARITY          Character parity to use

RECEIVE         Various parameters for receiving files

SEND            Various parameters for sending files

SLOT            Which slot # is communication device in?

VT52-EMULATION  Should Kermit emulate a VT52 when connected?

SET DEBUGGING

Syntax: SET DEBUGGING options

Record the packet traffic on your terminal.  Options are:

TERSE           Show packet info only (brief).

VERBOSE         Display  packet  field  descriptions  with  packet  info

(lengthy).

OFF             Don't  display  debugging  information  (this   is   the

default).

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK parameter value

This  command  will  tell  KERMIT-65 which disk drive should be used for

file transfers. The two parameters which may be set separately are  SLOT

and DRIVE. The value for SLOT ranges from 1 to 7. The value for DRIVE is

either 1 or 2.

SET DEVICE-DRIVER

Syntax: SET DEVICE-DRIVER parameter keyword

This  command  will  tell KERMIT-65 what type of communication device is

being used. Currently, three different  cards  are  supported,  however,

other  unsupported  cards  may  work  similar enough to one of the three

available that it may be possible to use them. KERMIT-65  must  also  be

told  where  the card is in the machine (see the SET SLOT command).  The

options for this set command are:

APPLE-COM-CARD  The old Apple communication card (300 baud).

DC-HAYES        The D.C. Hayes Micromodem II (300 baud).

SUPER-SERIAL-CARD

The Apple Super Serial Card (300-19.2k baud).

SET ESCAPE

Syntax: SET ESCAPE hexidecimal-number

Specify  the  control  character you want to use to "escape" from remote

connections back to KERMIT-65.  The default is 0 (Control-@).  The  num-

ber  is  the  hex value of the ASCII control character, 1 to 37, for in-

stance 2 is Control-B.

SET FILE-BYTE-SIZE

Syntax: SET FILE-BYTE-SIZE parameter

Byte size for Apple  DOS  file  I/O.    The  choices  are  SEVEN-BIT  or

EIGHT-BIT.

SEVEN-BIT

When sending a file, shut the H.O. bit. When receiving, turn the

H.O.  bit  on.  This is done since text files are the only files

which should be sent as SEVEN-BIT files and text files only make

sense to the Apple if  they  are  encoded  in  'negative  ascii'

values.

EIGHT-BIT

Always  send  and  receive the bytes intact. All 8 bits are sig-

nificant.

SET FILE-TYPE

Syntax: SET FILE-TYPE parameter keyword

This will inform KERMIT-65 what type  of  DOS  file  is  being  sent  or

received.  It  is  important  that this is set correctly since KERMIT-65

must create a file of the appropriate type when receiving (and it has no

way of knowing what kind of file it is). When KERMIT-65 is  sending,  it

must  also  know  the type of file since that tells it how to detect the

actual end-of-file. The options for this parameter  are  APPLESOFT,  IN-

TEGER, TEXT and BINARY.

APPLESOFT

The file being sent/received is an Applesoft Basic program.

INTEGER The file being sent/received is an Integer Basic program.

TEXT    The file being sent/received is an ASCII Text file.

BINARY  The file being sent/received is a Binary image.

SET FILE-WARNING

Syntax: SET FILE-WARNING ON or OFF

This  tells  KERMIT-65 whether to warn the user about incoming filenames

conflicting with existing files or not.

SET IBM

Syntax: SET IBM ON or OFF

SET IBM actually sets a number of parameters. When setting  it  on,  the

command  also turns on LOCAL-ECHO (half-duplex) and sets PARITY to MARK.

When setting IBM OFF, LOCAL-ECHO will revert back to OFF and PARITY will

be set to NONE.  It should be used when doing file transfers with an IBM

or similar mainframe.

SET KEYBOARD

Syntax: SET KEYBOARD 2P or 2E

SET KEYBOARD tells KERMIT-65 if the user has a full keyboard (2E) or not

(2P).  If the user is on an Apple ][+, this should be set to  2P  (which

is  the  default).  When set to that, certain character translations are

available by using the right-arrow key as a prefix character.

SET SLOT

Syntax: SET SLOT parameter

This option tells KERMIT-65 in which slot the  communication  device  is

located. The range for the parameter is 1-7.

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

ALL             All parameter settings (this is quite long).

DEBUGGING       Debugging mode.

DEFAULT-DISK    Which Diskette drive is used for file transfer?

DEVICE-DRIVER   Which communication device is being used?

EIGHT-BIT-QUOTING

Should eight-bit-quoting be used?

ESCAPE          Character for terminal connection.

FILE-BYTE-SIZE  SEVEN or EIGHT significant bits in a byte.

FILE-TYPE       Of Apple DOS file being sent/received.

FILE-WARNING    Warn users if incoming file exists?

IBM             For communicating with an IBM mainframe

KEYBOARD        ][+ or //e keyboard.

LOCAL-ECHO      Full or half duplex switch.

PARITY          Character parity to use

RECEIVE         Various parameters for receiving files

SEND            Various parameters for sending files

SLOT            Which slot # is communication device in?

VT52-EMULATION  Should Kermit emulate a VT52 when connected?

The above options are analogous to the equivalent SET commands.

THE STATUS COMMAND

Give  statistics  about the most recent file transfer. This includes in-

formation such as number of characters  sent/received,  number  of  data

characters sent/received, and last error encountered.

1.5. Customizing, Building, and Installing KERMIT-65

STANDARD INSTALLATION

The Procedure

The  procedure to bootstrap an assembled KERMIT object file to the Apple

is as follows:

1. On the Apple, Type in the APPLBT.BAS  program  supplied  (See

below).  It is recommended that the user save this program as

it may be needed to bootstrap newer  versions  of  KERMIT  or

APPHXL  in  the  future.  Also, type the APPLESOFT program in

with none of the REMs. It will execute quicker  and  take  up

less room.

2. Call and login to the mainframe on which the KERMIT-65 object

file resides. Do the following:

- ]IN#n ! Where n is between 1 and 7

- For Communication card, do the following:

* Dial number for computer system.

* Seat phone receiver in modem cradle.

* ]<Cntrl/A><Cntrl/F> !Full duplex, 300 baud

- For the D.C. Hayes Micromodem, do the following:

* ]<Cntrl/A><Cntrl/F> ! Full duplex, 300 baud

* MICROMODEM II: BEGIN TERM <Cntrl/A><Cntrl/Q>

* MICROMODEM  II: DIALING: nnn-nnnn<cr> ! nnn-nnnn is

number of computer system

- For the Apple Super Serial Card, do the following:

* ]<Cntrl/A>

* APPLE SSC:0D ! 8 data bits, 1 stop bit

* ]<Cntrl/A>

* APPLE SSC:0P ! Force no parity

* ]<Cntrl/A>

* APPLE SSC:nB ! n = 6 for 300 baud, n = 8  for  1200

baud

* Dial number for computer system.

* Seat phone receiver in modem cradle.

* ]<Cntrl/A>

* APPLE  SSC:T ! Terminal mode, now talking to remote

host

3. In your directory  on  the  mainframe,  the  following  files

should be present:

- APPLBT.FOR

- APPHXL.HEX

- APPLEK.HEX

Compile  and execute APPLBT.FOR. This will be used along with

APPLBT.BAS on the Apple to load the APPHXL program. Once  AP-

PLBT  is executing on the mainframe, give control back to the

Apple and then run APPLBT.BAS on the Apple.  For  either  the

Communication  Card  or the D.C. Hayes Micromodem, the proce-

dure is:

- <Cntrl/A><Cntrl/X><cr> ! Give control to Apple's Brain

- ]LOAD APPLBT.BAS<cr>

- ]LOMEM:9500<cr>

- ]RUN<cr>

4. Relocate and save APPHXL. Type the following:

- ]CALL -151<cr> ! Enter Apple's system monitor

- *9000<2000.2280M<cr> ! Move APPHXL from $2000 to $9000

- *<Cntrl/C><cr> ! Reenter APPLESOFT

- ]BSAVE APPHXL,A$9000,L$280<cr> ! Save APPHXL to disk

5. Now simply start executing APPHXL.

- ]CALL -151<cr> ! Enter monitor

- *9000G<cr> ! Start APPHXL

- SLOT  FOR MODEM CARD? (1 TO 7 )n ! 'n' is slot # of card

(no <cr>)

- ENTER FILENAME TO LOAD APPLEK.HEX<cr> ! Tell APPHXL what

to load

APPHXL will print the byte count and load  address  for  each

line  it  is receiving as it loads it into memory. At the end

of each line, it will print '[OK]' if the line  was  received

properly  or  '[CHECKSUM  ERROR]' if there was a problem with

it.

6. When APPHXL finishes type the following to the Apple:

]BSAVE KERMIT,A$801,L$4E00<cr> ! Save KERMIT to disk

7. The user may set up a turn-key system  by  having  the  hello

file  on  the disk load and run KERMIT. The user may also run

KERMIT-65, change the defaults  which  are  supplied  in  the

program  such  as  SLOT  and  DEVICE-DRIVER,  and then resave

KERMIT-65.  The next time it is run, the user will  not  have

to  set  these  values  again.  If the user does not set up a

turn-key system, he must start KERMIT-65 by typing:

]BRUN KERMIT<cr>        ! Execute KERMIT-65 on the Apple

The Apple will display the following:

STEVENS/CU - APPLE ][ KERMIT-65  VER. 2.1

KERMIT-65>

The user is now ready to transfer files.

NOTE: For those users with the Apple Super Serial  Card,  the  <cntrl/A>

intercept  character should be shut off by typing <cntrl/A>Z to the card

before Kermit is run. The reason for this is that Kermit uses  <cntrl/A>

as  a  Start-of-Header  character and this character may never reach the

program if the card is taking it to be a command.

APPLBT.BAS - The APPLESOFT Bootstrap program

10  REM  - LOADER FOR HXLOAD

11  OAD = 0

100 N$ = "0123456789ABCDEF"

110 D$ =  CHR$ (4)

130  PRINT D$;"IN#2" : REM CHANGE '2' TO SLOT OF COMM. CARD IF NECESSARY

135  PRINT  CHR$ (1); CHR$ (6)

136  PRINT D$;"PR#2" : REM CHANGE '2' TO SLOT OF COMM. CARD IF NECESSARY

137  PR#2  : REM THIS LINE SHOULD BE HERE ONLY FOR THE APPLE COMM. CARD

140 C3 = 0

150  HOME

199  REM  - REQUEST NEXT LINE

200  REM - PUT A DOT ON THE SCREEN FOR EACH LINE RECEIVED

201 C3 = C3 + 1: POKE 1024 + C3, ASC (".")

202 L$ = "":Y2% = 1: PRINT

203  GET A$:L$ = L$ + A$:Y2% = Y2% + 1: IF Y2% < 81 THEN 203

205 C1 = 0:C2 = 0:I = 0

208  IF  LEFT$ (L$,1) >  = "0" AND  LEFT$ (L$,1) <  = "9" THEN 220

210 L$ =  RIGHT$ (L$, LEN (L$) - 1): GOTO 208

220 LL =  LEN (L$)

249  REM - FETCH THE DATA BYTE COUNT FOR THIS LINE

250  GOSUB 1000:C1 = C1 + B:CO = B

255  IF CO = 0 THEN 990

259  REM - CONSTRUCT THE LOAD ADDRESS FOR THIS LINE

260  GOSUB 1000:C1 = C1 + B:AD = B: GOSUB 1000:C1 = C1 + B

:AD = AD * 256 + B

265 REM - IF THE LATEST VERSION OF CROSS IS USED, THIS SHOULD NOT

BE NEEDED : REM AD = AD - 28672

266  IF AD < OAD THEN 990

267 OAD = AD

270  FOR X = 0 TO CO - 1

275  REM - GO GET A BYTE AND PUT IT IN THE NEXT MEMORY LOCATION

280  GOSUB 1000:C1 = C1 + B

290  POKE AD + X,B

300  NEXT X

310  GOSUB 1000:C2 = B: GOSUB 1000:C2 = C2 * 256 + B

320  IF C1<>C2 THEN POKE 1024+C3,ASC("E")

330  GOTO 201

990  FOR X = 1 TO 1000: NEXT X

995  PRINT D$;"IN#0": PRINT D$;"PR#0": HOME : END

999  REM  - GET BYTE

1000  GOSUB 1501:B = N1: GOSUB 1501:B = B * 16 + N1

1010  RETURN

1500  REM - GET NIBBLE

1501  IF  LEN (L$) = 0 THEN N1 = 0: RETURN

1510 H$ =  LEFT$ (L$,1)

1511  IF  LEN (L$) = 1 THEN L$ = "": GOTO 1525

1515 L$ =  RIGHT$ (L$, LEN (L$) - 1)

1520  REM - RETURN VALUE OF HEX NIBBLE

1525  FOR X1 = 1 TO 16

1530  IF H$ =  MID$ (N$,X1,1) THEN 1610

1540  NEXT X1

1550 REM - DIGIT WAS NOT FOUND, RETURN ZERO

1560 N1 = 0: RETURN

1600  REM

1610 N1 = X1 - 1: RETURN

APPLBT.FOR - The Mainframe Bootstrap program

CHARACTER LINE*80,SENTNL*1

OPEN (UNIT=00,FILE='APPHXL.HEX',MODE='ASCII')

10 READ (UNIT=05,FMT=20) SENTNL

20 FORMAT (A1)

READ (UNIT=00,FMT=25,END=999) LINE

25 FORMAT(A80)

WRITE (UNIT=05,FMT=30) LINE

30 FORMAT(A80)

GO TO 10

999 READ (UNIT=05,FMT=20) SENTNL

STOP

END

ALTERNATIVE INSTALLATION PROCEDURES

HEXloading from Diskette

Once  the  user has a working version of KERMIT-65 on his system, he can

use this to load the .HEX files of future versions of KERMIT.  There  is

another  hexload program available called APPDXL. This program will load

a hex file from an Apple diskette into memory. To use this procedure, do

the following:

1. Start executing APPHXL.

- ]BLOAD APPHXL ! Get APPHXL into memory

- ]CALL -151<cr> ! Enter monitor

- *9000G<cr> ! Start APPHXL

- SLOT FOR MODEM CARD? (1 TO 7 )n ! 'n' is slot # of  card

(no <cr>)

- ENTER FILENAME TO LOAD APPDXL.HEX<cr> ! Tell APPHXL what

to load

APPHXL  will print what it is receiving on the screen as well

as loading it into memory.

2. Relocate and save APPDXL. Type the following:

- ]CALL -151<cr> ! Enter Apple's system monitor

- *9000<2000.2500M<cr> ! Move APPDXL from $2000 to $9000

- *<Cntrl/C><cr> ! Reenter APPLESOFT

- ]BSAVE APPDXL,A$9000,L$500<cr> ! Save APPDXL to disk

3. Use  Kermit-65  to  transfer  the new version of itself over.

Make the Apple file a Text file. WARNING: This file will take

LOTS of space (about 180 sectors) so make sure  the  disk  is

reasonably  empty.    The transfer takes a LONG time also, so

please be patient.

4. Start executing APPDXL.

- ]BRUN APPDXL<cr> ! Start APPDXL

- ENTER FILENAME TO LOAD APPLEK.HEX<cr> ! Tell APPDXL what

to load

5. When APPDXL finishes type the following to the Apple:

]BSAVE KERMIT,A$801,L$4E00<cr> ! Save KERMIT to disk

The new version of Kermit is now on disk.

Using KERMIT-65 to transfer APPLEK.BIN

There is yet another way to Bootstrap a new version of  KERMIT  onto  an

Apple. If the user has an older version of KERMIT-65 and has access to a

machine  with  a  valid  copy  of  APPLEK.BIN,  they can simply transfer

APPLEK.BIN  using  their  version  of  KERMIT.  Be  sure  to   set   the

File-byte-size  to  Eight-bit,  and  the File-type-mode to Binary before

transfering the file since this is the actual object  code.  No  special

loading or conversion is needed. The file will be placed on the disk and

ready to run.

FILES SUPPLIED FOR KERMIT-65

The following files should be supplied on the distribution tape:

- APPLBT.BAS - Initial bootstrap program to load APPHXL

- APPLBT.FOR - Program on mainframe to talk to APPLBT.BAS

- APPHXL.M65 - Source of program to load KERMIT-65

- APPHXL.HEX - Assembled version of Hex load program

- APPDXL.M65  -  Source  of program to load KERMIT-65 from Apple

diskette

- APPDXL.HEX - Assembled version of Disk Hex load program

- APPLEK.M65 - Source for the KERMIT-65 program

- APPLEK.HEX - Assembled version of KERMIT-65

- APPLEK.BIN  - Assembled version of KERMIT-65 (Eight-bit Binary

object code)

- CROSS.MAC - CROSS Microprocessor Assembler (Source)

- CROSS.EXE - CROSS Microprocessor Assembler (Object)

CUSTOMIZING AND BUILDING KERMIT-65

The source code to KERMIT-65 is in 6502 Assembler. It has been formatted

for CROSS which  is  a  micro-Cross  Assembler  program  which  runs  on

DECsystem-10s  and  DECSYSTEM-20s.  Customizations  would  be  made  the

easiest if CROSS were available.

KERMIT-65 currently supports the following communications devices:

- FTASER - The Apple Communication card

- FTHAYS - The D.C. Hayes Micromodem.

- FTSSC - The Apple Super Serial Card

All device drivers are included in the assembled version and may be used

by issuing a 'SET DEVICE-DRIVER' command to Kermit. If any of the device

drivers are not needed, it(they) may be  excluded  by  setting  the  ap-

propriate feature test to zero in the Feature test section of the source

code.  Excluding  one  or more device drivers can reduce the size of the

object code greatly. DO NOT disable all device drivers  since  KERMIT-65

will then have no way of talking over the communication device.

The  feature  test  FTCOM  must be set to the type of computer for which

KERMIT-65 is being assembled. The only machine  KERMIT-65  is  available

for currently is the Apple ][. This parameter must be set to FTAPPL.

After  setting  any  options  necessary  in  APPLEK.M65,  rename  it  to

KERMIT.M65, and do the following:

- .R CROSS<cr> ! Run CROSS Microprocessor Assembler

- *KERMIT.HEX/PTP:KIM=KERMIT.M65/M65 ! Generate .HEX file

This command will produce an ASCII HEX file which can be downline loaded

onto the Apple using APPHXL.  If  a  listing  is  desired,  one  can  be

produced  by  adding  ",KERMIT.LST"  after the "/PTP:KIM" in the command

line to CROSS.

Index

?-prompting   4

APPLE ][+   1

APPLESOFT   10

BINARY   10

Byte Size   10

CONNECT   7

Debugging   9

DEFAULT-DISK   9

DEVICE-DRIVER   9

DOS 3.3   1

Escape Character for CONNECT   10

EXIT   8

FILE-BYTE-SIZE   10

FILE-TYPE   10

FILE-WARNING   11

Filespec   5

GET   6

Help   4, 8

IBM   11

INTEGER   10

KEYBOARD   11

QUIT   8

RECEIVE   6

Recognition   4

SEND   5

SHOW   11

SLOT   11

TERSE   9

TEXT   10

VERBOSE   9

Table of Contents

1. APPLE-DOS KERMIT                                                    1

1.1. The DOS 3.3 File System                                           1

1.2. Program Operation                                                 4

1.3. Remote and Local Operation                                        5

1.4. KERMIT-65 Commands                                                5

1.5. Customizing, Building, and Installing KERMIT-65                  12

Index                                                                 19

######################################################