Instruction : STAR Converter Program CON24.COM

----------------------------------------------

1. How it works

The converter is a memory resident program, rooted to the existing printer

driver. It serves as a 24- needle driver as soon as the STAR NB-2410/15

is turned into grphic mode. In normal mode (text,download etc.) the driver

does not effect the data going to the printer.

2. Installation

The converter programm must be installed like the keyboard driver. This is

done by typing CON24 when being on DOS level. It is more convenient to put

CON24.COM into the AUTOEXEC.BAT file, so during system start or boot the

driver gets installed automaticly.

Anyway, after installing successfully following message appears on the

screen : "Residente Konvertierung installiert V1.x STAR/hmb 1987".

If it is necessary to disable the driver just type CON24 OFF, now the pre-

viously occupied memory is released.

3. DIP switch settings for NB 2410/15

The converter supports both IBM- emulation modes, position DIP switches

as followed ('X' stands for a random setting according handbook) :

switch #  position

S 1-1       x

S 1-2       ON

S 1-3       x

S 1-4       x

S 1-5       OFF

S 1-6       ON

S 1-7       ON

S 1-8       ON

S 2-1       IBM-P : ON    IBM-G : OFF

S 2-2         "   : OFF     "   : ON

S 2-3       x

S 2-4       x

S 2-5       ON

S 2-6       ON

S 2-7       x

S 2-8       x