HAMCOM

Author: Dan Diehlman AE6G

5478 N. Bond

Fresno, CA 93710    209-439-5520

Note: The author of this program has expended considerable time and energy

in writing, testing and debugging of this program. Each copy is

customized for one call sign. If you would like to buy a registered copy,

contact me at the above address. You will then receive also any

corrections/updates. I strongly suggest that you do not use an

unregistered copy. In addition, any attempt to use the program with

another's (registered) call and validation number will result in

the TNC, occasionally, and without notice, loading the registered

owner's call.*** Thus the registered owner assumes the responsibility

of not distributing the program to others.***

It has, I believe, a couple of unique features, one being the QSO

card file which enables you to do away with any card file and gives

practically instantaneous response on Call Sign searches. The other

feature is the low RFI(noise) generated. This is very noticeable on

HF. During inactivity, the processor chip is put into a full halt

condition, thus eliminating birdies caused by instruction loops.

This was made possible by the fact that the program is completely

interrupt driven, therefore it can be put to sleep, then awakened by

a keystroke or communications activity. This was written to take

advantage of a PK232 -IBM/clone configuration, with 128K or more.

Although it can be used with other TNC's, it's full capabilities will

not be realized.

RUNNING THE PROGRAM

There are two files initially, this file "Ham.doc" and the program itself

"Hamcom".  The program will create other files, and keep them updated as

necessary. It uses the default diskette.

To run the program, copy the two files "Ham.doc" and "Hamcom.exe" onto a diskette

which already has Dos on it.

Then type "Hamcom"

The first time you run the program, it will ask for information necessary

for customization as follows:

prompt                       reply

What is your validation number?  Insert your val number

What type TNC are you using?     Appropiate number.

What is your Call?               Your call

What is your Selcall?            Your Selcall (if you have not determined your

selcall, see 232 ref manual.)

Is this a PCJR? y/n             y or n

Which Comport 1 or 2 ?          1 or 2

Select type of Display Adapter  1-4

Are you using a Color Display?  y or n       n for monochrome monitors

y for any color monitors

Enter your CTEXT.Use > for CR   Your connect message. If you wish to imbed any

Carriage Returns, use the ">" symbol.

Enter your BTEXT.Use > for CR   Your beacon text.

Which Drive for default         Enter A,B,C or whatever drive you wish to be

the default drive for logging, receiving, and

sending files. It can be overridden in any

event when specifying a file i.e. B:xxx.xxx

Baud rate between TNC and computer.  Select any of the listed speeds. 1200 is

best. (You should use 1200 baud with the PK232)

Customized Packet Ending        This allows you to enter a special characters

which will be appended to your packets when

you release it with the "]" key. I set mine for

"==>" but set it for BK or whatever you like.

Special Parms:                   Opportunity to set additional/different

parameters. Separate them with a slash.

such as:    monitor on/txd 50

Do you have a 440-940 connected. Discussed later.

Is your primary output a Voice Synthesizer Y/N? Normally select "N". If "Y"

is selected then the screen is written thru

BIOS calls so that voice synthesizers can

intercept. Also assists the synthesizers in

pronouncing words instead of spelling the

words.

On subsequent runs of the program, you will not have to enter this information.

It will be preserved in a file.  If, in the future, you desire to change these

parameters, erase the file Hamparm.*". Then run the program, re-answering the

questions, with the new information.

The program will then proceed and stop on the main display, with the heading

"The following data was stored in the TNC". Below it will be listed any

connects you received, while your computer was off.

This data ends with the message "1: Proceed with TNC reset" and "2:Proceed

without TNC reset". The first time you run the program, use "1". This will

set certain parms in the TNC. On subsequent runs, if the TNC has not been

powered off, you can use "2".

After you depress a key the program will load it's other files , ending up

back on the Main Screen.

The Main Screen displays any text from the TNC above the horizontal line, and

any text that you type is displayed below the line. If you would like to

change the line position, use the "Alt D" to raise or lower the divider.

The lower left hand corner displays the current Mode of the TNC.

The lower right of the screen displays the status of whether you are also

logging to diskette, the data received from the TNC. At the far right on the

bottom of the screen, a number (0-99) will appear on some TNC's. This number is

an indication of how many connects you have had. It is reset to 0 by going to

the Scroll Back Buffer.

Also on the bottom line you can display Help in any mode by depress "Alt H". It

can be turned off in the same manner.

The Main Screen has the feature of being able to scroll back thru the received

text. To use this feature: use the "Page Up" key to scroll back further. If

you reach the top of the buffer, a message at the top of the screen will say

"Top of Buffer:". You can at any point, scroll back down through the buffer

using the "Page Down" key, or the "Home" key to get to the top of the buffer,

or you can return to the Main Screen by depressing the "Spacebar". At any

time, while in the Scroll Back Buffer, you can depress "Alt M" for a menu.

While in the Scroll Back Buffer, you can "Mark" the beginning and "End" of

a block of text, and then either "Send" the block or "Write" it to a file.

Notepad: You can reach the Notepad screen from the Main Screen by depressing

"Alt N". This screen is useful for any notes. I use it for recording tuner

settings etc. The only keys in effect on this screen are the regular alpha/

numeric keys and the Cursor positioning keys. Anything written onto this

screen will be preserved in a file and read back in automatically whenever

you rerun the program. You may exit this screen with the "ESC" key.

Menu: You can recall the Menu screen from the Main Screen by depressing "Alt M"

On this screen are listed all the functions available from the Main Screen.

F11 through F20 are invoked by using the regular function keys along with

the Shift Key. i.e. F12 is called by using "Shift F2". Similarly F21 is called

by using the Ctrl Key along with F1, additionally F31 is called by using the

Alt key long with F1.

F1-F30: This is the User Macro function

Depressing F1 to F30 will execute Macros 1 to 30.

Initially all the macros will be empty. To create a macro depress

"Alt F". This will take you to a screen which displays the macros,

and gives you the opportunity to enter any macro. The screen is

self explanatory.

examples: "c w1aw>"  will initiate a connect to w1aw.

"m 4> mfrom yes all> mto yes all>" will set monitor to 4 etc.

F31: This will pull down a window and allow you to select different modes

on those TNC's that have different modes.

F32:Log to Disk. Initially the received data is being logged to "Logdat.1"

as indicated in the lower right hand part of the Main Screen.

This key will pull down a small window, allowing you to stop/restart a

new log, or to display the previous log, or to view any ASCII file.

F33: This will set the Date and Time in the TNC from the Dos clock.

F34: Allows you to Create/Send a Brag tape, the size is limited only by

Available disk space.

F35:File Transfer. This will send/receive any Ascii or Binary file. Binary

will only work with other Hamcom users because of protocol differneces.

You will be prompted for the name of the file to send. A "File Xfer in

progress" message will be posted and at the completion a "File Xfer

Complete" message will be posted when all the data has been sent from

the computer to the TNC. Note: There may be remaining packets yet to be

sent and acknowledged in the TNC before the xfer is fully complete.

NOTE: To do Binary Transfers, you must have a Computer to TNC cable

that has pins 1-8 and 20. You cannot have just (TD,RD and ground).

(N O T E   T H I S   I S   V E R Y    I M P O R T A N T)

F39:Tone. This key toggles a tone on/off to help the visually impaired to

tune in data modes on hf. It sounds a 2100 hz note in all modes except

for Morse, in which mode, it sounds a 800 hz note.

F40:This will cause the received data to also go to the printer. Successive

depressions of this key toggles the feature On/Off.

Alt B:Sound. This pulls down a menu which gives you control over the bell etc.

The soft bell can be selected or the regular bell, which is the default.

The soft bell is rather nice when you are in the room, and don't require

the harshness of the loud regular bell.

Alt E:End Program. Always use this to exit the Program. It will insure that all

necessary files are updated, and that the tnc is put in the proper state

to receive connect messages, and also set monitoring to 0.

Alt M: Menu.  Takes you to the Menu Screen from the Main Screen.

Alt N: Notepad. Takes you to the Notepad Screen, already discussed

Home: Clears the Main Screen.

Del: When using a PK232, this clears the transmit buffer in all modes.

Alt F: Setup Macros....previously discussed

Alt C: Sends CQ-automtically adjusts to whatever mode is in effect.

Alt H: Toggles On/Off a summary of commands at the bottom of the Main Screen.

Alt K: This clears the Scroll Back Buffer.

The program is set up so that the Esc key duplicates the Ctrl C

function, thus lessening the number of keystrokes necessary therefore

whenever you want to enter a Ctrl C, just hit the ESC key.

The Tab key is set up in packet to do Channel Switching more easily.

For instance, if you are in converse mode, in packet, and you wish to

switch to another channel, instead of typing Ctrl C,Chs X, K simply

type "Tab" and the number of the channel you are going to. It will

switch you to that channel and return you to converse mode.

The program is set up so that the "Enter Key" does send the packet.

Or you may use the "right bracket" key to send the packet with your

customized ending. (substitute for BK or K or whatever).

Alt S:  Shows the current state of all packet streams.

Incidentally, when in a multiple connection situation, any change in

the received data stream, will automatically insert a solid line

accross the screen to alert you to the change.

Alt Z:  Allows you to create messages for individual callers. Allows

personalized connect messages.

Baudot: self explanatory

Ascii:  self explanatory

Morse:  self explanatory

Amtor:  normal operation except for turnaround, type only the "+". It is not

necessary to type the "?" also.

The "Tab" key has been setup to force Turnaround. It is the equivalent

of going to control mode, typing ACHG, enter, k, enter. Again, we are

trying to save unnecessary keystrokes.

FAX:    selects Fax mode, and turns Prcon On.

Alt L      This is a very usefull feature in that it is not necessary to

maintain a card file on your contacts. You can quickly go from

the Main Screen to search on Calls, and return, knowing the

calling party's name,qth, etc.

To implement this feature, type "Alt L".

You will be given several choices, however, until you have

created a database using the "Create" option, none of the other

functions will actually do anything. So intially choose.

"Scratch and Create DATABASE". It will then ask you to type the

word "CONFIRM" (must be in upper case), to proceed. The reason

for this added entry, is to prevent any accidental erasure. I

have a 128K file, and would hate to lose mine accidentally.

Enter a couple calls and the other requested information and then

do some searches to become familiar with this feature. A search

on a call is almost instantaneous with 1500 entries in the file.

Searches on QTH's, Activity(dates), takes longer as it must

sequentially go through the file to search for matches.

An "*" asterisk, limits the field searches, such as searching on

QTH thusly "TO*" will list all contacts with Tokyo and Toledo  and

Toocumcari if Toocumcari were spelled that way.

An Activity search such as "12/*/86" will list all qso's in Dec of

86. etc.

A Call Search such as "W3*" will list all qso's with W3's.

Alt P:     DOS SHELL. This feature allows you to temporarily leave the

program and go to DOS and run whatever you like. Whenever you

are finished, and have the Dos prompt, type "EXIT" and you will

return right back into the ham.com program, as though you had never

left. During your absence, the TNC will store any incoming messages

and give them to you on your return.

Alt T:     This is only active if you have stipulated in the configuration

the presence of a Kenwood 440 or 940 and the Kenwood inteface.

Selecting this function, will take you to a screen which

has a simulated transceiver dial, displaying the frequency, offset

modes, etc. The options are listed at the bottom of the screen.

To return to the Main Screen, depress Escape.

Naturally, it would be wise to backup your files occassionally.

If for some reason you need to include an ESC character ($1B or dec 27),

the reverse apostrophe will send that character.

On some of the Menus, choices are follwed by an asterisk. This does not

indicate the present state, but rather the default selection.

If you are transferring a physical card file into the program's card file,

it is best not to enter them in a sorted order, but rather in a harem/scarem

order. Because of the search algorithm, search times will be reduced.

When using a PK232 in Morse, Baudot, Ascii modes, you can type ahead the

text you want to send, even while receiving, and when you actually want

transmission to begin, enter a "ctrl X".

I would appreciate any criticism . Thank You

Hope you enjoy using the program.

***** NOTE: PLEASE DO NOT USE WRITE PROTECT TAB ON YOUR WORKING DISK

HAMCOM NEEDS TO WRITE SEVERAL FILES TO THIS DISK

-------------------------------------------------

Dan Diehlman AE6G

5478 N. Bond

Fresno, CA 93710

209-439-5520

#                       5478 N. Bond

Fresno, CA 93710