Documentation for operating SIMPLEX -- nonlinear curve fitting routine

The coding of the simplex procedure is from an article in Byte

1984 in the May issue written by Caceci and Cacheris.  The article

describes the SIMPLEX algorithm.  It has been adapted for TURBO PASCAL

V3.01 by Tom Dean, Dept. of Forest Resources, Utah State University.

The plotting routines and the bootstrap variance estimate routine was

added by Tom Dean, 5 Dec 1985 at Utah State University.

Simplex requires three files to be present on the default or

logged drive: SIMPLEX.PAS, NONFUNC.PAS, AND PARMS.DAT.  The data file

can be on any drive.  SIMPLEX is the main program. NONFUNC contains the

function definition and PARMS.DAT has the user specified control

parameters.  To run the program the function is specified in NONFUNC

after "f:=".  Two varibles which are arrays are used in the function

definition.  x[n] is the parameters to be fitted. d[m] is the data. N

ranges from 1 to the number of estimated parameters.  M ranges from 1

to the number of independent variables.  For example, the function

Y = A X1 + B(X2 X3)**C

would look like in NONFUNC

f:= x[1]*d[1] + x[2] * exp(x[3]*ln(d[2]*d[3]));

After the function has been specified and NONFUNC saved, three

constants need to be defined in SIMPLEX to match the function.  The

first specifies the number of parameters to be fitted-for this example,

this would be 3.  The second states the number of values, including the

dependent variable, present on each record, in this case 4.  The third

is simply the number of parameters plus one, here 4.  This is saved and

PARMS.DAT is edited. This file consists of 4 lines.

LINE 1: maximum number of iterations. 0=infinite.  The program always

converges, so I use 0.  If nothing is known about the relation between

the function and the data, I would use 100 to start with.

LINE 2: guesses at starting values for each parameter.  If the log will

be taken of a parameter it must not be zero.

LINE 3: starting increments for each parameter.  I usually use one

order of magnitude lower than starting guesses.  If an overflow error

occurs, I decrease the increments. Small increments will usually

increase the number of iterations needed to converge.

LINE 4: convergence criteria for each parameter plus one for the sum of

squares of the residuals.  I use 1e-4.  If a parameter has trouble

meeting the convergence criteria, I will use 1e-2.  I have never seen

any need to use smaller criteria than 1e-4.

The program is ready to run.  The program expects the data file to

be ASCII and to have the dependent variable as the last item in each

record.  It first asks for the data file name and will keep asking

until a valid name is entered.  It asks if residual plots and plots of

observed and predicted are needed.  If files of residuals and fitted

1

values are indicated then two files are created with the extension

'.err' and '.fit' appended to the file name.  These have the original x

values and either the residual or predicted Y in place of the original

dependent values.  These can be used in other programs.  If output is

also to go the printer a title is asked for.

Another feature that is kind of buried at the last of the program

is the ability to produce a file with parameter estimates derived from

the bootstrap technique.  From this file variance estimates of the

fitted parameters can be obtained.  To turn this feature on set the

constant 'variance' (fourth line in SIMPLEX.pas) to true, but beware,

without a math coprocessor this can take forever to run.  In brief, the

bootstrap technique samples with replacement from the original data to

produce a new file with the original number of observations.

Parameters are estimated from this data set and stored in a file named

b:BOOTSTRA.  This is repeated 100 times.  The standard deviation of the

100 repetitions for each parameter is a nearly unbiased estimator of

its standard error.  This procedure requires a file called 'SAMPLE' on

the a: drive that has 100 data sets of numbers each set with a range

equal to the original number of observations.  For example, if the data

file has 10 records, then a random sample with replacement is drawn

from 1 to 10 so that the first data set might consist of

1,5,6,3,8,4,7,4,10,5.  This would select those record numbers from the

original data file to run simplex on.  There are 100 such data sets,

one after another, in the file named SAMPLE.  This program expects the

SAMPLE file to have this format:

DATA SET  1

1 5 6 3 8 4 7 4 10 5

DATA SET  2

2 5 8 4 2 1 7 3 5 2

DATA SET 3

etc.

I use a statisical package named MSUSTAT to generate the SAMPLE file.

MSUSTAT is available from R.E. Lund at Montana State University in

Bozeman, Montana for about $200.  It will also do the simple statistics

to calculate the standard deviations.  If you do not have some program

to generate this file, you will need to write a uniform random number

generator that will sample with replacement from a given range of

numbers.

2