5/19/84           DOCUMENT FOR  EQUIP102.COM

EQUIP102.COM IS DESIGNED TO OPERATE ON AN  80 COLUMN DISPLAY.

EQUIP102.COM CHECKS THE SYSTEM AND DISPLAYS THE FOLLOWING INFORMATION

AS REFLECTED BY THE SETTINGS ON THE SYSTEM BOARD.

EQUIP102.COM IS A MODIFIED VERSION OF EQUIP101.COM. EQUIP102.COM SHOULD NOW

DISPLAY DRIVES THAT HAVE BEEN ADDED AS DEVICE DRIVERS UNDER CONFIG.SYS.

ANY DRIVES PRESENT THAT ARE NOT CONFIGURED BY THE SWITCHES OR THE

CONFIGURATION CHANGED BY THE EXTERNAL RAM-DRIVE INSTALLING PROGRAM SHOULD

APPEAR AS AN UNSCHEDULED DISK DRIVE.

Equipment Installed on your   IBM - PC

MEMORY on System Board   :        64 K              - - - FLOPPY DISK - - - -

| DRIVE SELECTED * * * * A |

TOTAL MEMORY in System   :       320 K             | SIDES on Disk:         2 |

| SECTORS/TRACK:         9 |

Number of DISK DRIVES    :   2 + 1 UNSCHEDULED --->| BYTES/SECTOR :       512 |

| BYTES/DISK   :    362496 |

Number of PRINTER CARDS  : 1 <Parallel Printers>   | BYTES-FREE   :     79892 |

- - - - - - - - - - - - -

Number of RS-232 CARDS   : 2 <Async. Communications>

COLOR GRAPHICS VIDEO CARD:

NO GAMES ADAPTER CARD    :

IBM PC-DOS VERSION: 2.10

R O M  Release Date : 10/19/81

A DESCRIPTIVE REPORT IS GIVEN ON THE SELECTED DRIVE.

THE SELECTED DRIVE CAN BE CHANGED BY GIVING THE DRIVE DESIGNATION AT

THE TIME THE PROGRAM IS EXECUTED:  Example   EQUIP102 B

IF THE SELECTED DRIVE IS A FIXED DISK THEN INFORMATION ABOVE THE DRIVE

DESCRIPTION WILL SHOW  FIXED DISK   INSTEAD OF   FLOPPY DISK

IN THE EQUIPMENT LIST WHERE THE EXAMPLE SHOWS COLOR GRAPHICS VIDEO CARD:

IF THE MONOCHROME CARD IS INSTALLED IT WILL SHOW THAT INSTEAD.

WHERE THE EXAMPLE SHOWS  NO GAMES ADAPTER CARD   :. IF A GAMES ADAPTER

IS INSTALL=0000  SI=0000  DI=0000

DS=373A  ES=373A  SS=373A  CS=373A  IP=0100   NV UP DI PL NZ NA PO NC

373A:0100 E9FB02        JMP     03FE

|__________________________________>THIS NUMBER WILL BE DIFFERENT FROM

ONE MACHINE TO ANOTHER. IT SHOULD BE

THE SAME AS THE NBR. IN THE CS REGISTER

-S CS:100 6AC C6 06 07 01 07 -------->PRESS ENTER

| |   |   |  |____________|_________>CODE WERE LOOKING FOR.

:

PLACE A DISK CONTAINING  DEBUG.COM  IN THE DRIVE AND TYPE IN  DEBUG  AND

PRESS ENTER. TAKE THIS DISK OUT AND PUT IN A DISK CONTAINING A BACKUP OF

OF THE PROGRAM  EQUIP102.COM.

TYPE IN   N EQUIP102.COM   AND PRESS ENTER

TYPE IN   L                AND PRESS ENTER

EQUIP102.COM IS NOT LOADED INTO DEBUG. FOLLOW THE INSTRUCTIONS FOLLOWING

THE TWO DRIVE INSTRUCTIONS.

TWO DRIVE:

PLACE A DISK CONTAINING  DEBUG.COM  INTO DRIVE A. PLACE A DISK CONTAINING

A BACKUP OF  EQUIP102.COM  INTO DRIVE B.

TYPE IN  | |   |   |_________________________>VALUE FROM CX REGISTER.

| |   |_____________________________>VALUE FROM IP REGISTER.

| |_________________________________>REGISTER WERE SEARCHING.

|___________________________________>COMMAND TO SEARCH A RANGE.

373A:05F1 --------------------------->PRESS ENTER

|    |______________________________>LOCATION WHERE CODE WAS FOUND.

|___________________________________>SAME AS NBR. IN CS REGISTER.

-U 5F1 ------------------------------>PRESS ENTER

|  |_______=0000  SI=0000  DI=0000

DS=373A  ES=373A  SS=373A  CS=373A  IP=0100   NV UP DI PL NZ NA PO NC

373A:0100 E9FB02        JMP     03FE

|__________________________________>THIS NUMBER WILL BE DIFFERENT FROM

ONE MACHINE TO ANOTHER. IT SHOULD BE

THE SAME AS THE NBR. IN THE CS REGISTER

-S CS:100 6AC C6 06 07 01 07 -------->PRESS ENTER

| |   |   |  |____________|_________>CODE WERE LOOKING FOR.

_________________________>LOCATION RETURNED BY THE SEARCH.

|___________________________________>COMMAND TO UN-ASSEMBLE A SECTION OF

>OF CODE.

373A:05F1 C606070107    MOV     BYTE PTR [0107],07

|            |                             |__>COLOR CODE.B&W

|            |________________________________>COLOR CODE.B&W

|_____________________________________________>LOCATION FOUND

_________________________>LOCATION RETURNED BY THE SEARCH.

|___________________________________>COMMAND TO UN-ASSEMBLE A SECTION OF

>OF CODE.

373A:05F1 C606070107    MOV     BYTE PTR [0107],07

|            |                             |__>COLOR CODE.B&W

|            |________________________________>COLOR CODE.B&W

|_____________________________________________>LOCATION FOUND

>BY SEARCH.

373A:05F6 A10501        MOV     AX,[0105]

373A:05F9 8EC0          MOV     ES,AX

373A:05FB B80006        MOV     AX,0600

373A:05FE B90000        MOV     CX,0000

373A:0601 BA7924        MOV     DX,2479

373A:0604 8A3E0701      MOV     BH,[0107]

373A:0608 CD10          INT     10

373A:060A B80002        MOV     AX,0200

373A:060D 33DB          XOR     BX,BX

373A:060F BA0016        MOV     DX,1600

-E 5F1 ------------------------------------------>PRESS ENTER

|  |____________________________________________>LOCATION WE WANT TO EDIT.

|_______________________________________________>EDIT COMMAND.

373A:05F1  C6.     06.     07.     01.     07.1E --->PRESS ENTER

|       |       |       |       |       |_____>ENTER COLOR CODE. SEE

|       |       |       |       |             >NOTE BELOW FOR COLORS.

|       |_______|_______|_______|_____________>HIT SPACE BAR TO ADVANCE

|                                             >TO THE COLOR BYTE.

|_____________________________________________>EDIT LOCATION

-W ----------->PRESS ENTER

|____________________________>COMMAND TO WRITE THE FILE BACK TO DISK.

Writing 06AC bytes

-Q ----------->PRESS ENTER

|____________________________>QUIT COMMAND   RETURN TO DOS.

NOTE:   THE FOLLOWING INSTRUCTION ENTERS THE CODE FOR THE COLORS YOU WANT

BOTH BACKGROUND AND FOREGROUND. THIS DOES NOT INCLUDE A BORDER.

THE BORDER COLOR IS BLACK.

THE FIRST DIGIT  1  IN  1E   IS FOR THE BACKGROUND COLOR. THIS CAN

ANY COLOR ALLOWED FOR THE BACKGROUND  0 - 7.

THE SECOND DIGIT  E  IN  1E  IS FOR THE FOREGROUND COLOR (COLOR

OF THE LETTERS)  0 - 7  FOR REGULAR INTENSITY COLORS, AND  8 -

F  FOR HI-INTENSITY COLORS. REMEMBER  HEXIDECIMAL NUMBERS  A -

F  EQUAL  DECIMAL NUMBERS  10 - 15.

1E   WILL GIVE YOU A REGULAR INTENSITY BLUE BACKGROUND WITH

HI-INTENSITY YELLOW FOREGROUND.

TYPE IN   1E         AND PRESS ENTER

YOU SHOULD NOW BE BACK TO THE REGULAR SYSTEM PROMPT. RUN THE PROGRAM TO

MAKE SURE IT IS WHAT YOU WANT. NOTE: THE COLOR MAY OR MAYNOT WORK CORRECTLY

IF YOU HAVE  ANSI.SYS  INSTALLED AND RUNNING ON YOU SYSTEM. TRY IT AND SEE.

EVERETT DELANO

224 HOOVER CIRCLE

ELK CITY, OKLA. 73644

############################################################################