******************************************************************************

*				IMPORTANT				     *

*									     *

*	If you have:							     *

*		a) a Compaq Video Graphics Controller Board with	     *

*		   ROM revision A.2 or less, and,			     *

*		b) a MicroSoft Mouse with Version 6 of the driver	     *

*	then you have a potentially serious problem.			     *

*									     *

*	A bug in this configuration of the system may cause PSpice or	     *

*	ANY other program to destroy the data on your hard disk.	     *

*									     *

*	You can either:							     *

*		a) obtain patch 'INT10VGA.SYS' from your Compaq dealer, or   *

*		b) unplug your mouse, then reboot the system prior to	     *

*		using our software.					     *

*									     *

*	Should you fail to take corrective action and destroy your disk,     *

*	rest assured that you have our sympathy.			     *

******************************************************************************

INSTALLATION

1.1  PSpice Installation on IBM-PC

The following sections give instructions for installing the Evaluation

Version of PSpice and its options on the IBM-PC family of computers.

1.2  System Requirements

PSpice will run on any IBM-PC/XT/AT with 512-kilobytes of

memory, a fixed disk, and the MS-DOS 3.0 (or later) operating system.

Either the monochrome or the color graphics display may be used.

No special brand of printer or special printer features are needed.

PSpice will also run on IBM-compatible systems, such as the Compaq

386, as long as they meet the above requirements.  The program has been

run on some unusual pieces of hardware and the result is that, if the

system will run MS-DOS 3.0, has the memory, and has a fixed disk,

then it will run PSpice.

1.3  Doing the Installation

In the package which you received, you should find:

1)  The PSpice user's guide

2)  The book "SPICE: A Guide to Circuit Simulation Using PSpice"

3)  Diskettes labeled "Diskette 1" and "Diskette 2"

Installing PSpice consists mainly of making working copies of

the files on the two diskettes.

Before beginning, print and read the file README.DOC.  It

contains last-minute information which did not make it into

the user's guide.

1.4  Installing PSpice

The installation is simple: just copy all the files from both

diskettes into a directory on your fixed disk.  Also, if you

do not have a CONFIG.SYS file in your root directory, copy CONFIG.SYS

from Diskette 1 into your root directory.  If you do already have a

CONFIG.SYS file, examine the one on Diskette 1 and add any

statements which it contains to yours (BUFFERS=20, and FILES=12).

Now, reboot your system.  THE CHANGES TO CONFIG.SYS DO NOT TAKE

EFFECT UNTIL DOS IS REBOOTED.  They also do not take effect unless

CONFIG.SYS is in the root directory.  The copy of CONFIG.SYS in the

PSpice directory has no effect.

The diskettes are your backup copies and should be stored in a safe

place.

1.5  Comments

PSpice is composed of one program file, PSPICE1.EXE, and several

overlay, .OVL, files,.  You can put the PSPICE1.EXE and overlay files

on any diskette or directory you choose, so long as you follow these

guidelines:

1)  Do not rename PSPICE1.EXE.  The overlay handler will

not find overlays unless the program file is named PSPICE1.EXE.

2)  Make sure that the overlay files are in a drive and directory

listed in your system's PATH command.  Normally the PATH command

is included in the file AUTOEXEC.BAT in your system's root

directory or on your system (boot) floppy.

The CONFIG.SYS file configures the system when it is booted.

The BUFFERS=20 statement gives PSpice enough disk buffering to

work efficiently, and the FILES=16 statement allows PSpice to

open all the files that it needs.

This finishes the installation procedure for PSpice.  Now, please look

at chapter 3 of the user's guide and run PSpice on the EXAMPLE1 circuit

which was on Diskette 1.