PMISpice							Release  9.89

Copyright 1989,  Precision Monolithics, Inc.

The files on this diskette, as of the release date above,

are the most up to date SPICE macro-models for PMI's parts.

As new models become available new releases of this diskette

will be issued.

All the files on this diskette are ready for immediate use.

The following example shows how to quickly use the op-amp

models:

*********************************************************

OP-AMP MODEL AND TEST CIRCUIT

X1  0  2  3  4  5  OP-42

R1  1  2  1K

R2  2  3  1K

VIN 1  0  AC 1

VP  4  0  DC 15

VM  5  0  DC -15

.INC OP42.CIR

.AC DEC 20 100 100MEG

.PRINT AC VDB(3) VP(3)

.END

*********************************************************

This circuit is a standard inverting gain configuration with

a gain of -1 using an OP-42.  To test a different model just

change the model name from the OP-42 to the desired op-amp,

and also change the name of the ".inc" file to the name of

the desired op-amp.

Printed net-lists and schematics for these models are

available in either the data sheets for the specific part or

in an application note for the specific model.  The

application notes also include a brief explanation of the

model and comparisons between the actual and simulated

results.  Contact your local sales office for more

information.