*************************************************************************

*  POST ACQUISITION USING POST                        CONDENSED VERSION *

*            Edited from chapter 6 of the CODAS user's manual           *

*************************************************************************

GENERAL

The post acquisition software package (POST) is a utility that allows you to

examine and qualify data previously acquired to disk using the CODAS data

acquisition package.  Using POST, you will be able to perform the following

functions:

* Retrieve a data acquisition file and examine its contents by moving the

waveforms in either a positive or negative time direction (i.e. right-to-left,

or left-to-right). Waveforms may be smooth scrolled (panned)  across the

screen to achieve maximum waveform continuity, or paged in  a manner that

allows the entire screen to be updated one screen  width at a time.

* Enable a cursor allowing you to measure the value at any point on a

displayed waveform.

* Measure time differences between two points on the same or different

waveforms.

* Copy and paste groups of waveforms from the original data acquisition file

to  a separate data file for more detailed analysis.

* Select a specific file conversion format that will allow other software-

based analysis packages such as Lotus 1-2-3, ASYSTANT and ASYST to read the

data.

RUNNING THE POST WAVEFORM PLAYBACK PACKAGE

Contained on the CODAS system disk is a file named "POST.EXE". Running the

POST package is as simple as typing "POST" from the DOS level. An alternate

method of invoking POST is to type POST followed by a space then the name of

the data file (e.g. POST DUM.DAT).  Be certain that file "CODASPAR.BIN" is on

the same disk and path when you invoke POST. CODASPAR.BIN is a configuration

file that is accessed to allow POST to function in a manner that is compatible

with your computer system.

When accessed, POST will clear the monitor and prompt you to enter the file

name of the data file you wish to act on (if the file name was not specified

in the command line). Data file names may assume any type that is compatible

with PC- or MS-DOS including paths. When a valid file name in input, and the

ENTER key pressed, POST will clear the screen and present its default screen

(assuming the default has not been changed from the data acquisition package):

1.  Display format 1 enabled.

2.  Acquired logical channel 1 displayed.

3.  Waveform data annotation off.

4.  Waveform cursor disabled.

5.  Time to Beginning of File (TBF:) annotation off.

6.  Time to Marker (TM:) annotation off.

7.  Distance to End Of File (% EOF:) annotation off.

POST ACQUISITION COMMAND SUMMARY                                6-3

KEYSTROKE                 ACTION                          REFERENCE

---------     -----------------------------------         ---------

F1            Cursors on/off.                             6-11

F2            Display waveform data.                      6-44

ALT-F2        Toggle between waveform/cursor and

waveform/base line annotation.              6-14,44

F3            Display Time-To-Beginning-Of-File data.     6-39

F4            Set/remove Time Marker.                     6-38

ALT-F4        Move cursor back to original position       6-29

F5            Display Distance-To-End-Of-File data.       6-13

F6            Copy/Paste function.                        6-6

F6            Hard Copy Utility (HCU).                    6-22

ALT-F6        Advanced CODAS CALC Hot Key.                6-4

F7            Waveform Compression function.              6-45,47

F8            Search-For-Event function.                  6-34

F9            Select low calibration constant.            6-14,16

ALT-F9        Specify lower chart-edge value.             6-14,16

F10           Select high calibration function.           6-14,16

ALT-F10       Specify upper chart-edge value.             6-14,16

ALT-G         Grid pattern control                        6-33

H             Display HELP screen.                        6-27

Home/End      Jump to Beginning/End of file.              6-28

ALT-J         Shell to DOS                                6-36

# or #        Amplitude expansion and contraction.        6-5

<- or ->      Waveform movement control.                  6-49

+ or -        Waveform movement control; Page Mode        6-49

L or U        Waveform annotation control                 6-42

M             Selects Min, Max, or Mid.                   6-45,47

PgUp & PgDn            Fine waveform offset.              6-5

Ctrl-PgUp & PgDn       Coarse waveform offset.            6-5

Number/letter          Enable waveform window.            6-5

Number/letter=number   Reassign waveforms to windows.     6-40

ALT-N         Notepad function with annotation.           6-30

Ctrl-N        Notepad function without annotation.        6-30

Q or ESC      Exit POST and return to DOS.                6-32

ALT-Q         Switch waveform data files.                 6-37

SHIFT number  Select Display Format.                      6-12

Scroll Lock   Lock/release cursor movement                6-21

*             Center cursor                               6-21

CHANNEL SCALING AND OFFSET                                      6-5

FUNCTION

Used to dynamically scale or offset a specified channel's waveform on the POST

display.

KEYSTROKE TYPE

Multiple keystrokes may or may not be required, depending upon  the last

keyboard action taken.

ACTIVATING KEYSTROKES

A channel is enabled for scale and offset operations by pressing the key

carrying the number of the waveform window to be modified. When selected, the

"X=Y" annotation near the bottom of the screen will be highlighted indicating

that window X is enabled for scale and offset adjustments.

The up-arrow cursor control key is used to expand the specified window's

waveform in the amplitude direction.

The down-arrow cursor control key is used to contract the specified window's

waveform in the amplitude direction.

The "Pg Up" cursor control key is used to apply a positive*, incremental offset

to the specified window.

The "Pg Dn" cursor control key is used to apply a negative*, incremental offset

to the specified window.

* "Ctrl-PgUp" and "Ctrl-PgDn" can be used to apply a coarse offset to the

waveform within the selected window. Offset directions indicated assume that

a (S)ignal-oriented preference was selected through the CONFIG.EXE program.

Note that the directions will be completely opposite if a waveform movement

preference of (W)indow was selected from the CONFIG routine (i.e. "Pg Up"

cursor control key is used to apply a negative, incremental offset to the

specified channel, and "Pg Dn" cursor control key is used to apply a

positive, incremental offset to the specified channel). Refer to the RUNNING

THE PROGRAM CONFIG.EXE section of the GETTING STARTED chapter for details

related to the (S)ignal vs. (W)indow preference settings.

SPECIAL CONSIDERATIONS

Scaling and offsetting data does not, in any way, affect the information stored

on disk. These operations are only provided to configure the POST display.

KEYSTROKE EXAMPLES

(Assumes that eight waveform channels are available, and that the (S)ignal-

oriented movement preference is enabled)

KEYSTROKE ONE  KEYSTROKE TWO                      COMMENT

-------------  -------------   --------------------------------------------

1               #              Expand window one's waveform.

#              Contract window one's waveform.

Ctrl-PgUp      Apply a coarse positive offset to window one.

PgDn           Apply a fine negative offset to window one.

4               PgUp           Apply a positive offset to window four.

5               #              Contract window five's waveform.

8               Ctrl-PgDn      Apply a coarse negative offset to window eight.

COPY AND PASTE FUNCTION                                         6-6

FUNCTION

Allows portions of CODAS-generated data to be copied and pasted  to separate

data files in a variety of different storage formats. Created data files may

be read by user-generated, or standard (i.e. ASYST, and Lotus 1-2-3) software

analysis packages, or by Advanced CODAS' CALC package.

KEYSTROKE TYPE

Multiple keystrokes.

ACTIVATING KEYSTROKES

Before data may be copied and pasted to a disk file, the copy range must be

selected. The copy range is defined as all data for each enabled waveform

channel lying between the time marker and cursor. The copy range is selected

by positioning the cursor at one  desired copy range limit, enabling the time

marker (by pressing the "F4" function key), then moving the waveform cursor

over the other desired copy range limit. Although the range is specified only

on  display channel one, when copied, data for all channels lying between the

limits are copied to your specified file name.

When the copy ranges are selected as described  above, the Copy and Paste

utility is invoked by pressing the "F6" function key.  When activated, the

POST screen will be cleared and the following message will be printed:

Enter the name of data storage file: _

Respond by entering the name of the data file (including desired path names) to

which POST is to write the copied data. Pressing just the ENTER key without

specifying a file name will return you to POST. When a file name is entered,

the following additional information is printed:

1) ASCII (sequential)

2) Binary (with CODAS header)

3) Binary (without CODAS header)

4) ASYST/ASYSTANT

5) LOTUS

6) Hard Copy

7) DADiSP

8) CALC

Select storage format (1-8): _

Select the storage format you want, and enter its number. All  data within your

selected range will be copied to the specified data file.

CURSOR ON/OFF FUNCTION                                          6-11

FUNCTION

Used to turn on or turn off the waveform cursor.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKE

Pressing the "F1" function key will turn the cursor on if it is off, or turn it

off if it is on.

SPECIAL CONSIDERATIONS

When waveform cursor is enabled, the speed of waveform scrolling (panning)

decreases. The amount of this speed loss will vary depending upon the speed of

the machine you are using. To maintain maximum waveform scrolling speed (when

you need to travel a large distance through the data file) be sure the cursor

is disabled.

KEYSTROKE EXAMPLES

(Assumes cursor is initially disabled)

KEYSTROKE                  COMMENTS

---------              -------------------

F1                     Cursor is enabled.

F1                     Cursor is disabled.

Etc.                   Etc.

DISPLAY FORMAT SELECTION                                        6-12

FUNCTION

Used to select a specific waveform display format.

KEYSTROKE TYPE

Double, concurrent keystroke.

ACTIVATING KEYSTROKES

Pressing the shift key, then a number (0,1,2,3,...9) inclusive will enable that

display format design to be displayed on the computer's monitor.

SPECIAL CONSIDERATIONS

Do not use the numbers on the keyboard's number pad. These are reserved for

other POST functions. Always use the numbers on the top row of the main

keyboard. The following describes each display format in terms of its window

design, maximum number of displayed channels, and enabling keystrokes:

FORMAT      WINDOW                        MAXIMUM     ENABLING

NUMBER      DESIGN                        CHANNELS    KEYSTROKES

------     --------------------------     ----        ----------

One        1 full screen window.          1           SHIFT  1

Two        2 full screen windows;         2           SHIFT  2

dual overlapping waveforms.

Three      2 windows;                     2           SHIFT  3

non-overlapping waveforms.

Four       4 windows;                     4           SHIFT  4

dual overlapping waveforms.

Five       4 windows;                     4           SHIFT  5

non-overlapping waveforms.

Six        8 windows;                     8           SHIFT  6

dual overlapping waveforms.

Seven      8 windows;                     8           SHIFT  7

non-overlapping waveforms.

Eight      16 windows;                    16          SHIFT  8

dual overlapping waveforms.

Nine       16 windows;                    16          SHIFT  9

non-overlapping.

Ten        32 windows;                    29          SHIFT 0

overlapping.

See the VARIABLE WAVEFORM ASSIGNMENTS section of this chapter for information

pertaining to the assignment of any waveform channel to any window.

DISTANCE TO END-OF-FILE FUNCTION                        6-13

FUNCTION

When enabled, this function will display the distance from the current

cursor position to the end of the data file.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKE

The distance to end-of-file function is enabled by pressing the "F5" function

key. Pressing the "F5" function key a second time disables the distance to

end-of-file function.

SPECIAL CONSIDERATIONS

Cursors must be enabled before the distance to end-of-file function can be

displayed.

The distance to the end of the data file is displayed as a percentage at the

bottom of the screen where "% EOF:0%" indicates that the cursor position is at

the beginning of the data file, and "% EOF:100%" indicates that the current

cursor position is at the end of the data file.

When waveforms are scrolled beyond the beginning of the data file, %EOF will

display a value less than zero. When waveforms are scrolled beyond the end of

the data file, %EOF will display a value greater than 100.

KEYSTROKE EXAMPLES

(Assumes that cursors are enabled, and the EOF function was initially

disabled)

KEYSTROKE        COMMENTS

---------  ----------------------

F5         Enable EOF annotation.

F5         Disable EOF annotation.

F5         Enable EOF annotation.

Etc.       Etc.

ENGINEERING UNITS FUNCTION / BASE LINE-ORIENTED          6-14

FUNCTION

Allows waveform data to be scaled and annotated in significant engineering

units instead of volts. The base line-oriented function allows each waveform

channel to be calibrated relative to its window base line by positioning the

waveform so the base line horizontally traverses the waveform at a given

calibration level. Such an approach to waveform calibration is extremely

useful with noisy calibration signals where it is not possible to position the

cursor at a reliable single point on the waveform. Instead, base line

calibration permits you to strike a visual average of the base line relative

to the mid-point of the noise. In contrast, see the ENGINEERING UNITS FUNCTION

/ CURSOR-ORIENTED section of this chapter.

KEYSTROKE TYPE

Multiple.

ACTIVATING KEYSTROKES

1. Press the key carrying the number (or letter) of the display window

containing the waveform to be scaled. When selected, the "X=Y" annotation will

be highlighted, indicating that window X is enabled for engineering unit

adjustments.

2. Enable the cursor and waveform data, if they are not already enabled, by

pressing the 'F1' and 'ALT-F2' keys. Ensure that the cursor mode as described

at the bottom of the screen changes from "DATA" to "BASE".

3. Position the waveform such that the base line traverses your desired lower

calibration level. You may want to expand the waveform's amplitude in the

vertical direction (see the CHANNEL SCALING AND OFFSET section of this

chapter) to better view signal transitions for a more accurate base line

position.

4. Press the 'F9' key, and enter the desired engineering unit value for the

lower calibration signal immediately following the "L ?" prompt. After

entering the value, enter a space, then up to a four character optional

engineering unit tag. Press the ENTER key.

5. Position the waveform such that the base line traverses your desired higher

calibration level. Again, you may want to expand the waveform's amplitude in

the vertical direction to better view signal transitions for a more accurate

base line position.

6. Press the 'F10' key, and enter the desired engineering unit value for the

higher calibration signal immediately following the "H ?" prompt. Press the

ENTER key.

7. The waveform channel will now display all waveform data values in the chosen

engineering unit.

8. If you desire, the value at upper and lower chart-edge may be specified (and

the waveform automatically scaled to fit) by pressing the ALT-F9 sequence to

specify lower chart-edge, and ALT-F10 to specify upper chart-edge values. This

feature comes in handy by allowing the channel's display span to be whole

numbers (e.g. 0 - 200 mmHg instead of 10.372 - 189.628 mmHg). The Hard Copy

Utility (see the Hard COpy Utility section of this chapter for details)

accepts such upper and lower chart-edge specifications yielding a more

readable record.

SPECIAL CONSIDERATIONS

The value of engineering units may range from 1.0 E+32 to 1.0 E-32.

The engineering unit tag may be entered after specifying either the low or high

calibration values. The engineering unit tag alone for any waveform channel

may be changed by first selecting the channel's window number and pressing

either the F9 or F10 key. Then, enter up to a 4-character (the first character

must be an alpha) engineering unit tag following the "?" prompt and press

ENTER.

The following procedure applies to entering engineering unit tags after typing

the high or low calibration values and without yet pressing the ENTER key:

1. Press the space bar then the ENTER key - Erases the previously entered

engineering unit tag.

2. Pressing the ENTER key without a leading space - leaves the previously

entered engineering unit tag intact.

The rules governing the entry of scaling constants m and b (which form a

scaling equation of y = mx + b) are as follows:

1. The F10 key modifies the m scaling constant.

2. The F9 key modifies the b scaling constant.

3. The b scaling constant may be entered separately to allow waveform base line

adjustments.

4. The m scaling constant may be entered separately in cases where the slope

intercept of the scaling equation is assumed to be zero (e.g. 1mv/C). In all

cases where an m value is specified alone, the b value is forced to zero.

KEYSTROKE EXAMPLES

KEYSTROKE                             COMMENTS

---------          ------------------------------------------------

F1                 Enable cursor.

ALT-F2             Enable BASE LINE annotation. Note that the cursor mode

as described at the bottom of the screen changes from

"DATA" to "BASE".

1                  Select the window to receive engineering unit data.

PgUp or PgDn       Position base line so it traverses the desired lower

calibration level.

F9                 Prepare to enter low calibration value.

10                 Enter low calibration value.

Space bar          Prepare to enter engineering unit tag.

mmHg               Enter engineering unit tag.

PgUp or PgDn       Position base line so it traverses the desired higher

calibration level.

F10                Prepare to enter high calibration value.

100                Enter high calibration value.

F2                 Revert to cursor-oriented annotation.  Note that the

cursor mode as described at the bottom of the screen

changes from "BASE" to "DATA".

ALT-F9             Prepare to enter lower chart-edge value.

0                  Enter lower chart-edge value.

ALT-F10            Prepare to enter upper chart-edge value.

200                Enter upper chart-edge value.

ENGINEERING UNITS FUNCTION / CURSOR-ORIENTED            6-16

FUNCTION

Allows waveform data to be scaled and annotated in significant engineering

units instead of volts. The cursor-oriented function allows two individual

points per waveform channel to be chosen as calibration values. In contrast,

see the ENGINEERING UNITS FUNCTION / BASE LINE-ORIENTED section of this

chapter.

KEYSTROKE TYPE

Multiple keystrokes.

ACTIVATING KEYSTROKES

1. Press the key carrying the number (or letter) of the display window

containing the waveform to be scaled. When selected, the "X=Y" annotation will

be highlighted, indicating that window X is enabled for engineering unit

adjustments.

2. Enable the cursor and waveform data, if not already enabled, by pressing the

'F1' and 'F2' keys. Note that the cursor mode as described at the bottom of

the screen is "DATA".

3. Position the cursor over the lower calibration value of the waveform.

4. Press the 'F9' key, and enter the desired engineering unit value for the

lower calibration signal immediately following the "L ?" prompt. After

entering the value, enter a space, then up to a four character optional

engineering unit tag. Press the ENTER key.

5. Position the cursor over the higher calibration portion of the waveform.

6. Press the 'F10' key, and enter the desired engineering unit value for the

higher calibration signal immediately following the "H ?" prompt. Press the

ENTER key.

7. The waveform channel will now display all waveform data values in the chosen

engineering unit.

8. If you desire, the value at upper and lower chart-edge may be specified (and

the waveform automatically scaled to fit) by pressing the ALT-F9 sequence to

specify lower chart-edge, and ALT-F10 to specify upper chart-edge values. This

feature comes in handy by allowing the channel's display span to be whole

numbers (e.g. 0 - 200 mmHg instead of 10.372 - 189.628 mmHg). The Hard Copy

Utility (see the Hard COpy Utility section of this chapter for details)

accepts such upper and lower chart-edge specifications yielding a more

readable record.

SPECIAL CONSIDERATIONS

The value of engineering units may range from 1.0 E+32 to 1.0 E-32.

The engineering unit tag may be entered after specifying either the low or high

calibration values. The engineering unit tag alone for any waveform channel

may be changed by first selecting the channel's window number and pressing

either the F9 or F10 key. Then, enter up to a 4-character (the first character

must be alpha) engineering unit tag following the "?" prompt and press ENTER.

The following procedure applies to entering engineering unit tags after typing

the high or low calibration values and without yet pressing the ENTER key:

1. Press the space bar then the ENTER key - Erases the previously entered

engineering unit tag.

2. Pressing the ENTER key without a leading space - leaves the previously

entered engineering unit tag intact.

The rules governing the entry of scaling constants m and b (which form a

scaling equation of y = mx + b) are as follows:

1. The F10 key modifies the m scaling constant.

2. The F9 key modifies the b scaling constant.

3. The b scaling constant may be entered separately to allow waveform base line

adjustments.

4. The m scaling constant may be entered separately in cases where the slope

intercept of the scaling equation is assumed to be zero (e.g. 1mv/C).  In all

cases where an m value is specified alone, the b value is forced to zero.

KEYSTROKE EXAMPLES

KEYSTROKE                                 COMMENTS

---------          ----------------------------------------------------------

F1  F2             Enable the cursor and waveform data if not already enabled.

1                  Select the window to receive engineering unit data.

<- or ->           Position cursor over low calibration point of waveform.

F9                 Prepare to enter low calibration value.

10                 Enter low calibration value.

Space bar          Prepare to enter engineering unit tag.

mmHg               Enter engineering unit tag.

<- or ->           Position cursor over high calibration point of waveform.

F10                Prepare to enter high calibration value.

100                Enter high calibration value.

ALT-F9             Prepare to enter lower chart-edge value.

0                  Enter lower chart-edge value.

ALT-F10            Prepare to enter upper chart-edge value.

200                Enter upper chart-edge value.

EVENT MARKER EDITING                            6-18

FUNCTION

Allows a convenient method for moving from one event marker location to the

next while waveform data is displayed. Allows a positive, or negative-going

event marker to be placed on any enabled windows waveform except waveform

channel 1.

KEYSTROKE TYPE

Single.

ACTIVATING KEYSTROKE

When a waveform window has been enabled by pressing the letter or number key

with which it is associated, the following operations may be performed:

Keystroke                      Operation                        Restrictions

---------  ------------------------------------------------- -----------------

Ctrl ->    Moves cursor to an enabled event marker in a            None.

positive time direction.*

Ctrl <-    Moves cursor to an enabled event marker in a            None.

negative time direction.*

Ins        Inserts a positive-going event marker on the      Any channel number

waveform at the point of waveform intersection    except channel 1.

if the "Ins" key is pressed once. Inserts a

negative-going event marker on the waveform at

the point of waveform intersection if the "Ins"

key is pressed twice.

Del        Deletes an event marker that is coincident with   Any channel number

the waveform cursor.                              except channel 1.

* These directions assume that a (S)ignal-oriented preference was selected

through the CONFIG.EXE program. Note that the directions will be completely

opposite if a waveform movement preference of (W)indow was selected from the

CONFIG routine (i.e. '<-' cursor control key is used to enable left-to-right

waveform scrolling, and '->' cursor control key is used to enable right-to-

left waveform scrolling). Refer to the RUNNING THE PROGRAM CONFIG.EXE section

of the GETTING STARTED chapter for details related to the (S)ignal vs.

(W)indow preference settings.

SPECIAL CONSIDERATIONS

Before any event marker editing operations may be used, event markers must be

enabled for display (by pressing "F8", then "E", then "ESC"; see the SEARCH

FOR EVENT section of this chapter for details).

Any event markers created on waveforms through use of the "Ins" key will be

permanently stored to the data file to allow review at a later date.

When attempting to seek an event marker in either direction where one does not

exist, an audible BEEP will sound and the display will fail to move.

KEYSTROKE EXAMPLES

KEYSTROKE                            COMMENTS

---------  -----------------------------------------------------------------

F8         Enter the SEARCH FOR EVENT function.

E          Enable the display of event markers.

2          Enable waveform window 2.

Ins        Place a positive-going event marker at waveform cursor intersection.

Del        Delete inserted event marker.

Ins Ins    Place a negative-going event marker at waveform cursor intersection.

1          Enable waveform window 1

Ctrl ->    Seek next event marker in a positive time direction.

Ctrl <-    Seek next event marker in a negative time direction.

FREE / LOCK CURSOR FUNCTION                             6-21

FUNCTION

Allows the cursor to be released from its default position of screen-center.

When released, it may be horizontally positioned anywhere on the screen while

displayed waveforms remain stationary.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKES

Pressing the "SCROLL LOCK" key will free the cursor and allow you to

horizontally position it anywhere on the screen using the left and right

cursor control keys ("<-" and "->"). When the cursor is in its free state and

if your computer's keyboard provides an indicator light for SCROLL LOCK, the

light will be off. The light will be on when the cursor is in its locked

state. The cursor state will toggle each time the "SCROLL LOCK" key is

pressed.

SPECIAL CONSIDERATIONS

The asterisk key "*" may be used to center the cursor in the waveform display

window. If SCROLL LOCK is off, when the "*" key is pressed, the cursor alone

moves to screen-center while waveforms remain stationary. If SCROLL LOCK is

on, when the "*" key is pressed, cursor and waveforms scroll to screen-center

together.

KEYSTROKE EXAMPLES

(assumes the cursor is initially in its default, locked state)

KEYSTROKE           COMMENTS

-----------        -----------

SCROLL LOCK        Free cursor.

<- or ->           Slide cursor from left to right

(note that waveforms remain stationary).

SCROLL LOCK        Lock cursor.

*                  Cursor (now locked to waveforms) moves to screen-center.

SCROLL LOCK        Free cursor.

<- or ->           Move cursor from left to right (waveforms remain stationary).

*                  Cursor moves to screen center (waveforms remain stationary).

HARD COPY UTILITY (HCU)                                 6-22

FUNCTION

Provides a hard copy record of selected waveform data to any IBM-compatible

graphics dot-matrix printer. Hard copy may be executed from within POST, or

deferred to allow printing from DOS.

KEYSTROKE TYPE

Multiple keystrokes.

ACTIVATING KEYSTROKES

The range of waveform data to be hard copied is selected in a manner similar to

that of the COPY AND PASTE function. The hard copy range is defined as all

data for each enabled channel lying between the time marker and cursor. The

range is selected by positioning the cursor at one desired hard copy range

limit, enabling the time marker (by pressing the "F4" function key), then

moving the waveform cursor over the other desired range limit. Although the

range is specified only on waveform window one, when copied, data for all or a

subset of channels lying between the limits may be enabled for hard copy.

Note that each waveform channel should be scaled and offset on the soft display

exactly the way you would like it to appear when hard copied.

When the hard copy ranges are selected as described above, the Hard Copy

utility is invoked by pressing the "F6" function key. When activated, the POST

screen will be cleared and the following message will be printed:

Enter the name of data storage or hard copy file: _

Respond by entering the name of the hard copy file (including desired path

names) to which POST is to write the hard copy information. We recommend that

you use a ".HCU" extension in your file name to distinguish hard copy files

from other, more general purpose files. Pressing just the ENTER key without

specifying a file name will return you to POST. When a file name is entered,

the following additional information is printed:

1) ASCII

2) Binary

3) Binary (CODAS format)

4) ASYST/ASYSTANT

5) LOTUS

6) Hard Copy

7) DADiSP

8) CALC

Select option (1-8): _

Selecting item 6 from the option list will enable the hard copy function. The

screen will be cleared and the following will be displayed:

Enter one line of comments.

Following this prompt, you may enter up to one full line (70 characters) of

comments that will be passed to the hard copy record. After your comments are

entered, you will be prompted with:

Do you wish to print file now (Y/N)?

Responding with "N"o will cause the hard copy routine to create a print file on

disk using your supplied file name and return you to POST. Printing of the

file can then invoked at a later time from DOS by typing at the DOS prompt:

HCU FILE

Where "FILE" is the file name you supplied when selecting the hard copy option

from POST.

Responding with "Y"es to the above prompt, or invoking hard copy from DOS will

cause the screen to display:

Total number of channels is (total shown here)

Enter channel number(s) to be printed

Enter 'A' for all channels

Enter '-' channel number to delete channel

[1, ]

This menu displays the total number of channels that were enabled at the time

of data acquisition. It then allows you to select all, or a specified number

of channels to be hard copied.  You can select  the channels to be hard copied

as follows:

1. Enable a channel number for hard copy by pressing the channel number '#',

then a comma ',' or <CR>. Repeat this process for additional channels.

2. Pressing the letter 'A' will enable all channels to be hard copied.

3. Remove an enabled channel from the hard copy list by pressing a hyphen '-',

then the channel number to be removed. Upon pressing <CR>, that channel number

will be removed from the hard copy list. Likewise, pressing "-A" will remove

all channels from the hard copy list.

Pressing just a <CR> will enable all channel numbers currently in the hard copy

list to be enabled.  Following this action, the screen  will display:

Compression is [XX]

Where 'XX' is the waveform compression factor that was enabled at the time the

hard copy file was specified through POST. This compression factor may be

enabled by pressing a <CR>, or a new factor may be entered. After the waveform

compression factor is specified  the screen will display:

Press 'Esc' to abort printing

At this point, waveform hard copy will begin to your printer. You may halt the

printing process at any time by pressing the 'Esc' key. When hard copy is

aborted by pressing the "Esc" key, or upon normal termination of the printing

process, the following message will be displayed:

Do you wish to DELETE file: FILE NAME (Y/N)?

Where FILE NAME is the name of the print file you supplied when you first

invoked the HCU, answering "Y"es to this prompt will cause the HCU to store

the specified file to disk to allow printing at another time from DOS by using

the DOS hard copy utility called HCU.EXE. Answering "N"o will abort the saving

process meaning that to recover the hard copy record just produced will

require that the entire procedure just described be duplicated.

SPECIAL CONSIDERATIONS

Controlling Waveform Size and Scaling:  A hard copied waveform will appear on

the hard copy record exactly the way it appears on the POST display screen

including its maximum excursion, as well as grid scaling (refer to the

WAVEFORM CALIBRATION sections of this chapter).

Changing Printer Ports:  POST's hard copy utility supports peripheral

communication ports LPT1: (or PRN), COM1:, and COM2:. The default port is

LPT1: which is the parallel port normally associated with printers. Some

printers may require a serial communication link such as that provided by

COM1: and COM2:. To use the hard copy utility with such a serial printer, it

is necessary to redirect output from the default LPT1: (PRN) to either COM1:

or COM2:. This may be accomplished through use of the MS-DOS MODE utility.

Typing from the DOS prompt "MODE LPT1=COM1" or "MODE LPT1=COM2" will redirect

hard copy output to either COM1: or COM2: respectively.

Interpreting the Hard Copy Record:  Figure 1 is a sample single channel POST

hard copy output. The following describes the information provided by the

record:

File Information       Original CODAS-acquired data file name, and date and

time of acquisition; the print data file name, and

date and time created. Your comments are shown as

entered when the HCU was enabled.

Waveform Information   Describes the +/-full scale and base line values in

whatever engineering unit was selected through the

ENGINEERING UNITS  FUNCTION for each hard copied

waveform.  If an engineering unit function was not

enabled, the units displayed will be volts.  POST

allows waveforms to be offset above or below  the

base line (see WAVEFORM SCALING AND OFFSET).  Any

waveform offsets applied  while using POST will  be

similarly duplicated by the HCU in the hard copy

record.  Since any arbitrary offset will  distort

your ability to make waveform measurements directly

from the hard copy record, the base line offset for

each channel is included in the information header.

If the base line for a given channel is any value

other than zero, that value must be  algebraically

added to the waveform's apparent value as determined

by its transition relative to full scale.   An

example will help clarify this  concept.

Assume: Full  scale=100mmHg; Base  line=-10mmHg;

waveform  transition=1/4  full  scale   (25mmHg

apparent). The actual value of the waveform at  the

point of interest is 25+(-10)=15mmHg.

Also annotated is the Waveform Compression Factor

selected at the time the HCU was enabled.  This

factor is provided for reference only since all time

functions are automatically adjusted as a function

of the compression factor.

Time Annotation        The HCU produces two types of time annotation: Time

referenced to the beginning of the hard copy record

(TBR); and time referenced to the beginning of the

parent data acquisition file (TBF). The hard copy

record produced by the HCU yields a time division

marker approximately every 1cm. Each such marker is

annotated on the right and left chart-edge with TBR,

and TBF in seconds respectively. The need for TBR

is obvious.  This annotation allows any waveform

event to be placed in time relative to the beginning

of the hard copy record. The need for TBF is more

subtle, but just as important.

POST provides a TBF (see  Time-To-Beginning-Of-File

Function) function which allows the current position

of a waveform cursor to be known relative to the

time in seconds to the beginning of the file being

reviewed.  TBF annotation provided by the HCU

directly correlates with POST's TBF function. This

means that a waveform event occurring on the hard

copy record can be similarly located on the

computer-based record by scrolling the waveforms

forward or backward in time until TBF equals that

observed in the hard copy record.

Correlation of a hard copy  record with  its

computer-based parent is a valuable feature,

especially when coupled with Waveform Compression.

Large portions, or even an entire data file may be

compressed, hard  copied, then used as a tool for

determining where detailed analysis should begin and

end. In this manner, the feature allows you to get

a handle on large quantities of data in a purely

logical and organized fashion.

KEYSTROKE EXAMPLE

KEYSTROKE        COMMENT

F1               Enable cursors.

<- or ->         Position cursor at first hard copy range limit on

display channel one.

F4               Enable time marker to flag first hard copy range limit.

<- or ->         Position cursor at second hard copy range limit.

F6               Invoke Hard Copy/Copy and Paste menu.

HELP                                                    6-27

FUNCTION

Erases the POST screen and displays a help screen that briefly summarizes the

contents of this chapter.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKES

Pressing the 'H' key enables the Help screen. Pressing any key after the Help

screen is displayed returns to the POST screen that was enabled at the time

the 'H' key was pressed.

SPECIAL CONSIDERATIONS

None.

KEYSTROKE EXAMPLES

KEYSTROKE               COMMENT

---------          -----------------------

H                  Display the Help screen.

Any other key      Return to POST.

JUMP TO BEGINNING/END OF WAVEFORM FILE                  6-28

FUNCTION

Instantly moves the display screen to the beginning or the end of the waveform

data file.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKES

Pressing the HOME key located on your computer's numeric keypad will move the

display to the beginning of the waveform data file.

Pressing the END key of your computer's numeric keypad will move the display to

the end of the waveform data file.

SPECIAL CONSIDERATIONS

None.

KEYSTROKE EXAMPLES

KEYSTROKE                           COMMENTS

---------          ---------------------------------------------------

HOME               Move display to the beginning of the waveform data file.

END                Move display to the end of the waveform data file.

JUMP TO TIME MARKER / JUMP TO ORIGIN                    6-29

FUNCTION

Instantly moves the display screen to the position of the file occupied by the

time marker (see the TIME MARKER FUNCTION section of this chapter). Instantly

moves back to the position of the file from which the jump to the time marker

occurred.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKES

Pressing the ALT-F4 key sequence moves the display so the cursor overlays the

previously enabled time marker. Pressing the ALT-F4 key sequence a second time

moves the display back to the original position.

SPECIAL CONSIDERATIONS

If a time marker is not enabled at the time the ALT-F4 key sequence is

activated, no display repositioning will occur.

KEYSTROKE EXAMPLES

KEYSTROKE                    COMMENTS

---------          ------------------------------

ALT-F4             Move to time marker.

ALT-F4             Move back to original position.

ALT-F4             Move to time marker.

Etc.               Etc.

NOTEPAD FUNCTION                                        6-30

FUNCTION

Allows POST data to be written to a notepad data file on disk.

KEYSTROKE TYPE

Multiple keystrokes.

ACTIVATING KEYSTROKES

The NOTEPAD function may be invoked at any time during data review by pressing

the CNTRL-N key combination. The first time during the session that CNTRL-N is

pressed, a prompt will appear asking for the name of the data file to which

screen data is to be written. Any valid DOS file name may be entered.

Thereafter, pressing CNTRL-N will send selected data to the named target file.

Using the ALT-N keystroke combination produces almost the same result. When

ALT-N is used, the data is sent to disk along with text information. The text

is entered following the prompt, and the number of text characters that may be

stored for each ALT-N keystroke conforms to the following equation: (11 x # of

channels enabled) + 22.

SPECIAL CONSIDERATIONS

Data is written to the target data file in a column format. A maximum of thirty

one columns may be written depending upon the number of waveforms in the data

file. When POST is exited, and a notepad file is typed to the screen or

printed, the columns seen are defined as follows:

COLUMN 1 (left-most)     Time-to-beginning-of-file

COLUMN 2                 Time marker

COLUMN 3                 Channel one data

COLUMN 4                 Channel two data (if available)

COLUMN 5                 Channel three data (if available )

.                        .

.                        .

.                        .

COLUMN 31 (right-most)   Channel twenty nine data (if available)

Data for all waveform channels, time-to-beginning-of-file, and time marker data

are all written to the notepad file whenever the CNTRL-N or ALT-N keys are

pressed.

All data will be written in scientific notation format. The only exception to

this rule is time marker data. If a time marker was enabled at the time the

NOTEPAD function was invoked, time marker data will be annotated in scientific

notation. If the time marker WAS NOT enabled, column two will contain nine

zeros "000000000" indicating that the time marker was not enabled.

Before data may be sent to the notepad file, the cursor (F1) and WAVEFORM DATA

(F2) functions must be enabled.

If the ENGINEERING UNITS function was enabled for a channel (or channels), the

data written to the notepad file will be in engineering units. Channels that

have not been enabled for engineering units conversion will be written as raw

analog voltages.

When the NOTEPAD is activated with a compressed waveform display, waveform

values reported to the notepad file will be the minimum, maximum, or mid point

value of the waveform's line segment that is coincident with the cursor

depending on the state of the cursor mode. Refer to the WAVEFORM COMPRESSION

sections of this chapter for details related to the function and operation of

the cursor mode with compressed waveforms.

All NOTEPAD files are directly compatible with LOTUS 1-2-3, ASYST, and

ASYSTANT. In the case of the latter two, the notepad file will have to be

converted from ASCII into the ASYST/ASYSTANT format before being read. ASYST

and ASYSTANT provide the file conversion utilities.

KEYSTROKE EXAMPLES

KEYSTROKE                           COMMENTS

---------                -----------------------------

<- or ->                 Position cursor over waveforms.

F2                       Enable WAVEFORM DATA if disabled.

CNTRL-N                  Enable NOTEPAD function.

NOTE.PAD                 Name NOTEPAD file.

ENTER                    Create NOTEPAD file and write data to it.

<- or ->                 Reposition cursors.

ALT-N                    Specify text input.

MAXIMUM PRESSURES        Enter descriptive text.

ENTER                    Send text and data to NOTEPAD file.

<- or ->                 Reposition cursors.

CNTRL-N                  Send data to NOTEPAD file.

QUIT                                                    6-32

FUNCTION

When issued, the Quit command exits the Post Acquisition software utility and

returns you to DOS.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKE

Pressing the "Q" key will exit the Post Acquisition software package. Upon

"Q"uitting, you will be prompted for the following:

Do you want to:

(C)alibration save before exit*

(E)xit without saving anything

(R)eturn to waveform display

(S)ave state and calibration before exit

Select:

* This prompt appears ONLY if calibration was adjusted during your review of

waveform data.

Selecting "C" from the above menu saves calibration only. The next time the

data file is reviewed, all waveforms will appear calibrated without needing to

repeat the calibration process.

Selecting "E" from the menu will force POST to exit to DOS without saving

calibration or its state. If calibration for the waveforms was previously

provided and saved, the constants provided are  unaffected by this command,

remaining intact for the next time the file  is reviewed.

Selecting "R" returns you to the POST display without saving anything.

Selecting "S" saves the entire state of POST (display format, scales, offsets,

compressions, etc., etc.) including any provided calibration constants before

returning you to DOS. The next time the data file is reviewed, it will default

to the exact display state that existed when exited and saved the time before.

SPECIAL CONSIDERATIONS

None.

KEYSTROKE EXAMPLES

KEYSTROKE             COMMENTS

---------        ---------------------

Q                Exit Post Acquisition.

C/E/R/S          As required.

SCREEN GRID PATTERN CONTROL FUNCTION                    6-33

FUNCTION

Used to enable or disable the waveform display grid pattern.

KEYSTROKE TYPE AND OPERATING MODE

Single keystroke. All operating modes.

ACTIVATING KEYSTROKES

Pressing the "ALT-G" key sequence enables the grid pattern if it was disabled,

or disables the grid pattern if it was enabled.

SPECIAL CONSIDERATIONS

None.

KEYSTROKE EXAMPLES

KEYSTROKE             COMMENTS

---------        -------------------

ALT-G            Enable grid pattern.

ALT-G            Disable grid pattern.

ALT-G            Enable grid pattern.

Etc.             Etc.

SHELL TO DOS FUNCTION                                   6-36

FUNCTION

Allows you to temporarily suspend operation of POST, and vector to the DOS

level.

KEYSTROKE TYPE

Single.

ACTIVATING KEYSTROKE

Pressing the "ALT" and "J" keys simultaneously will activate the DOS shell

function. Any valid DOS operation may be performed while in DOS. Typing "EXIT"

from the DOS command line will return you to POST at the point it was

suspended.

SPECIAL CONSIDERATIONS

None.

KEYSTROKE EXAMPLES

KEYSTROKE                 COMMENTS

---------     ----------------------------------------

ALT-J         Suspend POST and vector to the DOS level.

-           Perform DOS operation(s).

EXIT          Return to POST at the point exited.

TIME MARKER FUNCT                                       6-38

FUNCTION

Annotates the time difference from the cursor to the time marker in seconds.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKES

Pressing the "F4" function key enables the time marker function. Pressing the

"F4" key a second time disables the time marker function.

SPECIAL CONSIDERATIONS

The cursor function must be enabled before time-to-marker can be displayed. To

use the time marker function, position the cursor (by using the left- and

right-arrow keys) at a point on the waveform in window 1 where the time marker

is to be placed. Press the "F4" function key to set and display the time

marker. Move the cursor away from the time marker (again, by using the left-

and right-arrow keys) and the time difference between the cursor and time

marker will be annotated at the bottom of the screen as "TM(F4):XXXXX", where

"XXXXX" is the time difference in seconds. Pressing the "F4" key a second time

will remove the time marker.

Since cursors for all channels align perfectly in time, the time difference for

events on channels other than chat in window one may be measured even though

the time marker itself is displayed only on window one's waveform. Using the

VARIABLE WAVEFORM ASSIGNMENTS function you can assign any waveform channel

number to window one to allow time measurements to be made on any channel.

Time differences are annotated as both positive and negative numbers. When the

cursor is to the right of the time marker, the time difference will be

displayed as a positive value. When the cursor is to the left of the time

marker, the time difference will be displayed as a negative value.

When a time marker is placed directly over an event marker, the time marker has

priority and the event marker will disappear.

KEYSTROKE EXAMPLES

KEYSTROKE          COMMENTS

---------        --------------

F1               Enable cursors.

<- or ->         Position cursor over desired waveform event.

F4               Set and display time marker.

<- or ->         Move cursor to second event to measure time difference.

TIME-TO-BEGINNING-OF-FILE FUNCTION                      6-39

FUNCTION

When enabled, the Time-To-Beginning-Of-File (TBF) function annotates the

quantity of time from the beginning of the data file to the point of

waveform/cursor intersection in seconds.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKE

Pressing the "F3" function key enables TBF annotation if it is disabled.

Pressing "F3" a second time will disable TBF annotation.

SPECIAL CONSIDERATIONS

Cursors must be enabled before TBF annotation can be displayed. TBF is

annotated at the bottom of the screen as "TBF(F3):XXXXX" where "XXXXX" is the

time-to-beginning-of-file from the point of waveform/cursor intersection in

seconds.

KEYSTROKE EXAMPLES

(Assumes TBF annotation is initially disabled, and cursors are enabled)

KEYSTROKE              COMMENTS

---------        ----------------------

F3               Enables TBF annotation.

F3               Disables TBF annotation.

F3               Enables TBF annotation.

Etc.             Etc.

VARIABLE WAVEFORM ASSIGNMENT                            6-40

FUNCTION

Allows any valid waveform channel (from 1 - 29) to be assigned to a

specified display window.

KEYSTROKE TYPE

Multiple (at least three and possible four keystrokes).

ACTIVATING KEYSTROKES

The POST waveform playback utility supports up to 32 waveforms display windows

(1-9,A,B,C...,W) in which any one of 29 waveforms may be displayed. In some

cases, any two waveforms may be displayed in  an overlapping format within two

windows (refer to the DISPLAY FORMAT SELECTION section of this chapter).

Waveform windows carry  a designator number or letter as follows:

Waveform Window  Display Format      Waveform Window  Display Format

-------- ------  --------------      -------- ------  --------------

1      1        1 thru 10            17     H         10

2      2        2 thru 10            18     I         10

3      3        4 thru 10            19     J         10

4      4        4 thru 10            20     K         10

5      5        6 thru 10            21     L         10

6      6        6 thru 10            22     M         10

7      7        6 thru 10            23     N         10

8      8        6 thru 10            24     O         10

9      9        7,9, and 10          25     P         10

10     A        7,9, and 10          26     Q         10

11     B        7,9, and 10          27     R         10

12     C        7,9, and 10          28     S         10

13     D        7,9, and 10          29     T         10

14     E        7,9, and 10          30     U         10

15     F        7,9, and 10          31     V         10

16     G        7,9, and 10          32     W         10

As the above table shows, the windows enabled for display are a function of the

selected display format. By typing the following sequences, you can assign any

waveform channel number to the specified display window:

Keystroke Keystroke Keystroke Keystroke            Comment

One       Two       Three     Four

(window)            (channel)

--------- --------- --------- ---------    --------------------------------

1         =         2         not needed   Displays waveform channel two in

waveform display window one.

1         =         1            6         Displays waveform channel sixteen in

waveform display window one.

1         =         1          ENTER       Displays waveform channel one in

waveform display window one.

2         =         1            2         Displays waveform channel twelve in

waveform display window two.

A         =         3        not needed    Displays waveform channel three in

waveform display window ten.

4         =         1            6         Displays waveform channel sixteen in

waveform display window four.

4         =         1          ENTER       Displays waveform channel one in

waveform display window four.

The above works for any enabled display format and allows you to examine the

characteristics of any waveform channel within any display window.

SPECIAL CONSIDERATIONS

When performing any operation on a specific channel (i.e. up/down scale,

offset, etc.), that channel is enabled by pressing the letter or number of the

display window containing the waveform. When so specified, the "X=Y"

annotation at the bottom of the screen will be highlighted indicating that

waveform channel Y contained in waveform window X is enabled for operation.

The default window/channel assignment is as follows (remember that the number

or letter before "=" is the waveform display window; the number following is

the waveform channel number):

1=1       9=9       H=17      P=25

2=2       A=10      I=18      Q=26

3=3       B=11      J=19      R=27

4=4       C=12      K=20      S=28

5=5       D=13      L=21      T=29

6=6       E=14      M=22      U=unassigned

7=7       F=15      N=23      V=unassigned

8=8       G=16      O=24      W=unassigned

WAVEFORM DATA FUNCTION                                  6-44

FUNCTION

Allows annotation of waveform values at the point of waveform/cursor, or

waveform/base line intersection. The values are annotated near the bottom of

the screen immediately following the VARIABLE WAVEFORM ASSIGNMENT designator.

KEYSTROKE TYPE

Single keystroke.

ACTIVATING KEYSTROKE

Pressing the "F2" function key will turn waveform value annotation on if it is

off, or turn it off if it is on. Pressing and holding the ALT key while

pressing the F2 key will toggle between waveform/cursor intersection point

values, and waveform/base line intersection point values. When base line-

oriented waveform data is enabled, the cursor mode designator at the bottom of

the screen will indicate �BASE�. When cursor-oriented waveform data is

enabled, the cursor mode designator at the bottom of the screen will indicate

"DATA".

SPECIAL CONSIDERATIONS

The area near the bottom of the screen reserved for waveform data is large

enough to report eight or less waveforms. Should a file consisting of more

than eight channels be played back, Display Format 10 may be used to view up

to 29 channels along with a window designator for each. To examine the value

of a waveform that is not currently displayed in the waveform data area, press

the key corresponding to its window (1 thru 9, A,B,C,...,W). Note that the

default state of the waveform data area is the display of waveform windows 1

thru 8. Pressing the "G" key forces the display of windows A thru G.

Sequential pressing of the 9,A,B,C,...,W keys from the default state will

replace the display of waveform window values 1,2,3,4,5,6,7 and 8 as each is

pressed. When the waveform data area contains values for windows P thru W,

pressing the "1" key forces the display of windows 1 thru 8.

The annotated waveform value will be either the raw waveform voltage, or the

waveform voltage converted into engineering units if this feature has been

enabled. See the ENGINEERING UNITS sections of this chapter.

KEYSTROKE EXAMPLES

(Assumes waveform value annotation is initially off)

KEYSTROKE                          COMMENTS

---------   ----------------------------------------------------------

F2          Turn waveform/cursor intersection value annotation  on

(note cursor mode is "DATA:").

ALT-F2      Turn waveform/base line intersection value annotation off

(note cursor mode is "BASE:").

ALT-F2      Turn waveform/cursor intersection value annotation on

(note cursor mode is "DATA:").

F2          Turn waveform value annotation off.

WAVEFORM COMPRESSION - FILE                             6-45

FUNCTION

To compress the entire waveform data file on to one screen width.

KEYSTROKE TYPE

Multiple.

ACTIVATING KEYSTROKES

From the POST level, pressing the 'F7' key will enable the waveform compression

menu:

________________________________________________________________________________

Compression ratio   Display window time

XX           t seconds/minutes/hours

________________________________________________________________________________

For the above sample screen, value 'XX' is the selected compression ratio and

may range from 1 (time compression disabled) to whatever value is required to

compress the entire file on the screen. The value 't' is the amount of elapsed

waveform time displayed by one channel window's width for each displayed

channel. This time value may range from several seconds to many hours

depending upon the sample rate selected, and the number of channels enabled

during acquisition. Time units (seconds, minutes, hours) will be selected

automatically.

To achieve waveform data file compression so that the entire file in compressed

into one screen width, press and hold the '#' or 'PgUp' key #until the waveform

compression factor displayed on the screen no longer increments. Then press

the ENTER key. The following features have now been activated:

1. The entire waveform data file has been compressed to fit within one screen

width.

2. A cursor is displayed in the center of the screen.

3. The '<-' and '->' cursor control keys are active allowing the cursor to be

moved in a left and right direction across the waveform(s).

4. The cursor position at any given location on the screen will define the

center of the screen when the screen is expanded by reentering the WAVEFORM

COMPRESSION FUNCTION and selecting a lower compression factor.

5. The following POST functions are available while within the compressed file

format: CALC HOT KEY, CHANNEL SCALING AND OFFSET, COPY/PASTE (at the current

compression factor only), CURSOR ON/OFF, DISPLAY FORMAT SELECTION, DISTANCE

TO END-OF-FILE, HARD COPY UTILITY (at any compression factor), HELP, JUMP TO

BEGINNING/END OF FILE, QUIT, SEARCH-FOR-EVENT, SWITCH FILES, TIME MARKER,

TIME-TO-BEGINNING-OF-FILE, VARIABLE WAVEFORM ASSIGNMENTS.

6. The fully compressed file display is saved in memory to allow fast reentry

when moving between lower (or uncompressed), and the full file compressed

formats.

SPECIAL CONSIDERATIONS

The compression ratio is the number of data points for each enabled channel

that are evaluated for a min/max value. If the compression ratio equals

twenty, then a min/max line is plotted for every twenty evaluated data points.

Cursor Mode Control (Mid/Min/Max) for compressed displays - The waveform

compression function operates by accumulating n number of waveform samples

(where n equals the compression factor) then displaying a line whose end

points equal the absolute minimum and maximum values detected for the sample

group. When the cursor and WAVEFORM DATA functions are enabled, a dilemma

exists regarding the appropriate value to report since multiple waveform

segments have been compressed into one. To a degree, the cursor mode control

solves this problem by allowing you to specify the value of a compressed line

segment at cursor intersection as either its minimum, maximum, or mid point

quantity. Whenever waveform compression is enabled, consecutive striking of

the "M" key will toggle between MID, MIN and MAX line segment reporting. The

currently active cursor mode is automatically reported at the bottom of the

screen where the normal "DATA" annotation is replaced by "MIN", "MAX", or

"MID". The default state is "MID".

KEYSTROKE EXAMPLES

Keystroke                    Comments

---------    ----------------------------------------------

F7           Select the waveform compression function.

# or PgUp    Increment compression factor until the maximum count is reached.

ENTER        Enables the compressed file display.

M            Toggles from "MID" line segment value annotation to "MIN".

M            Toggles from "MIN" line segment value annotation to "MAX".

WAVEFORM COMPRESSION - SIGNAL                           6-47

FUNCTION

Allows displayed waveforms to be time-compressed by a given ratio.

KEYSTROKE TYPE

Multiple keystrokes.

ACTIVATING KEYSTROKES

From the POST level, pressing the 'F7' key will enable the waveform compression

menu:

________________________________________________________________________________

Compression ratio   Display window time

XX           t seconds/minutes/hours

________________________________________________________________________________

For the above sample screen, value 'XX' is the selected compression ratio and

may range from 1 (time compression disabled) to whatever value is required to

compress the entire file on the screen (refer to the WAVEFORM FILE COMPRESSION

FUNCTION section of this chapter). The value 't' is the amount of elapsed

waveform time displayed by one channel window's width for each displayed

channel. This time value may range from several seconds to many hours

depending upon the sample rate selected, and the number of channels enabled

during acquisition. Time units (seconds, minutes, hours) will be selected

automatically.

To increase the compression ratio, press the up-arrow cursor control key to

increment the compression ratio by one. Press the 'PgUp' cursor control key to

increment the compression ratio by a factor of 2. To decrement the compression

ratio by a factor of one, press the down-arrow cursor control key. To

decrement the compression ratio by a factor of two, press the 'PgDn' cursor

control key. Press the ENTER key to select the displayed compression ratio and

display window time value. As either arrow key is pressed, the display window

time will increment or decrement as a function of the selected compression

ratio.

SPECIAL CONSIDERATIONS

The compression ratio is the number of data points for each enabled channel

that are evaluated for a peak value. If the compression ratio equals twenty,

then one peak value is plotted for every twenty evaluated data points.

You may not be able to set display window time to a precise value, since the

degree of waveform compression is a function of the original sample rate and

the number of channels enabled. Both these numbers vary in a manner that

allows a only a finite set of display window time values.

The direction of waveform compression is from either side of the display window

centered on the cursor.

Cursor Mode Control (Mid/Min/Max) for compressed displays - The waveform

compression function operates by accumulating n number of waveform samples

(where n equals the compression factor) then displaying a line whose end

points equal the absolute minimum and maximum values detected for the sample

group. When the cursor and WAVEFORM DATA functions are enabled, a dilemma

exists regarding the appropriate value to report since multiple waveform

segments have been compressed into one. To a degree, the cursor mode control

solves this problem by allowing you to specify the value of a compressed line

segment at cursor intersection as either its minimum, maximum, or mid point

quantity. Whenever waveform compression is enabled, consecutive striking of

the "M" key will toggle between MID, MIN and MAX line segment reporting. The

currently active cursor mode is automatically reported at the bottom of the

screen where the normal "DATA" annotation is replaced by "MIN", "MAX", or

"MID". The default state is "MID". The currently active cursor mode during a

compressed display session remains active for the COPY/PASTE FUNCTION.

Waveform segments are pasted as MID, MIN, or MAX values. Refer to the

COPY/PASTE FUNCTION section of this chapter.

KEYSTROKE EXAMPLES

KEYSTROKE               COMMENTS

---------    --------------------------------

F7           Enable waveform compression menu.

#            Increment compression ratio by one.

#            Decrement compression ratio by one.

PgDn         Decrement compression ratio by a factor of two.

PgUp         Increment compression ratio by a factor of 2

ENTER        Select compression ratio and return to POST.

M            Toggles from "MID" line segment value annotation to "MIN"

M            Toggles from "MIN" line segment value annotation to "MAX"

WAVEFORM MOVEMENT CONTROL                               6-49

FUNCTION

Allows the smooth scrolling (panning) or paging of waveforms in either a right-

to-left (positive time) or left-to-right (negative time) direction.

KEYSTROKE TYPES

Single keystroke; single press-and-hold keystroke.

ACTIVATING KEYSTROKES

To enable right-to-left* waveform scrolling, press the '<-' cursor control

key. To enable left-to-right* waveform scrolling, press the '->' cursor

control key. Pressing and holding either key will incrementally accelerate

waveform scrolling in the appropriate direction.

To enable right-to-left waveform paging, press the '+' key. To enable left-to-

right paging press the '-' key.

* These directions assume that a (S)ignal-oriented preference was selected

through the CONFIG.EXE program. Note that the directions will be completely

opposite if a waveform movement preference of (W)indow was selected from the

CONFIG routine (i.e. '<-' cursor control key is used to enable left-to-right

waveform scrolling, and '->' cursor control key is used to enable right-to-left

waveform scrolling). Refer to the RUNNING THE PROGRAM CONFIG.EXE section of

the GETTING STARTED chapter for details related to the (S)ignal vs. (W)indow

preference settings.

SPECIAL CONSIDERATIONS

The keyboard's number lock (Num Lock) function must not be enabled when

waveform scrolling is activated.

The following items will adversely affect the maximum speed of waveform travel

in either direction:

1. Using a floppy disk-based data file as opposed to hard disk-based.

2. Use of an IBM PC/XT as opposed to an IBM PC/AT or equivalent.

3. A waveform compression ratio greater than 1.

To achieve maximum waveform scroll speed for your particular computer if your

data file of interest resides on floppy disk, and you have a hard disk

available, copy the file to hard disk. If you do not have a hard disk, copy

the data file to virtual disk and specify the virtual disk's "drive" letter

when invoking POST. Refer to the VDISK command of your computer's DOS manual

(version 3.0 or greater).

When a scroll direction is specified by pressing an arrow key, waveform travel

continues in the specified direction until the opposite arrow key is pressed.

When a paging direction is specified by pressing either the "+" or "-" key,

waveform travel will halt automatically when the screen contents are replaced

by a new page.

KEYSTROKE EXAMPLES

(<- <- and -> -> indicates the specified arrow key is pressed and held down.

Assumes [S]ignal-oriented preference as specified through the CONFIG program)

KEYSTROKE                              COMMENTS

---------          -------------------------------------------------

<- <-              Right-to-left waveform travel is enabled with gradually

increasing speed.

->                 Waveform travel in the right-to-left direction is halted.

-> ->              Left-to-right waveform travel is enabled with gradually

increasing speed.

<-                 Waveform travel in the left-to-right direction is halted.

+                  Page screen in a positive time direction.

-                  Page screen in a negative time direction.