Printerdriverdisc LC24-10

You will find drivers for LC24-10 for some Applications in MS-DOS Format.

For the following product are drivers available:

Software:

1st Word Plus

ConText 4

DBase IV

Euroscript 3.0

Q&A 3.0

Framework III

Framework IV

Microsoft Chart 3.0

Microsoft Multiplan 4.01, 4.2

Microsoft WINDOWS 3.0

Microsoft WORD 4.0

Microsoft WORD 5.0, 5.5

Microsoft WORKS 1.05

Microsoft WORKS 2.0

Open Access III

PCTEXT 4 V1.5, V2.5

StarWriter 5.0, 5.5

SmartWare II Ver. 1.02

SmartWare II Ver. 1.51

TexAss Window Plus

Witchpen mal 5

WordPerfect 5.1

Wordcraft

ALso on disk a program to convert 9-wire into 24-wire graphics data and a

Fonteditor to create Download Fonts.

The following DIP-Setting is recommended for all Applications:

Block 1            Block 2

1 2 3 4 5 6 7 8    1 2 3 4 5 6 7 8

ON       x x *   x   x    x x x x

OFF	x     *     x

**************************************************************************

WORD 5.0, 5.5

You can select the printer by using the menue Print Options.

You should select the correct model in the menue Printer Name and the correct Paper Feed.

LC24-10 with fanfold paper:

Printer Name: LC24-10				Paper Feed: Continous

LC24-10 with Single Sheet

Printer Name: LC24-10 (Manual Feed)		Paper Feed: Manual

LC24-10 with Automatic Sheet Feeder

Printer Name: LC24-10 (SF-10DK)			Paper Feed: Bin 1

**************************************************************************

WORKS 2.0

The Installation is as follows :

- Print

- Setup Printer

You should select the correct model in the menue MODEL and the correct

Page Feed.

LC24-10 with fanfold paper:

Model: LC24-10					Page Feed: Continous

LC24-10 with Single Sheet

Model: LC24-10 (Manual Feed)			Page Feed: Manual

LC24-10 with Automatic Sheet Feeder

Model: LC24-10 (SF-10DK)			Page Feed: Continous

**************************************************************************

SMARTWARE II Version 1.02

The Installation is as follows.

- Tools

- Configuration

- Hardware

**************************************************************************

WINDOWS 3.0

To Install Drivers

__________________

1> Copy all files to your WINDOWS/SYSTEM directory on your hard disk

2> CD to that directory

3> Run DRVSETUP

-Supply your windows path when asked.

4> From Windows Control Panel, Printers Select Add Model

5> All models should be listed

6> Supply the path of the directory you copied to, when asked

7> Configure the drivers as normal.

**************************************************************************

WordPerfect 5.1

Copy the file WPLC24.ALL to your WP51 directory.

The Installation is as follows :

[Shift] F7 - Print

S - Select Printer

2 - Additional Printer

Move the cursor to the entry STAR LC24-10.

1 - Select

After return to the menue Printer Selection press F7 twice. Then

1 - Select

*************************************************************************