P-CAD ILLUSTRATED USER'S GUIDE README FILE

This file contains corrections to the tutorial section of the P-

CAD Illustrated User's Guide.  The tutorial section includes

chapters 10 through 17.

Print out a copy of this file and include it in the tutorial

section of your book (before working on an exercise, check to see

what corrections there are, if any at all), or use the printed

file to mark the corrections directly in your book.

You can print this document with your word processor, or from

DOS.  To print it from DOS:

> At the A:\ prompt, type PRINT README.DOC [Return].

If your computer responds with:

Name of list device [PRN]:

You can press [Return] to accept the default output list device

PRN, or you can specify another output list device.  For more

information refer to the PRINT command in your DOS manual.

EXERCISE 10-2: Cursors.

-----------------------

Page 10-53.  In step 4 when you enter a wire, select an area on

the screen that is free of design (such as wires, nets,

components, etc.).  Otherwise if you cross a net, the program

will prompt whether you want the nets merged.

EXERCISE 10-5: Checking System Information.

-------------------------------------------

Pages 10-61 and 10-62.  In figures 10-35 and 10-36 the

information presented is dependent on a particular configuration

and may not necessarily match your statistical data.

EXERCISE 11-4: Creating a Symbol for A Nonhomogeneous Part.

---------------------------------------------------------

Work in symbol mode (SYMB) when you do this exercise.

On page 11-31, after you edit the pin number locations in step 6,

you should add a device name.  Use the following instructions:

Add the device name.

> Select DRAW/TEXT.

Make DEVICE the active layer from the status line.

Change the text size to 25.

Position the cursor under the symbol.

Press Button 1.

Type 75188 and press [Return].

On page 11-35, after you edit the pin number locations in step

12, you will not need to add a device name, as you would have

already done so.  Besides it should be the same on both gate

types.

EXERCISE 11-5: Building a Nonelectrical Symbol - A Ground.

----------------------------------------------------------

Work in symbol mode (SYMB) when you do this exercise.

EXERCISE 11-6: Creating A Custom Solder Dot.

--------------------------------------------

Work in symbol mode (SYMB) when you do this exercise.

EXERCISE 11-7: Creating A Special Symbol File.

----------------------------------------------

Work in symbol mode (SYMB) when you do this exercise.

Page 11-44.  When you view other instances of the solder dots in

step 7, after selecting ZOUT you must:

> Position the cursor at coordinates 300 1110.

Press Button 1 twice.

EXERCISE 12-1: Creating a Multisheet Schematic.

-----------------------------------------------

Page 12-15.  After you load the D-size drawing it will be easier

to work with a screen that is zoomed in a little more.  Zoom in

following these instructions:

> Select ZIN.

Position the cursor in the middle of the D-size drawing.

Press Button 1 twice.

Page 12-17.  In step 5, the instructions to enter and place

another set of components should read as follows:

> Select ENTR/COMP

Type 74ls93 and press [Return] for the component filename.

Select YES.

Position the cursor at the following coordinates, press

Button 1 after each set:

-220  400

-220  100

In the following step 6, to get a better view of the area, it is not

necessary press button 2.

EXERCISE 13-1: Creating a Homogeneous Part.

-------------------------------------------

Page 13-24.  When you assign the footprint attribute in step 11,

"text justification" should read "text orientation".

EXERCISE 13-2: Creating a Nonhomogeneous Part.

----------------------------------------------

Work in symbol mode (SYMB) when you do this exercise.

EXERCISE 13-3: Creating Padstacks.

----------------------------------

Work in symbol mode (SYMB) when you do this exercise.

Page 13-37.  After you add the first clearance pad in step 5,

select REDR so your drawing area will look similar to Figure 13-

19.

Page 13-43.  When you add the pin graphic in step 13, the

instructions should read:

> Change the active layer to PIN.

Select DRAW/CIRC.

Select the center point at coordinates 0 0 and the point

on the circumference at coordinates 20 0.

EXERCISE 13-4: Attaching and Detaching Padstacks.

-------------------------------------------------

Page 13-45.  After you load the C7 database in step 1, follow

these instructions:

> Select VLYR.

Enable the PIN layer and make it active.

Turn off the COMP and SOLDER layers.

Press Button 2 to return to the working screen.

Your drawing area should look similar to Figure 13-23 except your

screen will show vias.

EXERCISE 14-2: Packaging a PCB.

-------------------------------

Page 14-41.  The cross-reference file, C7.FIL was not included on

the original tutorial disk.  In order to complete this exercise,

make sure this file is in your \PCAD\TUTOR subdirectory so that

you can complete the exercises which use a cross-reference file.

Page 14-46.  After you load the C7.PKG database in step 7, follow

this instruction so you are zoomed in a little more:

> Select ZIN.

Position the cursor in the center of the working area.

Press Button 1 once or twice.

Your screen drawing area should look similar to Figure 14-20.

EXERCISE 15-3: Placing Your Components.

---------------------------------------

Pages 15-31 and 15-32.  The barrier you draw in step 8 will not

match the barrier shown in Figure 15-18.  This will not affect

the exercise and you will be successful in creating a placement

barrier.

Page 15-34.  The swap report shown in Figure 15-19 is not the

swap report for the database you used in the exercise.  The

correct swap report is shown in the file XC7A.RPT.

Page 15-35.  The placement report shown in Figure 15-20 is not

the placement report for the database you used in the exercise.

The correct placement report is shown in the file XC7A.PLR.

EXERCISE 16-1: Preparing the Database.

--------------------------------------

Page 16-22.  When you save the database and clear the screen in

step 4, you do not need to select YES.