ProDesigner IIs

Windows 3.1 Video Driver Installation Instructions

This driver disk is designed to allow you to select or change the

video driver for Windows 3.1 using Windows Setup. Setup can be

executed from within Windows, or from DOS. Note that these driver

files are in Microsoft compressed format and can only be accessed and

executed by installing through Windows Setup. Simply copying the files

into your \Windows\System subdirectory WILL NOT work and may corrupt

your Windows installation. Installing the drivers correctly is,

however, a simple process. It is accomplished by following these

instructions:

Installation from floppy disk

1) Insert your Driver Disk in the appropriate floppy drive.

2) From your Windows directory, type SETUP and select DISPLAY to

change the video driver. Or, with Windows running, select Setup from

the Program Manager Main group. Refer to your Microsoft Windows User's

Guide for assistance in running Setup from inside Windows.

3) Select "Other disk..." to direct the program to the floppy disk

holding the new drivers. Note that after the drivers are installed,

you will not have to select "Other disk..." again should you decide to

change resolutions. Look for the ProIIs drivers in the Display

selection menu to select the display resolution you want.

4) When asked where the drivers will be found, state the following:

A:\

Following instructions 6-7 below.

Installation from Hard Disk

1) Having the drivers loaded on the hard disk will make it easier and

faster to change resolutions in the future. Make a subdirectory to

hold the driver files.

example: C:>md \PROIIS

and switch to that subdirectory (C:>cd \PROIIS)

2) Insert the drivers disk into the appropriate drive and type the

following command:

COPY A:\*.*

3) From your Windows directory, type SETUP and select DISPLAY  to

change the video driver. Or, with Windows running, select Setup from

the Program Manager Main group. Refer to your Microsoft Windows User's

Guide for assistance in running Setup from inside Windows.

4) Select "Other disk..." to direct the program to the subdirectory

holding the new drivers. Note that once the drivers are installed, you

should not have to select "Other" again should you decide to change

resolutions. Look for the ProIIs drivers in the Display selection

menu to select the display resolution you want.

5) When asked where the drivers will be found, state the following

(deleting what the program has displayed in the input window):

[drive:]\[directory]

e.g.: C:>\PROIIS

6) A standard Windows selection menu will appear with additional video

drivers from which to select.

7) Proceed normally through the remainder of the process. Check

Microsoft Windows installation procedures if necessary.

8) If you are having problems installing these drivers please contact

Orchid Technical Support at (510) 683-0323.

** IMPORTANT **

The 800x600 and 1024x768 256-color Turbo drivers (ET4TURBO) require 1mb of

display memory to be installed on your video board.