OVERVIEW   HIGHLIGHTS

o   New, price-performance standard in an Intel 80286-based personal

computer

o   640 KB memory on the system board

o   Enhanced PC Keyboard

o   Supports both 3.5" and 5.25" Internal Diskette Drives

o   20 MB fixed disk file

DESCRIPTION

A 20 MB fixed disk and a 1.2 MB 5.25" diskette drive are

standard on the PC XT Model 286.  One additional diskette drive may

be internally installed as the B drive from the following new

offerings:

o   One 3.5" Internal Diskette Drive (mutually exclusive with the

4865 Model 2, 3.5" External Diskette Drive).

o   One High Capacity Diskette Drive

o   One 5.25" 360 KB Diskette Drive

New special features further enhance the performance and

function of the PC XT Model 286.

3.5" INTERNAL DISKETTE DRIVE

This feature is a half-high, double-sided, 3.5" diskette drive that

supports the 3.5" media with 720 KB of storage capacity.  The drive

utilizes the standard fixed disk and diskette drive adapter provided

on the PC XT Model 286.  When installed, the drive becomes the "B"

drive.  Initial Program Loading (IPL) from the 3.5" diskette is not

supported by the PC XT Model 286.  For operation, DOS users require

DOS 3.2 on a 5.25" diskette or fixed disk.  The 3.5" Internal

Diskette Drive is mutually exclusive with the 4865 Model 2, 3.5"

External Diskette Drive.

HIGH CAPACITY DISKETTE DRIVE

The High Capacity Diskette Drive is a half-high, 5.25", dual-sided

drive with 1.2 MB of storage capacity.  This drive is compatible with

the standard drive provided with the PC XT Model 286 in the "A"

position.  When installed as the second drive, the High Capacity

Diskette Drive becomes the "B" drive.  This drive utilizes the

standard fixed disk and diskette drive adapter provided on the PC XT

Model 286.

5.25" 360 KB DISKETTE DRIVE

This half-high, 5.25" diskette drive permits the exchange of

information between the PC XT Model 286 and other members of the IBM

PC family.  The drive is a dual-sided drive with 360 KB of storage

capacity.  When installed, the drive becomes the "B" drive.  This

drive utilizes the standard fixed disk and diskette drive adapter

provided on the PC XT Model 286.

SYSTEM PERFORMANCE

The IBM PC XT Model 286 processor is up to 2.5 times faster

internally than the prior PC XT family and up to 20 percent faster

than the Personal Computer AT (R) Model 239, depending upon specific

applications.

SYSTEM COMPARISON

5160              5162            5170

MODEL             MODEL           MODEL

089               286             339

Microprocessor      8088              80286           80286

4.77 MHz          6 MHz           8 MHz

0 wait-         1 wait-

state           state

Standard Planar     640 KB            640 KB          512 KB

Memory

Maximum Memory      640 KB            12.6 MB         15.5 MB

I/O/Slots           0-16 Bit          5-16 Bit        6-16 Bit

8-8 Bit           3-8 Bit         2-8 Bit

Clock/Calendar      No                Yes             Yes

Power Supply        130 W             157 W           192 W

Diskette and Fixed

Disk Adapter     2 Cards           1 Card          1 Card

Diskette            360 KB            1.2 MB          1.2 MB

(std)             (std)           (std)

720 KB            360 KB          360 KB

720 KB          720 KB*

Fixed Disk          20 MB             20 MB           30 MB

Drive            (85ms)            (85ms)          (40ms)

TECHINFO   TECHNICAL INFORMATION

SYSTEM UNIT

The IBM PC XT Model 286 provides the following standard functions and

characteristics:

o   640 KB of memory on the planar

o   6 MHz Intel 80286 microprocessor with zero wait-state

o   Five 16-bit and three 8-bit expansion slots

o   Time and date clock with battery backup (customer replaceable

battery)

o   One 1.2 MB 5.25" diskette drive (half-high)

o   One 20 MB/85ms fixed disk

o   One fixed disk and diskette drive adapter (occupies one 16-bit

expansion slot)

o   One serial/parallel adapter (occupies one 16-bit expansion slot)

o   Enhanced Personal Computer Keyboard

o   Hardware and software compatibility with current IBM PC family

o   BASIC language provided

o   157 watt universal power supply (auto-sensing for line frequency

and voltage)

o   One year warranty

o   Dimensions:

Length -- 500mm (19.6")

Depth  -- 410mm (16.1")

Height -- 142mm ( 5.5")

o   Weight:

12.7Kg (28 lbs)

o   Electrical:

90-137V, 57-63 Hz

180-265V, 47-53 Hz

o   Ambient Air Temperature:

System on:  15.6-32.2C (60 (degs)-90 (degs) F)

System off: 10.0-43C   (50 (degs)-110 (degs) F)

o   Humidity:

System on:   8-80% RH

System off: 20-80% RH

o   BTU Output:

824 BTU/hr

o   Noise Level:  Class 3 (office environment)

o   FCC Class:   B

o   Electrical current leakage:  The IBM Personal Computer XT Model

286 is specifically designed to meet current leakage requirements

established by the National Fire Protection Code NFPA76B for data

processing use in hospital environments.

The electrical current leakage of the PC XT Model 286

averages 350-450 microamps for the 230 volt range and does not

exceed 225 microamps for the 115 volt range.

Please consult local ordinances, the National Fire

Protection and UL codes for specific details on use in the

hospital environment.

SOFTWARE REQUIREMENTS:  IBM PC DOS users require DOS version 3.2.

IBM Personal Computer XENIX (TM) Operating System users require XENIX

Version 2.0.

XENIX is a trademark of Microsoft, Inc.

####################################