WINBENCH

PC Labs Windows Graphics Benchmark

Version 2.50 - 02 Mar 92

1. This release of WINBENCH adds the WINMARK, the geometric mean of twelve

frequently used Windows graphics operations.

Contents of this release:

WINBENCH.EXE   WINBENCH executable, all tests enabled

WINBENCH.HLP   Help file

WINBENCH.DOC   This file in ASCII text format

*.XLS          Sample reports used by the Compare Results menu

2. Installation and Use

Copy all files from the WINBENCH diskette to the same directory.  Run

WINBENCH like any Windows application.  For example, the DOS command

"win winbench" will launch WINBENCH from the current directory.

WINBENCH writes the files containing test results to the directory

from which it was launched.  Generally, you should launch WINBENCH

every time from the directory in which it is installed.  This avoids

having test results scattered all over your hard disk.

For other information about WINBENCH, see the help dialogue box.

3. A Summary of the Differences in WINBENCH 2.50 from WINBENCH 1.1

3.1. New Features in WINBENCH 2.50

-    WINBENCH 2.5 introduces the graphics WINMARK, the geometric mean

of eleven graphics benchmark tests.  PC Labs profiled the major

Windows 3.0 applications in November 1991, and determined that these

eleven functions were the most frequently used.  The weights shown in

the WINMARK results screen show the approximate proportions in which

these functions are used.  Windows applications also use other

graphics functions, but for much less time.  Because the software

available to profile Windows applications at the time was not very

sophisticated, the weights assigned are subject to change when better

Windows profiling and instrumentation software becomes available.

-    When WINBENCH starts up, you now have the choice to run the

graphics WINMARK tests, to run detailed tests, or to exit from

WINBENCH.  WINBENCH now has other push buttons and menu choices for

the graphics WINMARK.

3.2. Corrected Defect in WINBENCH 2.04

-    WINBENCH 2.04 corrects an error in calculating the result of the

Rectangles|Patterns|Hatched Patterns test.  Previously, the result had

been stated as 20% greater than what it actually was.

-    WINBENCH 2.04 corrects several minor internal logic errors that

caused it to fail when compiled with other compilers.  None of these

changes have any effect on results.

3.2. Corrected Defect in WINBENCH 2.03

-    WINBENCH 2.03 displays an informational dialog box when no .XLS

files are available for comparison.  Previous versions of WINBENCH

presented an empty dialog box if no .XLS files were available for

comparison, and clicking on OK would have no effect.  The only

resolution to this problem was to change the Windows focus and

terminate Windows.

3.3. Corrected Defects and Improvements in WINBENCH 2.02

-    Corrected the processing of files written in Lotus 1-2-3 .WKS

format.  WINBENCH 2.00 or 2.01 failed with a UAE when writing a file

in Lotus 1-2-3 format.

-    Corrected the Select Results dialog box so that a double mouse

click selects a file.  Previously, a double click only highlighted the

file, and a click on the OK button was required to confirm the choice.

Corrected the Compare Results dialog box so that it would fit

completely on a 640x480 VGA screen.  Previously WINBENCH displayed it

in a format that was too wide for VGA.

Changed the Modify Options dialog box, so that the optional field

"File Header" is now the mandatory field "Test Description".

Changed the report file formats so that the test description

information and test date/time appear in a structured format.  This

gives the user more useful information when either viewing results in

the Compare Results dialog box or comparing report files.  This added

information is also displayed by the System Information menu choice.

3.4. Corrected Defects in WINBENCH 2.01

-    Corrected the Select Results dialog box so that it does not show

the current report file as a choice.  Also, the Select Results dialog

box is wider, allowing twelve character file names to display

completely.

-    Eliminated the possibility of a UAE from the System Information

Option caused by a rapid sequence of many Ctrl-I keystrokes in

succession, typically 100 or more, and only under Windows 3.0, not

Windows 3.1.  Under these circumstances, Windows 3.0 exhausts its use

of system memory.

-    Added a message box to the View Results dialog box to inform user

that file being compared contains results from more than one test run.

Also, added logic to choose only the first information with a given

text key from a file.

-    Added a string resource to change the minimum test duration, with

a minimum of two seconds.

-    The Bitblt Align test dialog box now displays results correctly.

Previously, the displayed results were transposed between lower left

and upper right cells, but the information contained in report files

was correct.

3.5. New Features in WINBENCH 2.0

-    System Information adds the processor type and clock speed,

presence of math coprocessor, Windows version number and operating

mode, video driver file name and description, video driver file date,

video driver version, X-Width Aspect Ratio, Y-Width Aspect Ratio,

height of the system font, and average character width of the system

font.  All system information is written to the report file.

-    All tests now run for a minimum of five seconds, and report

throughput, rather than elapsed time.  There is no method to convert

results from WINBENCH 1.1 into the format for the results used by

WINBENCH 2.0.  Many of the tests are upgraded to be consistent with

Windows 3.0 and/or Windows 3.1.

-    All tests display text centered in the benchmark window to

identify the current test being run in a series of tests.  Identifying

the test when it is running keeps you better informed about what is

going on, and helps to debug WINBENCH and/or Windows should WINBENCH

terminate with an Unrecoverable Application Error (UAE).

-    Many tests change the color being displayed to give a better

visual indication that something is actually being done while the test

runs.

-    Where possible, throughput is either calculated or estimated as

"pixels/sec".  Results of text handling benchmarks are reported in

"characters/sec".  Results for some other tests are reported in

"operations/sec".

-    The Ellipse (Curves) test renames the WINBENCH 1.1 Full Arcs

test, which drew ellipses on the screen then filled the ellipse with

the current brush.  The Ellipse test also draws ellipses on the

screen, but it varies the brush color used to fill the ellipse.

-    The Patterns (Rectangles) test now runs ten tests, using a black

fill pattern, seven grey patterns, a white pattern, and a set of

hatched patterns.

-    The Scroll Text (Text) tests are new.  They display and scroll a

full screen of text using the same five fonts as displayed by the

Image Text (Text) tests.

-    The Mixed Fonts (Text) test uses Courier fonts rather than System

fonts.  With Windows 3.0, it is not possible to display a system font

with a bold attribute.

-    The Help dialogue box provides a complete description of all

tests.

-    The Modify Options menu writes to the WINBENCH.INI file in a

section called [Version.2.0] when you choose to save selected

parameters.  WINBENCH 1.1 used the [Bench] section of WIN.INI for the

same purpose.

-    The ASCII text report file now lines up data in vertical columns.

The "1-2-3" format for a report file produces comma-separated data,

and non-numeric literals are bounded by double quotes, e.g. "sample

text".  The Excel format produces tab-separated data.

-    Because all tests run for at least five seconds, there is no need

to use radio buttons to select the number of test iterations.  These

radio buttons are removed from WINBENCH 2.0.

-    If you run all tests or run in demo mode, WINBENCH displays an

appropriate message on screen when it is done, or after running of

tests has been interrupted.

-    If WINBENCH is run in real mode and there is enough memory to

load it, it displays the initial dialog box, then a message box saying

that it does not run in real mode, and terminates.  If there is not

enough memory to load it, then Windows displays a message box with an

application error.

-    The Compare Results menu choice presents a pair of dialog boxes.

The first one displays a list of existing .XLS files, from which one

is selected to compare with the results of testing for the current

video card and driver.  The second dialog box displays test results

for the current card and the selected card.  The results of each test

written to the report file now all have unique names or keys so that

the Compare Results dialog box can select them properly from files

that contain the results of previous tests.

-    WINBENCH 2.50 was compiled with Borland C++ Release 3.0 and its

resources edited with Borland's Resource Workshop editor.  Tests

between versions of WINBENCH compiled with various compilers show less

than one percent difference in test results.  The source code for

WINBENCH is fully compatible with the following:

Borland C++3.0 Professional

Microsoft C6.0a and SDK

Symantec/Zortech C++3.0 (requires Microsoft LINK instead of

Zortech ZLINK)

Watcom C8.5 (requires Microsoft LINK rather than Watcom WLINK,

due to a defect in WLINK)

JPI TopSpeed C3.02 (requires compilation for math coprocessor

rather than math emulation, due to defect in TopSpeed Windows math

emulation library)

4. Problems and Suggestions for Improvements

-    Please note and report any test results that seem to be

incorrect, including zero and negative results.  These indicate that

the test being run uses an incorrect throughput calculation, either

roundoff or overflow.

-    Please provide hard copy or electronic copy of test results with

any problems, and describe the video graphics adapter being used

particularly if the System Information are not completely self-

descriptive.

-    If you have suggestions for improvements to WINBENCH, please

call, write, or send an electronic mail message explaining what you

require.  There is no guarantee that it will be added to the next

version of WINBENCH, but it WILL be added to the list of enhancements

that will be evaluated and prioritized for subsequent versions.

Thank you.

Ben Myers

Spirit of Performance, Inc.

73 Westcott Road

Harvard, MA  01451-1821

tel. 508/456-3889

MCI  357-1400

Jim Galley

Director, PC Labs

PC Magazine

One Park Avenue

New York, NY  10016

tel. 212/503-5587