#ifndef __IODEF_H

#define __IODEF_H

#define PC_DMA 0x000  // DMA controller 8237A

#define PC_PIC 0x020 // Interrupt controller, 8259A.

#define PC_TIM 0x040

#define PC_KBD_DATA 0x060

#define XT_PPI 0x060

#define AT_KBD_DATA 0x060

#define AT_KBD_CMD 0x064

/* This port communicates with the 8042 which runs an on-chip control program for the keyboard. It accepts command codes and data bytes. */

#define PC_CMOS_ADR 0x070

#define PC_CMOS_DAT 0x071

#define AT_NMI_MASK 0x70

/* Use this port to mask the Non-maskable Interrupt.

An OUT with bit 7=0 enables the NMI;

with bit 7=1 disables NMI:

OUT 70H,2dH ;enable NMI

--2dH is a "reserved address" in the CMOS Memory

OUT 70H,0adH ;disable NMI

*/

#define AT_DIAG 0x080

/* manufacture's diagnostic checkpoint

page register used during POST to accumulate errors */

#define PC_DMAPG_0 0x080 /* 74LS612 */

#define PC_DMAPG_1 0x081

#define PC_DMAPG_2 0x082

#define PC_DMAPG_3 0x083

#define PC_NMI_MASK 0x0a0

/*

OUT 0a0H,80H ;enable NMI (RAM-parity-err)

OUT 0a0H,0 ;disable NMI

*/

#define AT_PIC 0x0A0 /* 8259A. */

#define AT_DMA 0x0c0 /* 8237A-5 */

#define AT_287_LATCH_CLEAR 0x0f0

/*

'Busy' gets latched upon an unmasked 287 error.

OUT 0f0H,0 ;clears the latch

*/

#define AT_287_RESET 0x0f1

/*

Coprocessor is reset from protected mode to real mode.

OUT 0f1H,0 ;resets the 80287 to real mode

*/

#define AT_287_DATA 0x0f8

/*

80287 data. 286 sends opcodes and operands and receives results.

AT_HD2 0x170

*/

#define AT_HD1 0x1f0

#define PC_GAME 0x200

/* Game I/O (joystick A/D port) Joystick Port */

#define XT_EXPANSION 0x210

#define PC_PAR3 0x278 /* with MDA */

#define PC_EGA2 0x2c0

#define PC_COM4 0x2e8

#define PC_COM2 0x2f8

#define PC_PROTOTYPE 0x300

#define XT_HD 0x320

#define PC_FD2 0x370

#define PC_PAR2 0x378

#define PC_SDLC1 0x380

#define PC_MDA1 0x3b0

#define PC_PAR1 0x3bc /* MDA */

#define PC_EGA1 0x3c0

#define PC_CGA 0x3d0

#define PC_COM3 0x3e8

#define PC_FD1 0x3f0

#define PC_COM1 0x3f8

#if 0

3.1.6 Enhanced Graphics Adapter I/O Ports

The Enhanced Graphics Adapter is upwardly-compatible with the CGA when accessed through the BIOS. The <EGA> is far more complex at the hardware level, but it does include emulation of many CGA registers/operations. See CGA I/O Ports This topic covers the EGA changes and extensions to CGA I/O usage.

Perhaps the most significant change for an applications programmer is that you don't need to check for retrace when writing to the screen. The EGA will not allow "video snow" in any case.

EGA #1 decodes ports 3c0H through 3dfH

EGA #2 decodes ports 2c0H through 2dfH

The EGA can generate interrupt level 2 (IRQ 2) during the overscan at the start of the vertical retrace.  IRQ 2 is vectored to INT 0aH.  If you use this interrupt, be aware that the �AT� uses IRQ 2 as a cascade request for a whole class of IRQs (IRQ 8-15). Techniques for handling multiple hardware interrupts through the same IRQ are covered in the AT Tech Ref 6183355 (Revised Edition, March 1986).

Note: BIOS stores mirrors of the current CRTC port values and other EGA

information in <EGA> Data Areas  To change values, you should read

the appropriate variable, set the bit, perform the OUT, and store the new

value in the variable.

Port Description

���� ��������������������������������������������������������������������������

3c0H Write: Attribute Controller (ATC) address and data port

Perform IN al,3daH  ;(or IN 3baH) to force address mode

OUT 3c0H,reg_no ;select ATC register (toggle to data mode)

OUT 3c0H,value ;store a value in an ATC register

Note: bits 0-4: select an ATC register

bit 5: 1=enable display; 0=set register

After selecting a register, perform another OUT 3c0H,data as below:

ATC Reg Data description

������� ������������������������������������������������������������

00-0fH palette registers: select colors for this attribute

�7�6�5�4�3�2�1�0�

�  �r�g�b�R�G�B�  Note: R,G,B are 2/3-intensity

���������������ҽ bit r,g,b are 1/3-intensity

� � � � � ��> 0: Red

� � � � ����> 1: Green

� � � ������> 2: Blue

� � ��������> 3: blue' (also MDA-emulation underline)

� ����������> 4: green' (also CGA-emulation intensity)

������������> 5: red'

10H mode control

�7�6�5�4�3�2�1�0�

�  � � � � �

���������������ҽ bit

� � � ��> 0: 1=graphics modes, 0=text

� � ����> 1: 1=MDA attributes, 0=color attributes

� ������> 2: 1=9th dot is 8th dot for line/box chars

� 0=use bkgd color as 9th dot of char

��������> 3: 1=enable blink, 0=allow 4-bit bkgd

11H  select overscan (border) color. Same rgbRGB as palette regs

12H  enable color planes

�7�6�5�4�3�2�1�0�

�  �  �  �

���������������Ľ bit

�˼ ��������> 0-3: set bits to enable color planes 0-3

�����������> 4-5: video status to multiplex

00=R&B, 01=b&G, 10=r&g, 11=unused

13H  hori

ontal pixel panning

Number of dots to shift data left. Bits 0-3 valid (0-0fH)

3c2H Write: miscellaneous control functions

�7�6�5�4�3�2�1�0�

� � � � �  � � �

���������������ҽ bit

� � � � �˼ � ��> 0: 1=select 3BxH (emulate MDA); 0=select 3DxH (CGA)

� � � � � ����> 1: 1=enable RAM; 0=disable RAM

� � � � �������> 2-3: Dot clock: 00=14mH

; 01=16mH

; 10=external; 11=res

� � � ����������> 4: 1=select feature output; 0=select internal

� � ������������> 5: Page bit for odd/even mode

� ��������������> 6: Hori

ontal retrace polarity: 1=negative, 0=positive

����������������> 7: Vertical retrace polarity: 1=negative, 0=positive

3c4H Write: sequencer address register

3c5H Write: sequencer data register

Perform OUT 3c4H,reg_no; then OUT 3c5H,data

Seq Reg Data description

������� ������������������������������������������������������������

00 sequencer reset.

Bits 0-1 indicate asynchronous/synchronous reset.

01 clocking mode

�7�6�5�4�3�2�1�0�

�  � � � � �

���������������ҽ bit

� � � ��> 0: 1=8 dots per character; 0=9 dots/char

� � ����> 1: CRT bandwidth: 1=low; 0=high

� ������> 2: 1=shift every char; 0=every 2nd char

��������> 3: dot clock: 1=halved; 0=normal

02  map mask: bits 0-3 enable writes to bit planes 0-3

03  character map select:

bits 0-1 select Map B (for when attribute bit 3=0)

bits 2-3 select Map A (for when attribute bit 3=1)

04  memory mode

�7�6�5�4�3�2�1�0�

� � � � �

���������������ҽ bit

� � ��> 0: 1=text char generator; 0=graphics

� ����> 1: 1=EGA ram > 64K; 0=64K

������> 2: 1=sequential; 0=odd/even (as in CGA)

3caH Write: graphics 2 position (must be 0 for EGA)

3ccH Write: graphics 1 position (must be 1 for EGA)

3ceH Write: graphics 1 and 2 address register

3cfH Write: graphics controller data register

Perform OUT 3ceH,reg_no; then OUT 3cfH,data

GDC Reg Data description

������� ������������������������������������������������������������

00  set/reset. bits 0-3 select planes for write mode 00

01  enable set/reset

02  color compare. Bits 0-3 select color for read mode 01

03  data rotate and function select for write mode 00

bits 0-2: set rotate count for write mode 00

3-4: fn select for write modes 00 and 02

00=no change; 01=AND; 10=OR; 11=XOR

04  read map select. Bits 0-2 select map number for read mode 00

05  mode register

�7�6�5�4�3�2�1�0�

�  � � � � �  �

���������������Ľ bit

� � � � ����> 0-1: Write mode 0-2

� � � ������> 2: test condition

� � ��������> 3: read mode: 1=color compare, 0=direct

� ����������> 4: 1=use odd/even RAM addressing

������������> 5: 1=use CGA mid-res layout (2-bits/pixel)

06  miscellaneous graphics control

�7�6�5�4�3�2�1�0�

�  �  � � �

���������������ҽ bit

�˼ � ��> 0: 1=graphics; 0=enable char gen (text)

� ����> 1: 1=chain odd maps after even maps

�������> 2: memory mapping for CPU

00=a000H (128K); 01=a000H (64K)

10=b000H (32K MDA); 11=b800H (32K CGA)

07  color masking disable

bits 0-3 disable planes from compare logic in read mode 01

08  bit mask. Bits 0-7 select bits to be masked in all planes

3b4H or 3d4H CRT controller address

3b5H or 3d5H CRT controller internal registers

Note: Port 3c0H bit 0 selects which port address-pair to use.

(3bxH is normal for MDA; 3dxH is normal for CGA).

Perform OUT 3x4H,reg_no; then OUT 3x5H,data

CRTC Rg Data description

������� ������������������������������������������������������������

00-11H See Video Parameter Table and CGA I/O Ports  Exceptions:

02 start hori

ontal blanking (character count)

03 end hori

ontal blanking

bits 0-4 are blank width; bits 5-6 are skew enable (0 to 3)

04 start hori

ontal retrace (character position)

05 end hori

ontal retrace

�7�6�5�4�3�2�1�0�

� �  � �

���������������Ľ bit

� �˼ ����������> 0-4: hori

ontal retrace width

� �������������> 5-6: retrace delay 0-3 character-clocks

����������������> 7: 1=start panning on odd address; 0=even

06 vertical total (scan lines)

07 bit 8 for certain CRTC registers

�7�6�5�4�3�2�1�0�

�  � � � � � � �

���������������ҽ bit

� � � � � ��> 0: vertical total (CRTC Reg 06)

� � � � ����> 1: vert disp'd enable end (CRTC Reg 12H)

� � � ������> 2: vertical retrace start (CRTC Reg 10H)

� � ��������> 3: start vertical blanking (CRTC Reg 15H)

� ����������> 4: line compare (CRTC Reg 18H)

������������> 5: cursor location (CRTC Reg 0aH)

08 preset row scan first scan line after vertical retrace

10H Write: vertical retrace start

11H Write: vertical retrace end

�7�6�5�4�3�2�1�0�

�  � � �  �

���������������Ľ bit

� � ��������> 0-3: retrace width in scan lines

� ����������> 4: 0=clear vertical interrupt

������������> 5: 0=enable vertical interrupt IRQ2

12H vertical displayed end (scan line)

13H vertical displayed adjustment

14H underline location. Bits 0-5 are scan line number.

15H start vertical blanking (scan line)

16H end vertical blanking

17H mode control

�7�6�5�4�3�2�1�0�

� � � � � � � � �

���������������ҽ bit

� � � � � � � ��> 0: 0=CGA graphics addr compatibility mode

� � � � � � ����> 1: select row scan counter

� � � � � ������> 2: hori

retrace: 1=double vertical res

� � � � ��������> 3: 1=use char clock /2 for word refresh

� � � ����������> 4: 1=force high impedance on output

� � ������������> 5: address wrapping mode

� ��������������> 6: 1=byte addressing mode; 0=word mode

����������������> 7: 0=hardware reset; 1=normal operations

18H line compare (scan line) Used for split screen operations.

3baH or 3daH Write: bits 0-1 control Feature Control outputs FC0 and FC1

Note: Port 3c0H bit 0 selects which port address to use.

Read: status bits are same as MDA (port 3baH) or CGA (port 3daH)

�7�6�5�4�3�2�1�0�

�  � � � � �

���������������ҽ bit

� � � ��> 0: retrace. 1=display is in vert or hori

retrace.

� � ����> 1: 1=light pen is triggered; 0=armed

� ������> 2: 1=light pen switch is open; 0=closed

��������> 3: 1=vertical sync pulse is occurring.

Note: With EGA, you don't need to synchroni

e with retrace when writing

to video RAM.

3bdH or 3dbH Write: light pen latch reset. Any OUT clears the latch.

3bcH or 3dcH Write: light pen latch set. Any OUT arms the latch.

PPI

#define XT_PPI_B 0x061

�7�6�5�4�3�2�1�0�

� � � � � �0� � �

���������������ҽ bit

� � � � �   � ��> 0: Timer 2 gate (speaker) ���> OR 03H=speaker ON

� � � � �   ����> 1: Timer 2 data ����������ͼ AND 0fcH=speaker OFF

� � � � ��������> 3: 1=read high switches; 0=read low switches (see 62H)

� � � ����������> 4: 0=enable RAM parity checking; 1=disable

� � ������������> 5: 0=enable I/O channel check

� ��������������> 6: 0=hold keyboard clock low

����������������> 7: 0=enable keyboard; 1=disable keyboard

#define XT_PPI_C 0x062

062H �PC/XT� PPI port C.

�7�6�5�4�3�2�1�0�

� � � �0�equip't�

���������������Ľ bit

� � �  ��������> 0-3: values of DIP switches. See Equipment List

� � ������������> 5: 1=Timer 2 channel out

� ��������������> 6: 1=I/O channel check

����������������> 7: 1=RAM parity check error occurred.

#define XT_PPI_CMD 0x063

BIOS sets to 99H (Ports A and C are input, B is output).

#endif

#endif //__IODEF_H