rs232_1          RS-232 port addresses

prn_1            Printer port addresses

equip_install    Equipment installed info bits

15  14  13   12   11  10   9   8

\    /      game  \       /

# of print  port  # of RS-232

ports 0-3   used   ports 0-4

7   6    5    4    3   2   1   0

\   /    \    /    \   / Math  |

# of   video mode  RAM    uP  no

disk-  at boot up  00=16K    dsk

ettes  00=EGA      01=32K   driv

1-4   01=CGA-40   10=48K   if 0

if bit 10=CGA-80   11=64K

0 = 1  11=MDA-80   (old PCs)

Note: bit 13=modem on PC lap-tops

bit  2=mouse on PS/2

init_test_flag   Initialization test

main_ram_size    Base memory size 0-1Meg, 1K steps

chan_io_size     Channel i/o size

���������������������������������

�         KEYBOARD DATA         �

���������������������������������

keybd_flags_1    Keyboard flag bits

7   6   5   4    3   2   1   0

ins- cap num scrl alt ctl lef rig

sert --toggles--- --shifts down--

keybd_flags_2    Keyboard flag bits

7     6     5     4

insert  caps  num  scroll

------now depressed------

3     2     1     0

pause  sys   left right

lock request -alt-down-

keybd_alt_num    Alt & digit pad number buffr area

keybd_q_head     Head ptr of circular key queue

empty if head ptr = tail ptr

keybd_q_tail     Tail ptr of circular key queue

empty if head ptr = tail ptr

keybd_queue      15 key queue for keyboard

���������������������������������

�          DISK DATA            �

���������������������������������

dsk_seek_stat    Recalibrate floppy drive bits

3       2       1       0

drive-3 drive-2 drive-1 drive-0

bit 7 = interrupt flag

dsk_motor_stat   Motor running status & disk write

bit 7=1 disk write in progress

bits 6&5 = drive selected 0 to 3

3       2       1       0

drive-3 drive-2 drive-1 drive-0

--------- 1=motor on-----------

dsk_motor_tmr    Motor timer, at 0, turn off motor

dsk_ret_code     Controller return code

00h = ok

01h = bad command or parameter

02h = can't find address mark

03h = can't write, protected dsk

04h = sector not found

08h = DMA overrun

09h = DMA attempt over 64K bound

10h = bad CRC on disk read

20h = controller failure

40h = seek failure

80h = timeout, no response

dsk_ctrl_stat    Status bytes-disk controller chip

Note: 7 info bytes returned from

controller are saved here. Refer

to the NEC uPD 765 chip manual

for the specific info, depending

on the previous command issued.

���������������������������������

�          VIDEO DATA           �

���������������������������������

video_mode       Present display mode(see int 10h)

video_columns    Number of columns

video_buf_size   Video buffer size in bytes

Note: size may be rounded up to

the nearest 2K boundary.  For

example, 80x25 mode=4000 bytes,

but value may be 4096.

video_segment    Segment of active video memory

MDA=0B000h, CGA=0B800h, etc.

video_cur_pos0   Cursor position page 0

bits 15-8=row, bits 7-0=column

video_cur_pos1   Cursor position page 1

bits 15-8=row, bits 7-0=column

video_cur_pos2   Cursor position page 2

bits 15-8=row, bits 7-0=column

video_cur_pos3   Cursor position page 3

bits 15-8=row, bits 7-0=column

video_cur_pos4   Cursor position page 4

bits 15-8=row, bits 7-0=column

video_cur_pos5   Cursor position page 5

bits 15-8=row, bits 7-0=column

video_cur_pos6   Cursor position page 6

bits 15-8=row, bits 7-0=column

video_cur_pos7   Cursor position page 7

bits 15-8=row, bits 7-0=column

video_cur_mode   Active cursor, start & end lines

bits 12 to 8 for starting line

bits 4  to 0 for ending line

video_page       Present page

video_port       Video controller base I/O address

video_mode_reg   Hardware mode register bits

video_color      Color set in CGA modes

���������������������������������

�         GENERAL DATA          �

� Note: next 5 bytes also used  �

�    for cassette interface in  �

�    older PCs.                 �

���������������������������������

@gen_io_ptr      ROM initialization pointer

gen_io_seg       ROM i/o segment

gen_int_occurd   Unused interrupt occurred

timer_low        Timer, low word, cnts every 55 ms

timer_hi         Timer, high word

timer_rolled     Timer overflowed, non-zero when

more than 24 hours have elapsed

keybd_break      Bit 7 set if break key depressed

gen_warm_boot    Boot (reset) type

1234h=warm boot, no memory test

4321h=boot & save memory

(PS/2 with MCA only)

���������������������������������

�        HARD DISK DATA         �

���������������������������������

hdsk_status_1    Hard disk status

00h = ok

01h = bad command or parameter

02h = can't find address mark

03h = can't write, protected dsk

04h = sector not found

05h = reset failure

07h = activity failure

08h = DMA overrun

09h = DMA attempt over 64K bound

0Ah = bad sector flag

0Bh = removed bad track

0Dh = wrong # of sectors, format

0Eh = removed control data addr

mark

0Fh = out of limit DMA

arbitration level

10h = bad CRC or ECC, disk read

11h = bad ECC corrected data

20h = controller failure

40h = seek failure

80h = timeout, no response

AAh = not ready

BBh = error occurred, undefined

CCh = write error, selected dsk

E0h = error register = 0

FFh = disk sense failure

hdsk_count       Number of hard disk drives

hdsk_head_ctrl   Head control (XT only)

hdsk_ctrl_port   Hard disk control port (XT only)

���������������������������������

� I/O PORT TIMER, KEYBOARD DATA �

���������������������������������

prn_timeout_1    Countdown timer waits for printer

to respond (printer 1)

prn_timeout_2    Countdown timer waits for printer

to respond

rs232_timout_1   Countdown timer waits for RS-232

port to respond (port 1)

rs232_timout_2   Countdown timer waits for RS-232

port to respond

@keybd_begin     Ptr to beginning of keybd queue

@keybd_end       Ptr to end of keyboard queue

���������������������������������

�  ADVANCED VIDEO DATA, EGA/VGA �

���������������������������������

video_rows       Rows of characters on display - 1

video_pixels     Number of pixels per charactr * 8

video_options    Display adapter options

bit 7 = clear RAM

bits 6,5 = memory on adapter

00 - 64K

01 - 128K

10 - 192K

11 - 256K

bit 4 = unused

bit 3 = 0 if EGA/VGA active

bit 2 = wait for display enable

bit 1 = 1 - mono monitor

= 0 - color monoitor

bit 0 = 0 - handle cursor, CGA

video_switches   Switch setting bits from adapter

bits 7-4 = feature connector

bits 3-0 = option switches

video_1_resv     Video reserved 1, EGA/VGA control

bit 7 = 200 line mode

bits 6,5 = unused

bit 4 = 400 line mode

bit 3 = no palette load

bit 2 = mono monitor

bit 1 = gray scale

bit 0 = unused

video_2_resv     Video reserved 2

���������������������������������

� OTHER FLOPPY & HARD DISK DATA �

���������������������������������

dsk_data_rate    Last data rate for diskette

bits 7 & 6 = 00 for 500K bit/sec

= 01 for 300K bit/sec

= 10 for 250K bit/sec

bits 5 & 4 = step rate

hdsk_status_2    Hard disk status

hdsk_error       Hard disk error

hdsk_int_flag    Set for hard disk interrupt flag

hdsk_options     Bit 0 = 1 when using 1 controller

card for both hard disk & floppy

hdsk_media_st0   Media state for drive 0

hdsk_media_st1   Media state for drive 1

7      6      5      4

data xfer rate  two   media

00=500K bit/s  step  known

01=300K bit/s

10=250K bit/s

3      2      1      0

unused  -----state of drive-----

bits floppy  drive state

000=  360K in 360K, ?

001=  360K in 1.2M, ?

010=  1.2M in 1.2M, ?

011=  360K in 360K, ok

100=  360K in 1.2M, ok

101=  1.2M in 1.2M, ok

111=  state not defined

hdsk_start_st0   Start state for drive 0

hdsk_start_st1   Start state for drive 1

hdsk_track_0     Track number for drive 0

hdsk_track_1     Track number for drive 1

���������������������������������

�    ADVANCED KEYBOARD DATA     �

���������������������������������

keybd_flags_3    Special keyboard type and mode

bit 7 Reading ID of keyboard

6 last char is 1st ID char

5 force num lock

4 101/102 key keyboard

3 right alt key down

2 right ctrl key down

1 E0h hidden code last

0 E1h hidden code last

keybd_flags_4    Keyboard Flags (advanced keybd)

7      6       5     4  3 2 1 0

xmit   char   Resend  Ack   \   /

error was ID  Rec'd  Rec'd   LEDs

���������������������������������

�  REAL-TIME CLOCK & LAN DATA   �

���������������������������������

@timer_wait_off  Ptr offset to wait done flag

@timer_wait_seg  Ptr segment to wait done flag

timer_clk_low    Timer low word, 1 microsecond clk

timer_clk_hi     Timer high word

timer_clk_flag   Timer flag 00h = post acknowledgd

01h = busy

80h = posted

lan_1            Local area network bytes (7)

���������������������������������

�   MORE ADVANCED VIDEO DATA    �

���������������������������������

@video_sav_tbl   Pointer to a save table of more

pointers for the video system

SAVE TABLE

offset type    pointer to

������ ���� ��������������������

0     dd  Video parameters

4     dd  Parms save area

8     dd  Alpha char set

0Ch    dd  Graphics char set

10h    dd  2nd save ptr table

14h    dd  reserved (0:0)

18h    dd  reserved (0:0)

2ND SAVE TABLE (from ptr above)

offset type functions & pointers

������ ���� ��������������������

0     dw  Bytes in this table

2     dd  Combination code tbl

6     dd  2nd alpha char set

0Ah    dd  user palette tbl

0Eh    dd  reserved (0:0)

12h    dd  reserved (0:0)

16h    dd  reserved (0:0)

Days since 1-Jan-1980 counter

prn_scrn_stat    Print screen status

00h = Print screen ready

01h = Print screen in progress

FFh = Error occurred

���������������������������������

�     Floppy Disk Parameters    �

���������������������������������

hi nibble = stepping rate in ms

lo nibble = head unload time, ms

2nd info byte bit 0 = 0 for DMA

Delay after use for motor off

Bytes per sector  0 =  128 bytes

1 =  256 bytes

2 =  512 bytes

3 = 1024 bytes

Number of sectors per track

Gap Length

Data Length

Format Gap Length

Format write byte

Head load time, in milliseconds

Motor startup wait time * .125ms

���������������������������������

�    Video Hardware Registers   �

���������������������������������

mode CGA 40 columns x 25 lines

mode CGA 80 columns x 25 lines

mode CGA graphics

mode MDA 80 columns x 25 lines

Video buffer bytes CGA 40x25

Video buffer bytes CGA 80x25

Video buffer bytes CGA Graphics

Video buffer bytes CGA Graphics

Video columns per modes 0-7

Video hardware modes (0-7)

dsk_info_table   ���������������������������������

� Hard Disk Information Tables  �

�                               �

�   Each sub-table contains a   �

�   a set of 16 bytes for each  �

�   particular disk type.       �

�                               �

�   Type number specified may   �

�   differ with manufactures    �

�   specification.  The hard    �

�   disk table shown here       �

�   assumes the first entry     �

�   is type 0.                  �

���������������������������������

Number of cylinders, hdsk_type_0

Number of heads

Low write current cyl begin *

Write pre-compensation cylinder

Error correction burst length *

Miscellaneous bit functions:

bits 0-2 disk option, XT only*

0-2 unused, all others

3 = 1 if > 8 heads

4   unused

5 = 1 for bad map at last

cylinder + 1

6 or 7 = 1 no retries

Normal timeout *

Format timeout *

Check timeout  *

Parking cylinder number

Number of sectors per track

Unused

* indicates XT machines only

Unused

video_char_table Video characters in graphic modes

Characters, EGA  8x14 font

Characters, EGA  8x8 font (0-7F)

Characters, EGA  8x8 font (80-FF)

Character changes, EGA  8x14 font

Characters, VGA  8x16 font

Character changes, VGA  9x16 font

Video VGA parameter table ptr

int_vec_table    Initial interrupt vectors

rom_versn_date   BIOS version date code

model_type       Model  FFh = PC

FEh = 1st XT

FBh = later XTs

FCh = AT type (286/386)

FAh = models 25/30

F9h = IBM lap-tops

F8h = models 70/80 (80386)

BIOS_data_seg    Extended BIOS data (PS/2)or printer 4

Printer 4 (PC,XT,AT & compatibles)

VGA_parm_tbl     ���������������������������������

�   Video VGA Parameter Tables  �

�                               �

�   64 byte sub-table per mode  �

�  offset/size     function     �

�  -- --------  --------------  �

�   0  1 byte   # of columns    �

�   1  1 byte   # of rows       �

�   2  1 byte   # bytes/char    �

�   3  1 word   # bytes/page    �

�   5  5 bytes  sequencer regs  �

�  10 25 bytes  display regs    �

�  35 20 bytes  attribute regs  �

�  55  9 bytes  graphics regs   �

���������������������������������

baud_rate_tbl    ���������������������������������

�        Baud Rate Table        �

�                               �

�   Table of hex divsors  for   �

�   the serial ports.   Table   �

�   divisors for bauds 110 to   �

�   9600, and 19,200 on some.   �

���������������������������������

110 Baud rate divisor

Config_tbl_size  ���������������������������������

�   System Configuration Table  �

���������������������������������

Size of table in bytes

08E Model type

0F8h = 80386 model 70-80 types

0FCh = 80286 model 50-60 types

also most 80286/80386

compatibles

0FAh = 8088/86 model 25-30 type

08F Sub-Model type

090 BIOS revsion number

091 Feature information

bit 7=1, hard disk uses DMA 3

bit 6=1, dual interrupt chips

bit 5=1, has real-time-clock

bit 4=1, int 15h, ah=4Fh is

supported (keyboard)

bit 3=1, external wait support

bit 2=1, has extended BIOS RAM

bit 1=1, micro-channel

bit 0=1, unused

092 Information bytes (future use)

093 ���������������������������������

�       Interrupt Vectors       �

���������������������������������

Divide error

094 Single Step

095 NMI

096 Debug

097 Overflow

098 Print Screen

099 Timer Ticks

09A Keyboard

09B Serial

09C Serial

09D Disk

09E Diskette

09F Bad Interrupt

0A0 Video

0A1 Equipment

0A2 Memory Size

0A3 Disk Services

0A4 General Services

0A5 ROM Basic

0A6 ���������������������������������

�         ROM BEGINNING         �

���������������������������������

ROM signature word

0A7 Number of 512 byte pages

0A8 unused

0A9 ���������������������������������

�     FONT - 8 wide, 14 high    �

���������������������������������

Video character font

0AA ���������������������������������

�     FONT - 8 wide, 8 high     �

���������������������������������

Video characters 0-127

0AB ���������������������������������

�     FONT - 8 wide, 8 high     �

���������������������������������

Video characters 128-255

0AC ���������������������������������

�     FONT - 9 wide, 14 high    �

�                               �

�  First byte indicates the     �

�    character to replace       �

�  Followed by 14 bytes of data �

�    (the 9th bit is provided   �

�    by the EGA/VGA adapter     �

���������������������������������

Video character font

0AD ���������������������������������

�     FONT - 8 wide, 16 high    �

���������������������������������

Video character font

0AE ���������������������������������

�     FONT - 9 wide, 16 high    �

�                               �

�  First byte indicates the     �

�    character to replace       �

�  Followed by 16 bytes of data �

�    (the 9th bit is provided   �

�    by the EGA/VGA adapter     �

���������������������������������

Video character font

0AF ���������������������������������

�       VIDEO MODE TABLES       �

�                               �

� Note: The following structure �

�       is based on most EGA/   �

�       VGA adapters.  Since    �

�       they are used only      �

�       internally the table    �

�       may not follow the      �

�       standard shown.         �

�                               �

�  EGA Cards - Mode order       �

�       0-0Fh, 10h normal       �

�       0Fh, 10h  16K RAM EGA   �

�       0-3, high res emulate   �

�  VGA Cards - Mode order       �

�       0-Fh, 10-13h normal     �

�       0-3, high res emulate   �

�                               �

�  THIS IS A GUIDELINE AND MAY  �

�  VARY DEPENDING ON THE CARD   �

���������������������������������

Number of columns on display

0B0 Number of rows on display

0B1 Pixels per character

0B2 Bytes per page

0B3 Sequencer register values

0B4 Miscellaneous register value

0B5 CRT controller register values

0B6 Attribute register values

0B7 Graphics register values

0B8 System boot

*/

/*

01

���������������������������������������������������������� int 1 �������

SINGLE STEP

02

���������������������������������������������������������� int 2 �������

NON-MASKABLE HARDWARE INTERRUPT

This function is called in the event of a memory parity error or may

occur in the event of other hardware problems or failures (depends on

the specific manufacturer's hardware).  Displays the appropriate

error message and halts the processor.

03

���������������������������������������������������������� int 3 �������

DEBUG BREAKPOINT

04

���������������������������������������������������������� int 4 �������

OVERFLOW

05

���������������������������������������������������������� int 5 �������

PRINT SCREEN

Send the present active display screen contents to the printer if

the printer is idle and not out of paper.  The status of the print

screen function is in prn_screen_stat at 0050:0000.  Control-Break

will terminate an active print screen operation.

06

���������������������������������������������������������� int 6 �������

BAD OPCODE DETECTED

07

08

���������������������������������������������������������� int 8 �������

TIMER TICKS  (called by hardware 8259-1, IRQ 0)

This is the primary timer used to control the clock and other key

system resources.  It is called indirectly by channel A of the 8253

timer every 18.2 milliseconds.  Every timer tick also calls int 1Ch

for user needs (int 1Ch points to an iret instruction unless changed

by a resident program).  The timer interrupt is given the highest

maskable interrupt priority upon power up.

The main timing functions of int 8 include increment of a 32-bit

time since powered on counter, timer_hi and timer_low, and after 24

hours since powered on, timer_rolled is incremented.

The last task of int 8 is turning off the floppy drive motor after

2 seconds of non-use.  The counter dsk_motor_tmr is decremented upon

each occurrence of int 8.  When the count reaches zero, the motor is

set off, and the motor running flags are cleared in dsk_motor_stat.

09

���������������������������������������������������������� int 9 �������

KEYBOARD I/O  (called by hardware 8259-1, IRQ 1)

When any key is pressed on the keyboard, the hardware calls this

interrupt to service the pressed key or key combination.   The

hardware provides the key pressed in a non-ASCII scan code format

read at i/o port 60h.  The servicer acknowledges receipt of the key

by toggling bit 7 of port 61h.  (Port 61h should be read first, then

bit 7 ored on, output to port 61h, then anded off, and resent to

port 61h).

The read key is decoded to yield an ASCII character, special function

key (such as F1) or a control function like Left Shift Key down.

The converted ASCII character is placed into the next available

position in the circular queue keybd_queue to keybd_q_end.  It is

put in the position indicated by keydb_q_tail when it will not

cause the loss of earlier entered data.  The value keybd_q_head

points to the oldest key pressed in the buffer which has not been

removed from the queue (the normal process uses int 16h to remove

keys from the queue and return the key value to the int 16h caller).

The 16 word queue holds up to 16 keys.  If keybd_q_head equals the

keybd_q_tail, the queue is empty.  Valid keys in the queue comprise

the upper byte scan code and the lower byte ASCII character.  If the

key pressed has no ASCII equivalent (i.e F1 to F12), the lower byte

is zero.

Toggle and shift keys are not placed in the buffer, but appear in 2

status registers keybd_flags_1 and keybd_flags_2.

Special key combinations will cause other events to occur:

Ctrl-Alt_Del  -  Reset computer by jumping to power_on_reset

Print screen  -  Call int_5_prn_scrn to print the current screen

Ctrl-Break    -  Call int_1Bh control break key processor (DOS)

Pause         -  Wait until an ASCII key is pressed, without

placing the key in the queue

0A

���������������������������������������������������������� int 0Ah �����

LAN ADAPTER (called by hardware 8259-1, IRQ 2 - see below)

On systems equipped with 2 interrupt controller chips (8259), IRQ 2

is used to support the second interrupt controller.  In this case,

int 71h (IRQ 9) is used to replace IRQ 2.  Hardware calls to int 71h

are redirected to this interrupt to maintain compatibility.

0B

���������������������������������������������������������� int 0Bh �����

RS-232 SERIAL HANDLER (called by hardware 8259-1, IRQ 3)

Called by the secondary serial port chip.

0C

���������������������������������������������������������� int 0Ch �����

RS-232 SERIAL HANDLER (called by hardware 8259-1, IRQ 4)

Called by the primary serial port chip.

0D

���������������������������������������������������������� int 0Dh �����

XT HARD DISK CONTROLLER (called by hardware 8259-1, IRQ 5)

0E

���������������������������������������������������������� int 0Eh �����

FLOPPY DISK CONTROLLER (called by hardware 8259-1, IRQ 6)

0F

���������������������������������������������������������� int 0Fh �����

PRINTER CONTROLLER (called by hardware 8259-1, IRQ 7)

This interrupt is normally avoided.  If a bad interrupt occurs,

it will vector to this spot (when caused by a misprogrammed 8259

interrupt controller)

10

���������������������������������������������������������� int 10h �����

VIDEO DISPLAY SERVICES

Called with:  ah = primary function number

Returns: (unless otherwise indicated)

ax - altered (some systems may not alter ax)

Functions:

ah = 0     Set video display mode in al.

ah = 1     Set cursor size

Call with:  ch = top line, 0-32

cl = bottom line, 0-32

ah = 2     Set cursor location

Call with:  bh = page number, 0 = 1st page

dh = row (0 for top row)

dl = column (0 for leftmost)

ah = 3     Get cursor location

Call with:  bh = page number, 0 = 1st page

Returns:    ch/cl = cursor size (top & bottom)

dh/dl = row and column of cursor

ah = 4     Get light pen location

Returns:    ah = 0 not on/unsupported & bx,cx,dx changed

1 status valid

bx = pixel column

cx = horizontal line number

dh/dl = row and column

ah = 5     Set Page number al, 0 = 1st page

ah = 6     Up scroll screen

Call with:  al = # bottom lines to clear, set 0 for all

bh = attribute to fill cleared lines

ch/cl = row/column of top left scroll corner

dh/dl = row/column of bottom right scroll corner

ah = 7     Down scroll screen

Call with:  al = # top lines to clear, set 0 for all

bh = attribute to fill cleared lines

ch/cl = row/column of top left scroll corner

dh/dl = row/column of bottom right scroll corner

ah = 8     Get character & attribute at cursor

Call with:  bh = page number, 0 = 1st page

Returns:    ah/al = attribute/character

ah = 9     Write character & attribute at cursor (graphics modes)

Call with:  al = character

bh = page number, 0 = 1st page

bl = attribute, bit 7 = 1 to xor with old color

cx = Number of same characters to write

ah = 0Ah   Write character at cursor (graphics modes)

Call with:  al = character

bh = page number, 0 = 1st page

cx = Number of same characters to write

ah = 0Bh   Set colors (typically in low res modes)

Call with:  bh = 0, bl = color low res background, border

bh = 1, bl = 0/1 for low res color group 0/1

ah = 0Ch   Write graphics dot

Call with:  al = color, bit 7 = 1 to xor bit with old color

cx/dx = pixel row/pixel column

ah = 0Dh   Read graphics dot

Call with:  cx/dx = pixel row/pixel column

ah = 0Eh   Write in ASCII mode (cr, lf, bell, and bs as operators)

Call with:  al/bl = character/color

ah = 0Fh   Get video info

Returns:    ah = columns active

al = active video mode

bh = active page, 0 = 1st page

ah = 10h   Colors  (EGA/VGA)   Called with al = subfunction

al = 1    bl = color register (0-15)  bh = color to set

al = 2    es:dx = ptr to change all 16 colors & overscan #

al = 3    bl = color high bit as: 0-intensity, 1-blink

For VGA equipped systems only

al = 7    bl = color register to get in bh

al = 8    bh = returned overscan value

al = 9    es:dx = ptr where to put all 16 colors & overscan

al = 10h  bx = color register to set; ch/cl/dl = grn/blu/red

al = 12h  es:dx = ptr to change color registers

bx = 1st register to set, cx = # of registers

al = 13h  bl = 0, set color page mode in bh

bl = 1, set page;  bh = page number

al = 15h  bx = color reg to read; ch/cl/dl = grn/blu/red

al = 17h  es:dx = ptr where to load color regs

bx = 1st register to get, cx = # of registers

al = 1Ah  get color page info; bl = mode; bh = page #

ah = 11h   Reset mode with new character set (leave memory as-is)

al = 0    Character set load

es:bp = ptr to table of chars

bl/bh = which block/bytes per char

cx/dx = # of characters/where to start in block

al = 1    bl = block to load the mono character set

al = 2    bl = block to load the double wide character set

al = 3    bl = block select related to attribute

al = 4    bl = block to load the 8 by 16 set (VGA)

al = 10h - 14h same as 0 - 4, must occur after setting mode

al = 20h  es:bp = ptr to table of char, using int 1Fh ptr

al = 21h  es:bp = ptr to table of char, using int 43h ptr

bl = 0-dl=# rows; 1-14 rows; 2-25 rows; 3-43 rows

cx = number of bytes per character in table

al = 22h  use 8 by 14 char set, bl=rows (see al=21h)

al = 23h  use double wide char set, bl=rows (see al=21h)

al = 24h  use 8 by 16 char set, bl=rows (see al=21h)

Get table pointer and info

al = 30h  es:bp = ptr returned; cx=bytes per char; dl=rows

bh = 0, get int 1Fh      bh = 4, get double wide

bh = 1, get int 43h      bh = 5, get mono 9 by 14

bh = 2, get 8 by 14      bh = 6, get 8 by 16 (VGA)

bh = 3, get double wide  bh = 7, get 9 by 16 (VGA)

ah = 12h   Miscellaneous functions, bl = sub-function

bl = 10h  Get info,  bh = 0-now color, 1-now mono

ch/cl = info bits/switches

bl = 20h  Set print screen to work with EGA/VGA

Functions for VGA only (bl = 30-34h return al = 12h)

bl = 30h  Set # of scan lines, 0-200, 1-350, 2-400

activates after mode change

bl = 31h  ax = 0/1  allow/prevent palette load with new mode

bl = 32h  al = 0/1  video off/on

bl = 33h  al = 0/1  gray scale summing off/on

bl = 34h  al = 0/1  scale cursor size to font off/on

bl = 35h  switch between adapter and motherboard video

al = 0, adapter off, es:dx = save state area

al = 1, motherboard on

al = 2, active video off, es:dx = save area

al = 3, inactive video on, es:dx = save area

bl = 36h  al = 0/1  screen off/on

ah = 13h   Write character string (cr, lf, bell, and bs as operators)

Call with:  al = 0/1 write characters, cursor not/is moved

bl = attribute of all characters

= 2/3 write characters, cursor not/is moved

string alternates character & attribute

bh = page number, 0 = 1st page

cx = number of characters

dh/dl = row and column to start

es:bp = ptr to character string

ah = 14h   LCD display support, al = sub-function

al = 0,    es:di = ptr to font table to load

bl/bh = which block/bytes per char

cx/dx = # of characters/where to start in block

al = 1,    bl = block # of ROM font to load

al = 2,    bl = how to support high intensity

ah = 15h   LCD info table ptr returned in es:di

ax = 0      there is no alternate adapter

ax = 5140h  LCD is the alternate

ax = 5153h  CGA is alternate

ax = 5151h  MDA is alternate

**** VGA SUPPORT ONLY ****

ah = 1Ah   Get/Set types of displays which are operating

al = 0, get displays, returns al = 1Ah

bl = in use display code

bh = alternate display code

al = 1, set codes, returns al = 1Ah

bl = in use display code

bh = alternate display code

codes:   0 = no display

1 = mono display

2 = CGA

4 = EGA, color

5 = EGA, mono

6 = PGA

7 = VGA, mono  (PS/2 MCA only)

8 = VGA, color (PS/2 MCA only)

Bh = VGA, mono  (PS/2 non MCA)

Ch = VGA, color (PS/2 non MCA)

FFh = display type not known

ah = 1Bh   Get video system information

call with:  bx = 0

es:di = ptr to buffer where to put info

ah = 1Ch   Video system save & restore, al = sub-function

call with:  al = 0, get buffer size

= 1, save system, buffer at es:bx

= 2, restore system, buffer at es:bx

cx bit 0 = 1 for hardware registers

bit 1 = 1 for software states

bit 2 = 1 for colors & DAC registers

11

���������������������������������������������������������� int 11h �����

EQUIPMENT INSTALLED

Returns:   ax = equip_install word

15   14   13   12   11   10    9    8

\    /       game  \          /

# of printer  port   # of RS-232

ports 0-3    used    ports 0-4

7    6    5    4    3    2    1    0

\    /    \    /    \    /  Math   |

# of   video mode   RAM     uP    no

floppy  at boot up  00=16K       floppy

drives  00=EGA      01=32K       drive

1-4   01=CGA-40   10=48K       if 0

if bit  10=CGA-80   11=64K

0 = 1  11=MDA-80   (old PCs)

Note: bit 13 = modem on PC lap-tops

bit  2 = mouse on PS/2

Regs used: ax

12

���������������������������������������������������������� int 12h �����

SYSTEM MEMORY SIZE

Returns:   ax = number of contiguous 1K bytes blocks of memory

from 0 to 640 K.

Regs used: ax

13

���������������������������������������������������������� int 13h �����

FLOPPY DISK SERVICES

Note: Floppy disk services may be vectored to this routine from int 40h

on newer systems with hard disk support inside the BIOS.

Call with: ah = sub-function number

dl = drive number (unless otherwise noted), drive a: = 0

Returns:   carry = 0 if function ok

ah = status (unless otherwise noted)

ah = 0    Function ok

ah = 1    Invalid value passed or unsupported function

ah = 2    Can not locate address mark

ah = 3    Floppy write protected

ah = 4    Sector not found

ah = 6    Floppy changed line on (1.2 meg drives)

ah = 8    DMA overrun occurred

ah = 9    DMA attempted across 64K byte boundary

ah = 0Ch  Media type not found

ah = 10h  CRC read error

ah = 20h  Floppy controller failure

ah = 40h  Seek operation failed

ah = 80h  Floppy drive not ready

Functions:

ah = 0     Floppy disk controller reset

ah = 1     Get last status

ah = 2     Read sectors

Call with:  al = number of sectors to read

ch/cl = starting track/starting sector

dh = head number, head 0 = 0

es:bx = ptr to buffer where to put data

ah = 3     Write sectors

Call with:  al = number of sectors to write

ch/cl = starting track/starting sector

dh = head number, head 0 = 0

es:bx = ptr to buffer where to get data

Return:     al = number of sectors written

ah = 4     Verify sectors

Call with:  al = number of sectors to compare

ch/cl = starting track/starting sector

dh = head number, head 0 = 0

es:bx = ptr to buffer where to compare data

Return:     al = number of sectors checked

ah = 5     Format track

Call with:  al = number of sectors to format

ch/cl = starting track/starting sector

dh = head number, head 0 = 0

es:bx = ptr to table of address fields

ah = 8     Get drive information (not supported by all systems)

Returns:    ax/bh = 0          size dia.     size dia.

bl = drive type: 1=360K 5.25   3=720K 3.5

2=1.2M 5.25   4=1.4M 3.5

cx bits 7&6, 15-8 = number of tracks, 0=1 track

bits 5-0 = number of sectors per track

dh/dl = # of heads/number of floppy drives

es:di = ptr to floppy parameter table

ah = 15h   Get drive type (not supported by all systems)

Returns:    ah = 0 no drive

ah = 1/2  floppy changed line not/is available

ah = 16h   Get changed floppy status (not supported by all systems)

Returns:    ah = 0   floppy in drive,     carry = 0

ah = 1   bad drive number,    carry = 1

ah = 6   floppy out of drive, carry = 1

ah = 80h drive not ready,     carry = 1

ah = 17h   Specify media type for a drive

Call with:  al = 1  use a 320/360K floppy in 360k drive

al = 2  use a 360K floppy in a 1.2M drive

al = 3  use a 1.2M floppy in a 1.2M drive

al = 4  use a 720K disk in a 720K drive

ah = 18h   Prepare for format (not supported by all systems)

Call with:  cx bits 7&6, 15-8 = number of tracks, 0=1 track

bits 5-0 = number of sectors per track

Returns:    es:di = ptr to floppy parameter table

14

���������������������������������������������������������� int 14h �����

RS-232 SERIAL PORTS SERVICES

Call with: ah = sub-function number

dx = communications number, com1 = 0

Returns:   ah = line status,  bit 0 = Data ready

bit 1 = Overrun error

bit 2 = Parity error

bit 3 = CRC framing error

bit 4 = Break detect

bit 5 = Xmit buffer register empty

bit 6 = Xmit shift out register empty

bit 7 = No response, ignore bits 0-6

al = modem status, bit 0 = Change in Clear To Send

bit 1 = Change in Data Set Ready

bit 2 = Falling edge ring detect

bit 3 = Change in receive detect line

bit 4 = Clear to Send (CTS)

bit 5 = Data Set Ready (DTS)

bit 6 = Ring detected

bit 7 = Receive detect line

Functions:

ah = 0     Reset the specified port

Call with:  al serial port register values

bits  7    6    5    4    3    2    1    0

--baud-rate--  -Parity- Stop  --Word--

000 =  110      bits    bit    size

001 =  150                    (bits)

010 =  300    00 = off  0=1

011 =  600    01 = odd  1=2   10 = 7

100 = 1200    10 = off        11 = 8

101 = 2400    11 = even

110 = 4800

111 = 9600

ah = 1      Transmit a character

Call with:  al = character to send (not changed on exit)

ah = 2      Received a character

Returns:    al = character received

ah = 3      Get status

ah = 4      Advanced Initialization (not supported on all systems)

Call with:  al = 0/1  no break/break

bh = Parity,    0 = none     3 = stick odd

1 = odd      4 = stick even

2 = even

bl = 0/1 Stop bits set to 1/2

ch = Word size, 0 = 5 bits   2 = 7 bits

1 = 6 bits   3 = 8 bits

cl = Baud rate, 0 =  110     5 =  2400

1 =  150     6 =  4800

2 =  300     7 =  9600

3 =  600     8 = 19200

4 = 1200

ah = 5     Advanced port control (not supported on all systems)

al = 0  Get modem register

Returns: bl bit 0 = Data Terminal Ready (DTR)

bit 1 = Request To Send (RTS)

bit 2 = Out 1

bit 3 = Out 2

bit 4 = Loop modem

al = 1  Set modem register

Call with:  bl = value for register (see above)

15

���������������������������������������������������������� int 15h �����

GENERAL SERVICES

On old PCs this function is only used for cassette servicing.

Additional functions have been added for advanced features with

newer equipment.

Called with: ah = function code

Returns:     CF = 0 if successful

CF = 1 if failure or function not supported

Functions:

ah = 0     Cassette motor set on

ah = 1     Cassette motor set off

ah = 2     Read cx bytes from cassette, es:bx = ptr to load area

ah = 3     Write cx bytes to cassette,  es:bx = ptr to read area

ah = 0Fh   Disk format hook, called from disk format routines

after a cylinder access completed.  Called with:

al = 1-undergoing surface analysis, 2-if formating

ah = 21h   Error log (PS 2), al = 0 to read, al = 1 to write

***  LAP TOP MACHINES ONLY - 40h to 44h  ***

ah = 40h   Systems info in cx, bx - al = 0 to read, 1 to write

for modem info in bx - al = 2 to read, 3 to write

ah = 41h   Wait for an event, dx = i/o port to read or use es:di

as ptr to user event.  al = type of event 0-4,11-14

bh = event mask, bl = # of 55ms counts to timeout

ah = 42h   Power off, al = 0 or 1 for mode of power off

ah = 43h   Get system status in al

7     6     5     4       3     2     1    0

low   extrn  bad  pwr up  modem  ports      LCD

battery  pwr  time  alarm    on     on      missing

ah = 44h   Modem power, al = 0 turn off,  al = 1 turn on

*** FUNCTIONS IN RECENT BIOS VERSIONS ONLY ***

ah = 4Fh   Keyboard input hook, called every keystroke with

al = scan code.  CF = 1 to change or use the scan

code in al.  CF = 0 to ignore the key.

ah = 80h   Open device number bx, in process number cx.

ah = 81h   Close device number bx, in process number cx.

ah = 82h   Terminate device number bx

ah = 83h   Wait for timeout, and set bit 15 at ptr es:bx when

timed out.  al = 0 to set wait period cx:dx uSec,

al = 1 to stop timeout timer.

ah = 84h   Game port read, dx = 0 to read switches into al

high nibble,  dx = 1 to read linear position

(Unit 1: ax = x, bx = y   Unit 2: cx = x, dx = y)

ah = 85h   Get system request key state, al = 0 down, al = 1 up

ah = 86h   Wait cx:dx microseconds before return

ah = 87h   Transfer cx words to/from memory above 1 meg

es:si ptr to table with source & destination ptrs:

es:si -> zeros          db  16dup(0)

src_seg_limit  dw  ?        ; 1-64K bytes

scr_ptr_low    dw  ?        ; 0-64K range

scr_ptr_high   db  ?        ; 0-16 Meg range

scr_rights     db  93h      ; r/w access

scr_zero       dw  0

dst_seg_limit  dw  ?        ; 1-64K bytes

dst_ptr_low    dw  ?        ; 0-64K range

dst_ptr_high   db  ?        ; 0-16 Meg range

dst_rights     db  93h      ; r/w access

dst_zero       dw  0

zeros2         db  16dup(0)

Notes:

cx = 32K words max xfer, ptr_low & ptr_high form

a 24 bit physical address (do not use a segment)

Return codes: al = 0 if ok, al = 1 parity error

al = 2 other errors, al = 3 gate addr line 20 bad

ah = 88h   Get extended memory size (above 1 Meg boundary)

ax = (number of bytes / 1024)

ah = 89h   Go to protected mode of operation. es:si ptr to table

comprised of 8 groups of 8 byte ptr sub-arrays.  Each.

group is formed from 8 bytes as follows:

seg_limit    dw  ?     ; 1-64K bytes

ptr_low      dw  ?     ; 0-64K range

ptr_high     db  ?     ; 0-16 Meg range

rights       db  93h   ; read/write access

zero         dw  0     ; set to zero

Ptr_low & high form a 24 bit physical address.

The 8 groups are as follows:

es:si ->  Group_1   Set to all zero

Group_2   ptr to beginning of Group_1

Group_3   ptr to Interrupt Descript Table IDT

Group_4   ptr to present DS segment

Group_5   ptr to present ES segment

Group_6   ptr to present SS segment

Group_7   ptr to present CS segment

Group_8   ptr where to jump to when done

Also: bh & bl set as offset from beginning of IDT to

spot where 1st & 2nd set of 8 hardware interrupts begin.

Returns ah = 0 if ok, regs changed: ax,bp,ds,es,ss,cs

ah = 90h   Device busy - used to signal DOS when it must wait.

Type   0  Disk timeout          0FCh  Hard disk timeout

codes  1  Floppy timeout                (PS 2 only)

(al)   2  Keyboard no timeout   0FDh  Floppy motor timeout

3  Mouse timeout         0FEh  Printer timeout

80h  Local area network (es:bx = network cntrl block

Returns CF = 0 if failure, 1 if minimum wait time occurred

ah = 91h   Interrupt done - used to signal DOS when hardware done

al = type code (Device busy type codes)

ah = C0h   Get ROM system information table ptr in es:bx.  See data

area starting at byte "sys_info_size".

*** PS 2 & COMPATIBLES ONLY - C1h to C4h ***

ah = C1h   Get Extended BIOS data area segment in es

ah = C2h   Mouse support (BIOS).  Sub-function code in al

al = 0  if bh = 0 mouse off, bh = 1 mouse on

al = 1  reset mouse & parameters, returns bh = 0

bl altered on return

al = 2  Set number of updates per second in bh,

bh = 0 - 10 updates/sec  4 - 80  updates/sec

1 - 20 updates/sec  5 - 100 updates/sec

2 - 40 updates/sec  6 - 200 updates/sec

3 - 60 updates/sec

al = 3  Set resolution in counts per millimeter:

bh = 0 - 1 cnts per mm   2 - 4 cnts per mm

1 - 2 cnts per mm   3 - 8 cnts per mm

al = 4  Get device type, bh = 0

al = 5  Initialize pointing device interface data size

bh = size 1 to 8 (1 to 8 bytes)

al = 6  Additional sub-functions in bh:

bh = 0  Get status, cl = cnts per mm (see al=3)

dl = updates/sec in hex, bl = status:

bit 6 = mode (0=stream, 1=remote)

5 = 0 disabled, 1 enabled

4 = scaling (0=1:1, 1=2:1)

2 = Left button pressed

0 = right button pressed

bh = 1  Set scaling to 1:1

bh = 2  Set scaling to 2:1

al = 7  Set location to "call far" when data available

in es:bx

Return code for all sub-functions in al:

al = 0  successful             3  interface error

1  bad function call      4  resend

2  invalid input          5  no far call installed

ah = C3h   Watchdog timer mode al = 0 disable, al = 1 enable,

bx = set counter value 1 to FFh

ah = C4h   Bus option select (POS) subfunction in al,

al = 0  Get base POS adapter register address in dx

1  Enable setup for slot bl

2  Adapter enable

16

���������������������������������������������������������� int 16h �����

KEYBOARD SERVICES

Call with:   ah = sub-function number

Functions:

ah = 0     Get a key, and wait until one is available if none are

in the queue.

Returns:    ah = keyboard scan code

al = ASCII character, or 0 if non-ASCII

ah = 1     Get key status

Returns:    zero flag = 0 if a valid key is in the queue

ah = keyboard scan code

al = ASCII character, or 0 if non-ASCII

zero flag = 1 if no keys in the queue

ah = 2     Get shift status register

Returns:    al = bits  7    6    5    4    3    2    1    0

right left ctrl alt scrl  num  cap insrt

-------shifts------ ------toggles-------

1 = down            1 = on

ah = 3     Set repeating character rate (not supported in all systems)

Call with:  al = 5

bh = start delay, 0 = 250 ms    2 =  750 ms

1 = 500 ms    3 = 1000 ms

bl = speed in characters per second - values

range from 0 = 30 cps, to 1Fh = 2 cps

ah = 5     Load keyboard queue (not supported in all systems)

Call with:  ch/cl = scan code/character to load into the queue

Returns:    al = 0 if ok, 1 if keyboard queue is full

**** Support for extended keyboard functions ****

ah = 10h   Get a key (similar to ah = 0, not supported in all systems)

ah = 11h   Get key status (similar to ah = 1, not supported in all systems)

ah = 12h   Get shift info (similar to ah = 2, not supported in all systems)

17

���������������������������������������������������������� int 17h �����

PRINTER SERVICES

Call with:  ah = function code

dx = printer number 0-2 (some systems allow 0-3)

Returns:    ah = status bits

7      6      5      4      3     2  1     0

not  acknow-  no   select   I/O   unused   no

busy  ledge  paper         error        response

__________from printer__________

Functions:

ah = 0   Send character to printer, al = character

ah = 1   Printer port initialization

ah = 2   Get printer status in ah

18

���������������������������������������������������������� int 18h �����

ROM BASIC

19

���������������������������������������������������������� int 19h �����

BOOTSTRAP LOADER

Floppy drive a: attempts a read from the first sector of the disk.

If read properly (i.e a boot disk is in drive a:), control is

transferred to the loaded program.  The boot sector at track 0,

sector 1 is transfered to memory at 0:7C00h.  dh = drive which

was used (0=A:).  Control is passed to 0:7C00h.

1A

���������������������������������������������������������� int 1Ah �����

REAL-TIME CLOCK SERVICES

Call with:   ah = sub-function number

Functions:

ah = 0     Get system timer (increments every 54.92 ms from int 8)

Returns:    al = # of 24 hour periods since read/powered up

cx:dx = 32 bit count

ah = 1     Get system timer & reset 24 hour counter

Returns:    cx:dx = 32 bit count

**** System must have CMOS clock for services 2-0Bh ****

ah = 2     Get time in BCD format

Returns:    ch/cl = hours/minutes

dh = seconds

dl = 0/1 if daylight savings time operation off/on

carry flag = 0 if clock ok, 1 if stopped

ah = 3     Set time in BCD format

Call with:  ch/cl = hours/minutes

dh = seconds

dl = 0/1 if daylight savings time operation off/on

ah = 4     Get date in BCD format

Returns:    cx = four digit year

dh/dl = month/day

carry flag = 0 if clock ok, 1 if stopped

ah = 5     Set date in BCD format

Call with:  cx = four digit year

dh/dl = month/day

ah = 6     Set 24 hour alarm (alarm vectors to int 4Ah)

Call with:  ch/cl = hours/minutes

dh = seconds

ah = 7     Clear alarm

ah = 9     Get alarm time & status

Returns:    ch/cl = hours/minutes

dh = seconds

dl = 0/1 alarm off/on

ah = 0Ah   Get days counter

Returns:    cx = number of days since 1-Jan-80

ah = 0Bh   Set days counter

Call with:  cx = number of days since 1-Jan-80

1B

���������������������������������������������������������� int 1Bh �����

KEYBOARD BREAK (when Control-Break pressed)

1C

���������������������������������������������������������� int 1Ch �����

TIMER TICKS - Called every 18.2 ms

1D

���������������������������������������������������������� int 40h �����

FLOPPY DISK SERVICES

1E

���������������������������������������������������������� int 70h �����

REAL-TIME CLOCK  (called by hardware 8259-2, IRQ 8)

This interrupt services the real-time clock hardware.  The hardware

supports 2 modes of operation, an interrupt at a specific 24 hour

interval (i.e 9:42 am), or repeatedly every 0.976 ms (1,024 Khz).

Both modes can operate at the same time if needed.

In the 24 hour alarm mode, the interrupt is vectored here by hardware

and interrupt 4Ah is called to alert the application program of the

alarm.  Int 4Ah is not handled by the BIOS other than to return, and

is normally revectored by a particular application using the alarm.

When repeating interrupt mode is active, the 32-bit microsecond

counter consisting of timer_clk_low and timer_clk_hi is decremented

by 976 us on every interrupt.  When the timer reaches zero, the

byte pointed to by the offset @timer_wait_off and and segment

@timer_wait_seg is set to 80h (this pointer is set by an application

program through int 1Ah function ah=6).

1F

���������������������������������������������������������� int 71h �����

LAN ADAPTER 1  (called by hardware 8259-2, IRQ 9)

Calls interrupt 0Ah.

20

���������������������������������������������������������� int 72h �����

Hardware servicer (called by hardware 8259-2, IRQ 10)

21

���������������������������������������������������������� int 73h �����

Hardware servicer (called by hardware 8259-2, IRQ 11)

22

���������������������������������������������������������� int 74h �����

Hardware servicer (called by hardware 8259-2, IRQ 12)

23

���������������������������������������������������������� int 75h �����

MATH CO-PROCESSOR (called by hardware 8259-2 IRQ 13)

The math co-processor 80287 invokes this interrupt.  Int 75h calls

the non-maskable interrupt int 2 to halt the system (80287 is not

used if this vector is left pointing here).  Programs which use

the 80287 must re-vector this interrupt to use the 80287.

24

���������������������������������������������������������� int 76h �����

HARD DISK CONTROLLER (called by hardware 8259-2 IRQ 14)

When the hard disk controller has completed its task, it signals

completion though hardware activation of int 76h.  The status in

hdsk_int_flags is set to "done", a value of 0FFh.  Interrupt 15,

function 91h may also be called to signal the interrupt is done.

25

���������������������������������������������������������� int 77h �����

Future servicer (called by hardware 8259-2 IRQ 15)

26

���������������������������������������������������������� int 13h �����

HARD DISK SERVICES

Call with: ah = sub-function number

dl = drive number (unless otherwise noted) 80h or 81h

for fixed disk 0 or 1.

Returns:   carry = 0 if function ok

ah = status (unless otherwise noted)

ah = 0    Function ok

ah = 1    Invalid value passed or unsupported function

ah = 2    Can not locate address mark

ah = 3    Write protected

ah = 4    Sector not found

ah = 5    Reset failure

ah = 7    Parameter activity failed

ah = 8    DMA overrun occurred

ah = 9    DMA attempted across 64K byte boundary

ah = 0Ah  Sector flag bad

ah = 0Bh  Cylinder bad

ah = 0Dh  Wrong # of sectors (format)

ah = 0Eh  Detected control data address mark

ah = 0Fh  DMA arbitration level has invalid range

ah = 10h  CRC or EDC (Error Detect & Correct) has an

unresolvable error

ah = 11h  Data corrected by EDC

ah = 20h  Disk controller failure

ah = 40h  Seek operation failed

ah = 80h  Hard disk not ready

ah = BBh  Error not defined

ah = CCh  Write error

ah = E0h  Error register is zero

ah = FFh  Disk sense error

If dl bit 7 was set to 1 on entry:

cx bits 7&6, 15-8 = highest cylinder number

bits 0-5 = highest sector number

dh = highest head number

dl = number of hard disks (1 or 2)

Functions:

ah = 0     Hard disk controller reset

ah = 1     Get last status

ah = 2     Read sectors

Call with:  al = number of sectors to read

cx bits 7&6, 15-8 = cylinder number

bits 0-5 = sector number

dh = head number, head 0 = 0

es:bx = ptr to buffer where to put data

ah = 3     Write sectors

Call with:  al = number of sectors to write

cx bits 7&6, 15-8 = cylinder number

bits 0-5 = sector number

dh = head number, head 0 = 0

es:bx = ptr to buffer where to get data

ah = 4     Verify sectors

Call with:  al = number of sectors to compare

cx bits 7&6, 15-8 = cylinder number

bits 0-5 = sector number

dh = head number, head 0 = 0

ah = 5     Format cylinder (non-ESDI type disks)

Call with:  cx bits 7&6, 15-8 = cylinder number

dh = head number, head 0 = 0

es:bx = ptr to bad sector map (non XT)

al = interleave number (XT only)

ah = 6     Format cylinder & set bad sector flags (XT only)

Call with:  al = interleave number

cx bits 7&6, 15-8 = cylinder number

dh = head number, head 0 = 0

ah = 7     Format drive, begin at specific cylinder (XT only)

Call with:  al = interleave number

cx bits 7&6, 15-8 = cylinder number

dh = head number, head 0 = 0

ah = 8     Get drive parameters

Returns:    cx bits 7&6, 15-8 = max cylinder number

bits 0-5 = max sector number

dh = max head number, head 0 = 0

ah = 9     Set parameters for drive (ignored for ESDI drives)

Call with   dh = 80h to use int 41h ptr, drive 0

dh = 81h to use int 46h ptr, drive 1

ah = 0Ch   Disk seek

Call with:  cx bits 7&6, 15-8 = cylinder number

dh = head number, head 0 = 0

ah = 0Dh   Secondary Disk Reset

ah = 10h   Check if drive is ready

ah = 11h   Recalibrate drive

ah = 15h   Get drive type (all but XT systems)

Returns:    ah = 0 no drive (cx & dx = 0)

ah = 3 hard disk

cx:dx = size of disk in 512 byte blocks

al = 4  use a 720K disk in a 720K drive

ah = 19h   Park disk heads (PS/2 systems only)

ah = 1Ah   Format hard disk (ESDI only)

Call with:  al = number of blocks in defect table (0=none)

cl bit 0 = 1 when no primary defect map

bit 1 = 1 when no secondary defect map

bit 2 = 1 allow logging found surface errors

in the secondary defect map

bit 3 = 1 Do extended surface analysis

bit 4 = 1 interrupt after each cyldr format

27

���������������������������������������������������������� unused �����

Unused interrupt vector termination

This routine processes invalid and unused interrupt requests.  The

hardware IRQ number is loaded into gen_int_occured, and the interrupt

cleared.  For software calls to an unused interrupt, a value 0FFh is

loaded into gen_int_occured, and the routine returns to the caller

without changing registers.

Alternatively, some systems simply return (iret).

28

���������������������������������������������������������� int 18h �����

ROM BASIC

Note: ROM basic points to F600 segment, and has been adjusted to

segment F000 to process in-line with the balance of the

BIOS code.  Internal data references may shift to segment

F600 depending on implementation.

29

����������������������������������������������������� System Reset �����

SYSTEM RESET

Reset the computer system.  General operation includes a test of

of the CPU, ROM checksum, and initialization of hardware including:

Memory system

Timer/Counter (which is also used for RAM refresh)

Interrupt Controller(s)

DMA Controller(s)

Keyboard Controller

Video Controller & Video RAM

Floppy Controller

Hard Disk Controller (if present)

Portions of the hardware may also have specific tests made to

insure reliable operation.  Test failures may display error code

on the screen if the video subsystem is operational, or generate

beeps or LED blinks to signify the error.

Note: A soft reset uses the warm_boot_flag to skip the memory

tests. (i.e. from pressing Ctrl-Alt-Del).

The system checks for installed ROMs by searching memory from

C000h to the beginning of the BIOS, in 2K chunks.  ROM memory is

identified if it starts with the word AA55h.  It is followed a one

byte field length of the ROM (divided by 512).  If ROM is found,

the BIOS will call the ROM at an offset of 3 from the beginning.

This feature was not supported in the earliest PC machines.

The last task turns control over to the bootstrap loader (assuming

the floppy controller is operational)

2A

������ BIOS EXTERNAL ENTRY POINT ���������������������������������������

2B

����������������������������������������������������� BIOS Setup �������

BIOS SETUP ROUTINES

When a special key combination is pressed, this setup routine is

run to allow changing CMOS BIOS values.  Typical key combinations

include CTRL-ALT-ESC, CTRL-ALT-S, or others to start the setup.

Once invoked, it may issue a system reset to let other application

software, other BIOS routines, and TSR's to configure the system

with the new information.

Typical options include:

Present time and date

Floppy disk settings

Hard disk table selection

Video display

Memory size

Speed of clock and/or bus

2C

�������������������������������������������������������� INTERRUPT �����

2D

�������������������������������������������� BIOS Initialization �������

This portion is called when the main computer BIOS passes control to

allow this BIOS to perform it's one-time start up code.