SECTION 4	SYSTEM  BOARD  JUMPER

SELECTION

The mainboard jumpers allow the user to select the desired system configuration.  The tables below shows the function and the default settings of these jumpers.  Details of these jumpers will be discussed in the following sections.

| Jumper | Function |
| --- | --- |
| JP1 | Parity enable/disable |
| JP2 | BIOS ROM chip selection |
| JP3 | Display adapter selection |
| JP4 | DRAM speed selector |

| Default | Settings |
| --- | --- |
| JP1 | Pin 2,3 short (parity check enable) |
| JP2 | Pin 2,3 short (256K ROM BIOS) |
| JP3 | Open ( Monochrome) |
| JP4 | Pin 2,3 short |

4.1		PARITY ENABLE/DISABLE

What is Parity?

Parity is a method for detecting errors in data communications.  The parity bit is added at the end of a data word.  The value of this bit is a function of the rest of the data word.  There are several ways that the parity bit can be calculated.

"Even parity" means that the parity bit is set so that the sum of all the bits in the data word (including the parity bit) is even.

"Odd parity" is similar; the parity bit is set so that the sum of all the data bits in a word, including the parity bit, is an odd number.

"No parity" means that no parity bit is added to the end of a data word.

For the Fox-M286 System board, Memory Data Parity Check can either be enabled or disabled by altering the setting of jumper JP1.

When installing DIP type RAM chips; U19, U20, U21 and U22 are the DRAM chips for the parity bit.   If data parity check is not desired, the sockets for U19, U20, U21 and U22 can be left blank; thus enabling the user to minimize the number of RAM chips installed for the system.

For 1MB memory configuration, 41256 RAM chips are used for parity bits.

For 640KB memory configuration, 41256 RAM chips are used for BANK 0 parity bits (U21 and U22); 4164 RAM chips are used for BANK 1 parity bits (U19 and U20).

Refer to the figure below for the location of jumper JP1, parity bits and the appropriate setting:

| JP1 Setting | Parity Check |
| --- | --- |
| 1-2 | Disable |
| 2-3 | Enable |

*	For 1MB SIMM memory, the parity check setting to enable

4.2		ROM CONFIGURATION AND INSTALLATION

The Fox-M286 contains sockets for two BIOS EPROMs that can either be 27128 or 27256.  Setting of jumper JP2 determines the ROM size and the type of ROM being used in the system board.  Installation of the chips is explained in the table below.  No matter what BIOS is used, the low-byte chip should be inserted in socket U5 and the high-byte chip should be inserted in socket U6.

| JP2 | Type of | ROM chip | installation |
| --- | --- | --- | --- |
| Setting | BIOS | U5 | U6 |
| 1-2 | 64KB size | 27256 | 27256 |
| 2-3 | 32KB size | 27128 | 27128 |

4.3		DISPLAY ADAPTER SETTINGS

The Fox-M286 system can work with various display units if provided with a suitable display adapter.  Jumper JP3 is used to signal the system what type of display adapter is installed.  If you want to use two monitors (a color monitor and a high-resolution monochrome monitor, for example), set the adapter type to the monitor you want to use when the system boots.  Refer to the figure below for the location of jumper JP3 and the appropriate setting:

| JP3 Settings | Adapter Type |
| --- | --- |
| ON | Color Graphics Adapter or Enchanced Graphics Adapter |
| OFF | Monochorme Adapter |

4.4		DRAM SPEED SELECTOR

The jumper JP4 allows the user to be able to use DRAM from different sources available in the market.  This option is included in order to render superior tolerance to the wide range of DRAM's specifications. This will ensure the highest degree of compatibility performance with those various DRAMs' timing requirements.

The default setting for JP4 is pin 2,3 shorted. This setting should satisfy most DRAMs' timing requirements.  In case system memory is not behaving reliably, try to alter JP4 setting to pin 1,2 to solve such problem.

Refer to the figure below for the default setting.

SECTION 5	SYSTEM BOARD CONNECTOR

This section describes with details of the hardware feature in the system board. You may find the information in this section useful. Under typical conditions, these connectors will have to connect to the indicator and switches of the system unit.

| Connector | Function |
| --- | --- |
| P1 | Speaker Connector |
| P2 | Hardware Reset Connector |
| P3 | Turbo LED Connector |
| P4 | Power LED and Ext-Lock Connector |
| P5 | Turbo Switch Connector |
| P6,P7 | Power Supply Connector |
| P8 | External Battery Connector |
| KB1 | Keyboard Connector |

Pin assignments are as follows:

5.1		P1 - Speaker Connector

The speaker connector is a 4-pin , keyed ,Berg strip.  The pin assignment follow.

| Pin | Assignments |
| --- | --- |
| 1 | Data out |
| 2 | +5 Vdc |
| 3 | Ground |
| 4 | +5 Vdc |

5.2		P2 - Hardware Reset Connector

A reset will restart the computer from the RAM test stage. If you encounter any problem while using unfamiliar software, you can always restart from the beginning by pressing the restart button.

| Pin | Assignments |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

5.3		P3 - Turbo LED Connector

The turbo LED indicates operation in turbo mode (12MHZ). This is a 2-pin keyed, Berg strip (0.1") male pin connector.

| Pin | Assignments |
| --- | --- |
| 1 | +5 Vdc |
| 2 | LED signal |

5.4		P4 - Power LED and Ext-lock Connector

The power LED indicates whether the power is on.  The keylock is used to enable or disable the keyboard.  By disabling the keyboard, the user ensures that anyone who does not have a key will be unable to use the computer. Unlocking the keylock enables the keyboard.

The power LED and keylock connector is a 5-pin Berg strip.  Its pin assignment are as follow:

| Pin | Assignments |
| --- | --- |
| 1 | LED Power |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

5.5		P5 - Turbo Switch Connector

The operating speed of the system board can be  selected by the turbo switch setting.  A speed switchover circuit ensures a smooth switchover between operating speeds, even during system operation.  Also software selection via keyboard is allowed if appropriate BIOS is used.

Hardware Select Mode:

The turbo switch changes operation mode between 8MHz and 16MHz. This setting determines the speed at which the processor runs after booting.  If you have a hardware switch on your case panel, connect it to jumper P5.  You can push the hardware switch to enter either Normal mode or Software select mode. The pin assignment are as follows:

| Pin | Assignments |
| --- | --- |
| 1 | Select Pin |
| 2 | Ground |

Software Select Mode:

In addition to setting the processor clock speed by front-panel switch (Turbo switch), you can also change processor speed 'on-the-fly' via keyboard command. The Fox-M286 has speed switching circuitry allowing it to change speed during operation, even while you are running a program.

The system is default to run in high speed, but the operating mode can also be changed. You can toggle  the operating speed between turbo mode and normal mode by pressing the following keystrokes simultaneously:

[Ctrl], [Alt], [+]

or			[Ctrl], [Alt], [-]

The turbo LED light will light up to show whether your computer is now running in turbo mode or normal mode. When the Turbo LED is turned on, the system is in Turbo mode (16MHz).  IF the Turbo is off, the system is in Normal mode (8MHz).

5.6		P6,P7 - Power Supply Connector

|  | Pin | Assignments |
| --- | --- | --- |
| P6: | 1 | Ground |
|  | 2 | Ground |
|  | 3 | -5 Vdc |
|  | 4 | +5 Vdc |
|  | 5 | +5 Vdc |
|  | 6 | +5 Vdc |

|  | Pin | Assignments |
| --- | --- | --- |
| P7: | 1 | Power good |
|  | 2 | +5 Vdc |
|  | 3 | +12 Vdc |
|  | 4 | -12 Vdc |
|  | 5 | Ground |
|  | 6 | Ground |

5.7		P8 - External Battery Connector

This is for connecting four size "AA" batteries instead of the on-board battery to the CMOS RAM.  This feature provides the same function as the on-board batteries.

| Pin | Assignments |
| --- | --- |
| 1 | 6 Vdc |
| 2 | N/C |
| 3 | Ground |
| 4 | Ground |

5.8		KB1 (DIN Connector) - Keyboard Connector

The keyboard connector is a five-pin DIN 90-degree printed circuit board (PCB) mounting. The pin assignments are as follows:

| Pin | Assignments |
| --- | --- |
| 1 | Keyboard Clock |
| 2 | Keyboard Data |
| 3 | Spare |
| 4 | Ground |
| 5 | +5 Vdc |

5.9		80287 Numeric Processor Installation

If you use certain application or programming languages, you may want to install a math co-processor to enhance the performance of your system.  Be ware however, that your application program must be specifically designed to take advantage of the math co-processor to benefit from its presence in the system.

The 80287 Numeric Processor operates in conjunction with the 80286 CPU and will enhance the system's math capabilities.  To install the 80287, simply insert the LSI chip into the empty socket (U7) below the power connector.

Important

Each pin on this LSI has a unique function.  The 80287 should therefore be inserted into the socket in the correct direction. If it is inserted incorrectly, the LSI or the computer may be damaged. Install the LSI correctly by referring to the illustration. Care should be taken not to bend and damage any of the pins. When inserting the LSI into the socket, apply force evenly over the LSI body.