EAGLE Version 3.5 Update Information

====================================

This file contains information for users of previous EAGLE versions.

Please read this file entirely if you are updating from an EAGLE version

prior to 3.5!

WARNING: Due to some necessary changes in the data structure once you edit

a file with version 3.50 you will no longer be able to edit it

with versions prior to 3.50!

PLEASE MAKE BACKUP COPIES OF YOUR CURRENT BOARD-, SCHEMATIC- AND

LIBRARY-FILES BEFORE EDITING THEM WITH VERSION 3.50!

- The SET XXX_PATH parameters have been replaced with the

Options/Directories dialog in the Control Panel. Please remove any line

containing a "SET XXX_PATH" command from your EAGLE.SCR file and enter

these definitions in the Directories dialog.

- In order to use Forward- and Backannotation a matching board and

schematic file must have the same name and must reside in the SAME

directory. If you have been keeping your boards and schematics in

different directories (using BRD_PATH and SCH_PATH), you will have

to move them into one common directory, referred to in the

Options/Directories dialog as the "Design" directory.

- The default extension when starting EAGLE with a filename has been changed

to ".EPF" to start with a Project File. If no such file exists, it defaults

back to ".BRD", to load a board file.

- The filename parameter in the ERC command no longer exists. The ERC report

is now always written into a file with the same name as the schematic,

but with the extension ".ERC".

- Airwires that would lead from one layer to another at the same X/Y

coordinates (like with two Smds on either side of the board, belonging to

the same signal) are now shown as dots in the Unrouted layer and can

be routed with the Route command.

- The Route command now sets the layer when starting from an Smd, and won't

end at an Smd on a different layer.

- By default no logfile is created (option -L).

- A part's value now has the same maximum length as an element's value

(14 characters).

- The Pins layer is now off by default.

- Layer colors and fillstyles are now stored in the data files.

- The Autorouter now stores its values in the CTL file with units.

- Multiple windows can now be opened to have the schematic, board, library,

CAM Processor and text files on the desktop all at the same time.

- EAGLE is now available on the two platforms DOS and OS/2.

- The new Help function uses hypertext links.

- The new EAGLE User Language allows you to write C-like programs that

can access the EAGLE data structures and produce virtually any kind of

output.

- The library edit dialog now has three buttons for "Dev", "Sym" and "Pac",

rather than the combo box of version 3.02.

- The CAM Processor menu now has a check box for the "Upside down" option.

- The file dialog now has a Drive and Path combo box.

- The LastMessage/Shift-Crsr-Up function has been removed.

- The program now keeps up to 9 backup copies of your board, schematic and

library files.

- The program can now be exited at any point by pressing Alt+X.

- The new Online Forward- and Backannotation always keeps board and schematic

consistent.

- The Electrical Rule Check now performs a consistency check between board

and schematic.

- All confirm messages now have a Cancel button when a script file is running.

- The CAM-Processor now writes .GPI and .DRI files into the same directory

as the output file.

- The CAM-Prcoessor now offers an easy to use job mechanism, which allows

you to create a complete sequence of output files.

- The file EAGLE.CFG is still used to read in options, but it is no longer

written by the CAM Processor.

- EAGLE now runs without a dongle! You will need a personalized license

key file to install the program.

Release notes for EAGLE 3.02r5 (Mar 17, 1995)

---------------------------------------------

- When entering Signal names, the program considered S1, S01, S001 etc. all

the same. Now these names are kept apart.

Generated names will always be filled with '0' to the length of the longest

name with the same prefix found in the drawing.

For example, if there is already a signal name S001, all generated signal

names with prefix 'S' will be like S002, S003 etc.

However, if there is, e.g., a signal name S4, the name generator will leave

out the name S004.

- When entering a new Signal or Net name, that name's prefix will be used

for generated names (like it has already been done for parts, pads etc.).

- The numeric part of names are now long integer numbers, so you can have

names like S9999999.

- The new command line option -Q has been implemented to allow setting the

mouse ratio. The Syntax is

-Qx,y

with x and y being numbers in the range 1..32767. The default for both values

is 8, which gives a 1:1 translation of mouse "mickeys" to screen pixels.

Larger values slow down the mouse, meaning you have to move the mouse for

a longer distance to get the same cursor movement as with the default values.

Smaller values speed up the mouse, which may result in skipping some pixels

on the screen. For example, setting -Q4,4 means "four mickeys for eight

pixels", which results in the cursor being positioned only on every other

pixel on the screen.

Any values that are different from the default -Q8,8 cause the mouse driver

to operate in "absolute" mode. This may result in the mouse cursor skipping

pixels even at values that are higher than 8. This is always a problem of

the mouse driver (usually supplied by the manufacturer of the mouse) and

can in most cases be overcome by installing a newer version of that driver.

If you cannot get a newer mouse driver, please keep the default values.

The special value -Q0,0 switches to absolute mode with no mouse ratio

setting, which should be used for digitizers.

Release notes for EAGLE 3.02r2 (Jan 23, 1995)

---------------------------------------------

- Signal names that are too long are now treated as an error in the Signal

command.

- A superfluous line-feed character has been stripped from the carriage

return command for DotCol8x24 printers.

- The HPLJET driver's INIT string has been modified to print from the very

top of the page.

Release notes for EAGLE 3.02r1 (Nov 30, 1994)

---------------------------------------------

- Added 'Units' and 'Decimals' parameters to the PhotoPlotter and

DrillStation sections of EAGLE.DEF.

Setting Units to Inch, MM or Auto defines the units used to list

aperture and drill sizes in the information files and in a generated

wheel file. With Inch and MM all sizes are given in the defined unit,

whereas Auto uses the unit with the least decimal digits (default is Auto).

The value of Decimals defines how many decimal digits are used when

Units is set to Inch or MM (default is 4).

- Added driver STYLUS800C for the Epson Stylus 800 Color Bubble Jet printer.

- When the output device is changed in the CAM processor menu, the parameter

values for that device are restored to the values defined in EAGLE.DEF.

When an output device is selected in the command line (with option -d) all

parameters that are not defined in the command line are restored to the

values defined in EAGLE.DEF.

- Circles with a Width of 0 are drawn as a filled pie, and are subtracted

from polygons as a filled pie.

- Unnamed pads in packages are now reported as an error when updating from

version 2.6.

- The Polygon command now displays the polygon's signal name if it is drawn

into one of the signal layers.

- Matrix printers get a separate Init before each page when the drawing is

printed on several pages.

Updating from EAGLE versions prior to 2.6:

------------------------------------------

Data files created with EAGLE versions prior to version 2.6 cannot be

loaded directly into version 3.x. You have to either convert these files

with the UPDATE26.EXE program or load and save them with EAGLE version 2.6x.

Please note that converting to version 2.6 requires the file LAYERS.NEW,

which may have to be adapted to your layer requirements.